# 3GPP TSG-RAN WG4 Meeting #110bis                                                           R4-2406649

# Changsha, China, 15th - 19th April 2024

# Title:	TP for TR37.718-21-11 Support of DC_3A-11A_n1A

# Source:	Softbank

# Agenda Item:	5.4.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR 37.718-21-11[6] combinations as requested in RAN4#109.

- EN-DC 3-11_n1 (2 band LTE (2 DL / 1 UL)) and 1 NR bands (1 DL / 1 UL))

- Reference

[1] 3GPP TR 21.905 Vocabulary for 3GPP Specifications (Release 17) V17.1.0

[5] 3GPP TS37.718-11-21 Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) (Release 18) V0.8.0

[6] 3GPP TS37.718-21-11 Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band (Release 18) V0.8.0

[7] 3GPP TS38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18) V18.3.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

## 5.x	DC_3-11_n1

- 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-11A_n1A | DC_3A_n1A DC_11A_n1A |

- 5.x.2	Co-existence studies

When uplink is DC_3A_n1A there is IMD3 and IMD5 interfering band 11 downlink.

There is no impact from UL 11_n1 interfering band 3 downlink.

- 5.x.3	∆TIB and ∆RIB values

- Table 5.x.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-11_n1 | 0.9 | 0.8 | 0.3 |

- Table 5.x.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3-11_n1 | 0.5 | 0.3 | - |

- 5.x.4	Reference sensitivity exceptions

Table 5.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-11A_n1A | 3 | 1730 | 5 | 25 | 1820 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2150 | N/A | N/A |
|  | 11 | N/A | 5 | 25 | 1490 | 15.2 | IMD34 |
| NOTE 4:This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

-- Unaffected parts omitted –

-- End of TP --