3GPP TSG-RAN WG4 Meeting # 110-bis												R4-2406425

Changsha, China, 15 – 19 April, 2024

Agenda item:			6.21.4

Source:	Moderator (Ericsson)

Title:	WF on R18 NR RedCap RRM requirements

Document for:	Approval

Measurements on higher priority carriers when configured with eDRX

- When eDRX cycle ≤ 10.24 sec

  - Option 1 (QC, vivo, E///, HW, MTK): UE shall search every layer of higher priority at least every Thigher_priority_search = max(60 , 15 * eDRX cycle length) * Nlayers

  - Option 2 (Nokia): Nlayers inside ()

- When 10.24 sec < eDRX cycle

  - Option 1 (vivo, MTK, HW, QC, E///): UE shall search every layer of higher priority at least every Thigher_priority_search = max(60, Y * eDRX cycle length) * Nlayers

    - Option 1: Y= 8 for PTW length of 2 DRX cycles.

    - Option 2: Y = 4 for PTW lengh of 4 DRX cycles.

  - Option 2: Nlayers inside ()