3GPP TSG-RAN WG4 Meeting #111	R4-2407016

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	7.25.1

Source:	MediaTek Inc.

Title:	Simulation results on PDCCH requirements for DSS enhancement

Document for:	Information

# Introduction

For DSS enhancement WI, RAN4 agreed to define requirements with the assumption “PDCCH channel estimation is assumed to use only the clean PDCCH symbol”. In this contribution, we present simulation results based on simulation assumptions in Table 1 [1].

Table 1. Simulation assumptions for PDCCH requirements for DSS enhancement

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD/TDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

# Simulation Results

In RAN4#110 meeting, it was agreed to define requirements with 10MHz/15kHz for both FDD and TDD. In RAN4#110-bis, it was agreed to have antenna configurations for FDD with 2x4 and 4x2, and TDD with 4x4 and 2x2 to avoid the same requirement values for both FDD and TDD [1]. We summarize SNR values achieving 1% BLER for PDCCH under the considered DSS scenario in Table 2. Also, simulation curves for BLER versus SNR are shown in Figure 1~4.

Table 2. SNR achieving 1% BLER

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (alignment) | SNR (dB) (impairment) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | -2.25 | 0.25 |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | -2.98 | -0.48 |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 4x4 Low | 1 | -2.31 | 0.19 |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 Low | 1 | -2.80 | -0.3 |

| Figure 1. BLER versus SNR for 2T4R, FDD | Figure 2. BLER versus SNR for 4T2R, FDD |
| --- | --- |
| Figure 3. BLER versus SNR for 4T4R, TDD | Figure 4. BLER versus SNR for 2T2R, TDD |

# Conclusion

In this contribution, we provide PDCCH simulation results for DSS enhancement WI.

# Reference

- R4-2406019, “Way forward on [110bis][333] NR_DSS_enh”, Ericsson