3GPP TSG-RAN WG4 Meeting # 111									          R4-2407158

# Fukuoka, Japan, 20 – 24 May 2024

# Title:	PC2 CA_n71(2A) MSD

# Agenda Item:	6.19.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# 1	Introduction

This contribution proposes an intra-band non-contiguous downlink (DL) CA_n71(2A) secondary component carrier (SCC) MSD analysis for one uplink carrier power class 3 (PC3) and power class 2 (PC2) operation based on [1]. We account for MSD test point proposals of [2,3,4] to reduce the number of test points from the current six entries down to two test points: one test point for legacy Band n71 UEs that support up to 20MHz uplink (UL) channel bandwidth (CBW), and one test point specific to BCS4/5 and to UEs that support the symmetrical UL/DL CBW. A companion draftCR is proposed to complete this band combination at this meeting.

# Discussion

## 2.1 One UL Measurement Results

## 2.1.1 Power Amplifier Calibration & Front-end assumptions

For power amplifier (PA) measurements, we assume:

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB, C-IM3 -60 dBc, C-IM5 -70 dBc

- PC3 operation and 4 dB post PA insertion losses

- The PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR and:

  - ACLR -30 dBc for PC3 and transmit diversity (TxD) operation for each PA

  - ACLR -31 dBc for PC2 operation

For MSD analysis, we assume:

- Tx-Rx duplexer isolation: 55 dB

- Tx-antenna duplex rejection in Rx range: 50 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR)

- Based on the agreed WF [5], the PC2 ΔRIBC is calculated relative to PC3 SCC CBW REFSENS requirement as specified in Table 7.3.2-1a.

## 2.1.2 One UL PC3, PC2 Noise Levels

Table 1 summarizes the measured interference levels affecting the DL SCC. Out of the three test points evaluated in [1], we retain only two test points:

- the legacy PCC + SCC 15MHz + 10MHz, Wgap = 10MHz test point,

- the BCS4/5 test point corresponding to PCC+SCC 25MHz + 5MHz, Wgap=5MHz.

Table 1: 1Tx PC3, PC2 single-Tx and PC2 dual-Tx measured noise levels

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | PC3 (1x23dBm) [dBm/MBW] | PC2 single TX (1x26dBm) [dBm/MBW] | PC2 dual Tx (23+23dBm) [dBm/MBW] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10.0 | 5 (RBstart = 2) | -16.1 | -14.2 | -15.7/-15.5 |
|  |  | 25MHz + 5MHz | 5.0 | 20 (RBstart = 8) | -14.1 | -12.1 | -13.7/-13.5 |

Figure 1 shows the measured PA1 output spectrum for the legacy PCC+SCC 15MHz + 10MHz, Wgap=10MHz configuration at 27 dBm output power.

Figure 1: PC3 PA output spectrum for CA_n71(2A) 15MHz+10MHz 5RBstart=2

## 2.1.3 One UL PC3 MSD

The MSD analysis can be found in the Annex.

The CA_n71B PC3 one uplink carrier MSD for SCC 5MHz is evaluated at 0.7dB. The MSD test point and text proposal based on [5] is captured in Proposal 1.

Proposal 1: For PC3, consider adopting the changes of Table 2 where it is proposed to:

- Reduce the number of PC3 legacy test points from 6 to 1 by keeping only the PCC+SCC = 15MHz + 10MHz (22.2dB) and add “SCC” to the title of the column that captures “ΔRIBNC.

- Introduce a PC3 test point specific to BCS4/5 and to UEs that support the symmetrical UL/DL CBW with PCC+SCC=25MHz + 5MHz and 23.7dB MSD for the 5MHz CBW SCC.

Table 2: Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink carrier (Table 7.3A.2.2-1)

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
|  |  | 25MHz + 5MHzX | Wgap = 5.0 | 20 (RBstart = 8) | 23.7 |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE X: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

## 2.1.3 One UL PC2 MSD

The MSD analysis can be found in the Annex.

Proposal 2: For PC2, consider adopting the test points of Table 3 and new core requirement text highlighted in blue below.

For aggregation of two or more downlink FDD carriers with one uplink carrier, the power class 3 reference sensitivity is increased by ΔRIBNC only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. For power class 2, the reference sensitivity power level is increased by ΔRIBNC for specific uplink and downlink test points which are specified in Table 7.3A.2.2-1a. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 3: Power class 2 intra-band non-contiguous CA reference sensitivity with one uplink carrier (new Table 7.3A.2.2-1a)

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | [24.1] | [30.5] | FDD |
|  |  | 25MHz + 5MHz1 | Wgap = 5.0 | 20 (RBstart = 8) | [25.7] 3 | [32.1] 3 |  |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with dual Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |  |

# Conclusion

In this contribution, the one uplink carrier SCC MSD requirements for CA_n71B for PC3 and PC2 operation are analyzed and we bring the following proposals for changes to TS 38.101-1. A companion draftCR is presented at this meeting to help capture the RAN4 agreements.

Proposal 1: For PC3, consider adopting the changes of Table 2 where it is proposed to:

Table 2: Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink carrier (Table 7.3A.2.2-1)

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
|  |  | 25MHz + 5MHzX | Wgap = 5.0 | 20 (RBstart = 8) | 23.7 |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE X: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

Proposal 2: For PC2, consider adopting the test points of Table 3 and new core requirement text highlighted in blue below.

For aggregation of two or more downlink FDD carriers with one uplink carrier, the power class 3 reference sensitivity is increased by ΔRIBNC only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. For power class 2, the reference sensitivity power level is increased by ΔRIBNC for specific uplink and downlink test points which are specified in Table 7.3A.2.2-1a. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 3: Power class 2 intra-band non-contiguous CA reference sensitivity with one uplink carrier (new Table 7.3A.2.2-1a).

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | [24.1] | [30.5] | FDD |
|  |  | 25MHz + 5MHz1 | Wgap = 5.0 | 20 (RBstart = 8) | [25.7] 3 | [32.1] 3 |  |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with dual Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |  |

# References

- R4-2400365 PC2 CA_n71(2A) MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc.

- R4-2405447 n71(2A) PC2 MSD, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Qualcomm Inc.

- R4-2404762 MSD considerations on CA_n71(2A), 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Murata Manufacturing Co. Ltd.

- R4-2404176 MSD Analysis for PC2 CA_n71(2A), 3GPP TSG-RAN WG4 Meeting #110bisChangsha, China, Apple.

- R4-2403628 WF on PC2 HPUE MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc., Huawei, Qualcomm Inc.

# Annex

MSD analysis for legacy test point: PCC+SCC 15MHz + 10MHz, Wgap = 10.0MHz, Lcrb= 5 (RBstart = 2).

- The PC3 MSD of 18.6dB is 3.6dB smaller than the agreed 22.2dB

- The PC2 single Tx MSD = 20.5dB, i.e. 1.9dB greater than the calculated PC3 MSD level. We therefore propose to apply this MSD increase to the agreed legacy test point, ie 22.2+1.9 = 24.1dB

- The PC2 dual Tx MSD = 26.9dB, i.e. 8.3dB greater than the calculated PC3 MSD level. We propose PC2 dual Tx MSD = 22.2 + 8.3 = 30.5dB.

MSD analysis new test point: PCC+SCC 25MHz + 5MHz, Wgap = 5.0MHz, Lcrb= 20 (RBstart = 8).