3GPP TSG-WG4 Meeting #111 	R4-2407212

# Fukuoka, Japan, May 20 - May 24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.101 - inclusion of 8Rx Applicability Rule and definition of simplified and baseline Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of PDSCH, PDCCH and PBCH applicability rules for 8Rx. Introduction of definitions of simplifeid and baseline receivers |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated tables in Clause 5.1.1.2, introduction of 8Rx Applicability rules, inclusion of new definition for simplified and baseline receiver. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 8Rx applicability rules will not be included in TS 38.101-4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

DL BWP: DL bandwidth part as defined in TS 38.213 [11].

EN-DC: E-UTRA-NR Dual Connectivity as defined in clause 4.1.2 of TS 37.340 [13].

Enhanced Receiver Type 1: SU-MIMO interference mitigation advanced receiver [14]

-	R-ML (reduced complexity ML) receiver with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas

-	R-ML (reduced complexity ML) receiver with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas

FR1: Frequency range 1 as defined in clause 5.1 of TS 38.101-3 [8].

FR2: Frequency range 2 as defined in clause 5.1 of TS 38.101-3 [8].

RedCap: A UE with reduced capabilities as defined in clause 4.2 in TS 38.306 [14].

Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers, and a single codeword, with two joint 4Rx MIMO detectors.

SSB: SS/PBCH block as defined in clause 7.8.3 of TS 38.211 [9].

<End of change 1>

# <Start of change 2>

#### 5.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2, 4 or 8 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.2 of TS 38.101-1 [6]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1.

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports |  | Test type |  | Test list |  | Exceptions |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE supports only 2RX |  | PDSCH |  | All tests in Clause 5.2.2 |  |  |  |
|  |  | PDCCH |  | All tests in Clause 5.3.2 |  |  |  |
|  |  | PBCH |  | All tests in Clause 5.4.2 |  |  |  |
| UE supports only 4RX or both 2RX and 4RX |  | PDSCH |  | All tests in Clause 5.2.3 (Note 2) |  |  |  |
|  |  | PDCCH |  | All tests in Clause 5.3.3 (Note 2) |  |  |  |
|  |  | PBCH |  | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |  |  |  |
| UE supports 8Rx (Baseline SU-MIMO 8Rx Receiver), 4Rx and 2Rx |  | PDSCH |  | Tests in Clause 5.2.3.(Note 2, 3) All tests in Clause 5.2.4.(Note 2) |  | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |  |
| or |  | PDCCH |  | All tests in Clause 5.3.3.(Note 2,3) |  |  |  |
| UE supports only 8Rx (Baseline SU-MIMO 8Rx Receiver) and 4Rx |  | PBCH |  | All tests in Clause 5.4.3(Note 1) |  |  |  |
| UE supports only 8Rx (Baseline SU-MIMO 8Rx Receiver) and 2Rx |  | PDSCH |  | Tests in Clause 5.2.2.(Note 2, 4) All tests in Clause 5.2.4.(Note 2) |  | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |  |
|  |  | PDCCH |  | All tests in Clause 5.3.2.(Note 2,4) |  |  |  |
|  |  | PBCH |  | All tests in Clause 5.4.2 |  |  |  |
| UE supports only 8Rx (Baseline SU-MIMO 8Rx Receiver) |  | PDSCH |  | All tests in Clause 5.2.4.(Note 2) |  |  |  |
|  |  | PDCCH |  | N/A |  |  |  |
|  |  | PBCH |  | N/A |  |  |  |
| UE supports 8Rx (Simplified SU-MIMO 8Rx Receiver), 4Rx and 2Rx | PDSCH | Tests in Clause 5.2.3.(Note 2, 3) All tests in Clause [5.2.5] (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3.(Note 2,3) |  |
| UE supports only 8Rx (Simplified SU-MIMO 8Rx Receiver), and 4Rx | PBCH | All tests in Clause 5.4.3(Note 1) |  |
| UE supports only 8Rx (Simplified SU-MIMO 8Rx Receiver) and 2Rx | PDSCH | Tests in Clause 5.2.2.(Note 2, 4) All tests in Clause [5.2.5](Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2.(Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx (Simplified SU-MIMO 8Rx Receiver) | PDSCH | All tests in Clause [5.2.5](Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

<End of change 2>