3GPP TSG-RAN WG4 Meeting #111	R4-2407401

Fukuoka City, Fukuoka, Japan, 20 - 24 May 2024

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for PDCCH with punctured CORESET0

Agenda item:	7.8.7.1

Document for:	Information

- Introduction

In this paper, we share our simulation results for the PDCCH test cases, based on the simulation assumptions agreed in the WF from RAN4 meeting #110 (R4-2402863) and the requirements table agreed in RAN4#111.

# PDCCH Simulation Results

PDCCH alignment and impairment results are included in Table 2-1.

Table 2-1

| TX | BW MHz | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Pm-dsg (%) | Alignment SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | 1 | 5.2 | 7.2 |
| 2 | 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | 1 | -2.9 | -0.9 |

The results included in the table 2-1 do not assume channel estimation in the partially transmitted REG bundle which is, according to the RAN1 agreements, up to UE implementation.

Observation 1: PDCCH Impairment results in Table 2-1 assume no channel estimation on the partial REG bundle.

- Conclusions

Observation 1: PDCCH Impairment results in Table 2-1 assume no channel estimation on the partial REG bundle.

Proposal 1: RAN4 to consider the PDCCH impairment results provided in this document for the definition of the requirements.