3GPP TSG-RAN4 Meeting #111	R4-2407984

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source:	LG Electronics

Title:	Tx requirements for SL intra-band contiguous CA

Agenda item:	10.9.3.2

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirements for intra-band contiguous SL CA for the Band n47 based on the approved WF[1].

Discussion

Based on the approved WF[1], we would like to discuss the UE RF requirements for SL intra-band contiguous CA in n47.

- UE RF architecture WF: - It is proposed to use option 1 and option 3 as the starting point assumption for PC2 intra-band contiguous CA. - RAN4 can consider all candidate RF architectures to derivePC2 MPR/A-MPR requirements for intra-band contiguous CA. - Further discuss these options in the next meeting. - Option 1: 1PA + 1LO. Figure 3-a PC2 intra-band contiguous CA assumption of 1PA+1LO - Option 2: 2PA + 2LO, Figure 3-b PC2 intra-band contiguous CA assumption of 2PA+2LO - Option 3: 2PA + 1LO Figure 3-c PC2 intra-band contiguous CA assumption of 2PA+1LO - Simulation assumption WF: - Agree on below table for simulation assumption - The carrier leakage can be further discussed Center frequency; 5.9GHz. Bandwidth; per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW). Maximum output power for aggregated CBW; 26dBm. Numerology; 15 kHz/30kHz/60kHz. Modulation per CC; QPSK/16QAM/64QAM/256QAM. Waveform; CP-OFDM. ACLR; 31dBc. Carrier leakage; 25dBc. IQ image; 34dBc. CIM3; 60dBc. PA calibration; PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case..

UE Tx RF requirement

UE RF architecture

For PC2 intra-band contiguous CA, 1PA+1LO(Option1) and 2PA+1LO(Option3) were proposed as a starting point assumption for MPR/A-MPR evaluation.

For PC2 in Option2, 2x26dBm + 2LO was assumed. However, its total maximum output power is 29dBm(PC1.5). 2x23dBm + 2LO is more practical and has its total MOP of 26dBm(PC2). In aspect of MPR, 2x23dBm + 2LO architecture is expected to be higher than 2x26dBm + 2LO. Therefore, 2x23dBm + 2LO architecture should be considered instead of 2x26dBm + 2LO.

In the existing NR intra-band CA, PC2 UE MPR requirements were specified considering ‘dualPA’, ‘TxD’, and ‘single PA’ architectures as Table 2-1.

Table 2-1. PC2 UE Architecture for NR intra-band UL CA

| Architecture | Description | Indicated capability | Support UL MIMO | Applicable cases | Note |
| --- | --- | --- | --- | --- | --- |
| #1 | 2x23 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | C/NC CA | No Frequency Separation limitation |
| #2 | 2x23 dBm PA + 1 LO with 200MHz BW | TxD | Yes | C/NC CA | Frequency Separation ≤ 200MHz For NC CA, BW Gap size＜CC1 +CC2 CBW, when UE declares intraBandFreqSeparationUL-v1620 value ≤ 200 MHz |
| #3 | 1x26dBm PA + 1 LO | - | Yes | C/NC CA |  |

Here, Architecture #1 = Option 2, Architecture #2 = Option 3, Architecture #3 = Option 1

Proposal 1: Consider 3 architectures, 1x26dBm+1LO, 2x23dBm+2LO, and 2x23dBm+1LO for PC2 SL intra-band contiguous CA MPR evaluation.

Figure 2-1 shows the example for PC2 intra-band contiguous CA. Here, LCRB1 and LCRB2 are assumed as the number of the allocated RB for CC1 and CC2 respectively.

Figure 2-1. PC2 intra-band C CA architecture

For Option 1 and Option 3, PSD of LCRB1 is same as that of LCRB2. However, for Option2 (2PA+2LO), PSD of LCRB1 is different from that of LCRB2. The MPR of each option can be different.

The ‘dualPA-architecture’ needs to be indicated like NR CA.

Proposal 2: Indicate ‘dualPA-architecture’ like NR intra-band contiguous CA.

Conclusion

In this contribution, we provided our view on Tx requirements of SL intra-band non-contiguous CA. Based on those, we propose as follows.

Proposal 1: Consider 3 architectures, 1x26dBm+1LO, 2x23dBm+2LO, and 2x23dBm+1LO for PC2 SL intra-band contiguous CA MPR evaluation.

Proposal 2: Indicate ‘dualPA-architecture’ like NR intra-band contiguous CA.

- References

[1] R4-2406611, “WF on NR_SL_ intraB_CA_ITS”, OPPO, Meta Ireland

[2] RP-240840, “New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band”, Moderator(Apple)