3GPP TSG-RAN WG4 Meeting # 111 	R4-2408092

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	vivo

Title: 	Preliminary evaluation on AIoT coexistence

Agenda Item:	10.13.2.2

Document for:	Approval

- Introduction

In the last meeting, several scenarios for AIoT are prioritized but the cases for co-existence evaluation are still quite numerous which will bring too much burden on the upcoming simulation. In this contribution, we provide our co-existence analysis on different scenarios to try to further reduce the cases that need to be simulated.

- Discussion

  - Performance impact due to the CW interference

Before we jump into the detailed co-existence analysis, the first issue we want to figure out is how to treat the impact of CW. For AIoT, in [1], the following parameter for CW cancellation.

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| [2K] | CW cancellation (dB) | N/A | For [monostatic backscatter], FFS - [140dB for BS] - [120dB for UE] For [bistatic backscatter] Assuming CW has no impact to the receiver sensitivity loss. |

The above information means that only when a Reader needs to transmit CW and receive D2R simultaneously, the CW has impact on the Reader receiver performance, and the Reader is capable to cancel the CW to avoid severe self-interference. However, in our understanding, the Reader performance degradation is not cause by CW itself.

Figure 1 SNR vs BLER under different CW power

As shown in Figure 1, when the CW power changes, the curve of SNR vs BLER keeps the same. The reason here is that after down-conversion, the CW is transformed to DC and only the amplitude of D2R. Only if the amplitude doesn’t exceed the dynamic range of ADC, the impact of CW itself can be ignored.

Observation 1: The Reader received performance degradation due to CW itself can be ignored.

In our understanding, the main reason that causing the receiver performance degradation is the IM3 product from CW and D2R, as shown in Figure 2.

Figure 2 Spectrum for R2D and IM3 product

85 dB CW cancellation is assumed in the Figure 2 based on algorithm. It is obvious that the IM3 product will overlap with D2R which reduce the SNR of D2R and bring obvious receiver performance degradation.

Observation 2: The IM3 produced by CW and D2R will overlap with D2R which will bring SNR degradation.

Based on the analysis above, we propose:

Proposal 1: For the D2R, only the interference of IM3, which produced by D2R and CW, is considered as the impact of CW in the co-existence evaluation, and the impact of CW itself can be ignored.

For NR, as we analyzed in the last meeting, the CW with high PSD may cause severe blocking issue at NR receiver, so at least the CDF of CW power at NR receiver should be provided and companies can further check the performance impact.

Proposal 2: The CDF of CW power at legacy NR receiver (including BS/UE) should be provided in co-existence evaluation and further check the degradation due to the receiver blocking.

  - Coexistence analysis for D1T1 based on link budget

Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors.

Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB

In [2], two options are prioritized for D1T1

There are too many cases to be evaluated since on top of these two options, we still have different topology and spectrum usage, which will bring significant burden on the simulation work. In this part, we try to analyze worst cases for different scenarios to show which cases are not suffering from co-existence issue. The following metrics are used:

- For NR as victim, 5% throughput loss is used, and the throughput is calculated based on the formula in TR 38.803.

- For AIoT as victim, the SNR@10% BLER is used, i.e., -15 dB @Reader, 25dB @Tag based our LLS

The following parameters are used in the link budget analysis:

Table I Parameters for link budget analysis

| Item | value | Unit |
| --- | --- | --- |
| Freqency | 900 | MHz |
| Penetration loss(High loss) | 18.49611171 | dB |
| Penetration loss(Low loss) | 15.08597448 | dB |
| UE/AIoT height | 1.5 | m |
| Reader anenna height | 8 | m |
| Macro BS height | 25 | m |
| UE maximum output power | 23 | dBm |
| UE minimum output power | -40 | dBm |
| UE channel bandwidth | 5 | MHz |
| UE ACLR (PC3) | 30 | dB |
| UE IBE on 1st RB (QPSK) | -18.13923903 | dB |
| UE ACS | 33 | dB |
| AIoT channel bandwidth | 180 | kHz |
| Tag amplifier gain (for device 2a only) | 10 | dB |
| Interference supperession@Tag Tx (provided by matching network and BPF) | 10 | dB |
| Reader/CW power in DL spectrum | 33 | dBm |
| Reader/CW power in UL spectrum | 23 | dBm |
| Reader/CW antenna gain in DL spectrum | 6 | dBi |
| Reader/CW antenna gain in UL spectrum | 0 | dBi |
| Macro BS output power | 43 | dBm |
| Reader/BS ACS | 50 | dB |
| Reader/BS ACLR | 45 | dB |
| Noise Figure @ Reader/BS | 5 | dB |
| Noise Figure @ UE | 9 | dB |
| Noise Figure @ Tag | 24 | dB |
| IIP3@Reader LNA | -10 | dBm |
| CW self-cancelation | 85 | dB |

Firstly, we analyze the cases under option 1-1. The rule for worst case is to maximize the interference power while minimize the wanted signal power.

- CW in UL spectrum, aggressor: NR UL, victim: D2R

| Interference power |  |  |
| --- | --- | --- |
| Interference power@UE Tx | -69.92612334 | dBm |
| Interference power@Reader Rx | -101.7868843 | dBm |
| Noise power@Reader | -116.4472749 | dBm |
| Interference total | -69.92319808 | dBm |
| Signal power of D2R |  |  |
| Tag output power (device 1) | -35.82253864 | dBm |
| Tag output power (device 2a) | -25.82253864 | dBm |
| Received power@Reader (device 1) | -98.47687402 | dBm |
| Received power@Reader (device 2a) | -88.47687402 | dBm |
| SINR@Reader (in-channel) (device 1) | -28.55367594 | dB |
| SINR@Reader (in-channel) (device 2a) | -18.55367594 | dB |

| Interference power |  |  |
| --- | --- | --- |
| Interference power@UE Tx (In-channel) | -69.92612334 | dBm |
| Interference power@Reader Rx (In-channel) | -101.7868843 | dBm |
| Interference power of IM3 | -198.6450773 | dBm |
| Noise power@Reader | -116.4472749 | dBm |
| Interference total (in-channel) | -69.92319808 | dBm |
| Signal power of D2R |  |  |
| Tag output power (device 1) | -35.82253864 | dBm |
| Tag output power (device 2a) | -25.82253864 | dBm |
| Received power@Reader (device 1) | -94.64507728 | dBm |
| Received power@Reader (device 2a) | -84.64507728 | dBm |
| SINR@Reader (device 1) | -24.72187921 | dB |
| SINR@Reader (device 2a) | -14.72187921 | dB |

For the cases above, to maximize the interreference power, the UE is assumed at cell edge and transmitted with maximum output power.

- CW in UL spectrum, aggressor: D2R, victim: NR UL

| Interference power |  |  |
| --- | --- | --- |
| Interference power@Tag Tx | -117.6922886 | dBm |
| Interference power@BS Rx | -157.6922886 | dBm |
| Noise power@BS | -116.4472749 | dBm |
| Interference total | -114.0148332 | dBm |
| Signal power of NR UL |  |  |
| Received power@BS | -105.9054043 | dBm |
| SINR@BS before interference added | 10.54187065 | dB |
| SINR@BS after interference added | 8.109428949 | dB |
| Throughput loss | 2.619227169 | % |

For this case, the worst case is to maximize the R2D power, so there is no difference between D1T1-A1 and D1T1-A2. To minimize the signal power, the UE is assumed near the BS and transmitted with minimum output power.

- CW in DL spectrum, aggressor: NR DL, victim: D2R

| Interference power |  |  |
| --- | --- | --- |
| Interference power@Macro BS Tx | -82.56234653 | dBm |
| Interference power@Reader Rx | -87.56234653 | dBm |
| Interference total | -81.36768742 | dBm |
| Signal power of D2R |  |  |
| Tag output power (device 1) | -19.82253864 | dBm |
| Tag output power (device 2a) | -9.822538642 | dBm |
| Received power@Reader (device 1) | -82.47687402 | dBm |
| Received power@Reader (device 2a) | -72.47687402 | dBm |
| SINR@Reader (device 1) | -1.109186605 | dB |
| SINR@Reader (device 2a) | 8.890813395 | dB |

| Interference power |  |  |
| --- | --- | --- |
| Interference power@Macro BS Tx | -82.56234653 | dBm |
| Interference power@Reader Rx | -87.56234653 | dBm |
| interference power of IM3 (device 1) | -25.93376518 | dBm |
| interference power of IM3 (device 1) | -170.5623465 | dBm |
| Interference total | -81.36768742 | dBm |
| Signal power of D2R |  |  |
| Tag output power (device 1) | -19.82253864 | dBm |
| Tag output power (device 2a) | -9.822538642 | dBm |
| Received power@Reader (device 1) | -78.64507728 | dBm |
| Received power@Reader (device 2a) | -68.64507728 | dBm |
| SINR@Reader (device 1) | 2.722610131 | dB |
| SINR@Reader (device 2a) | 12.72261013 | dB |

For cases above, to maximize the interference power, it is assumed that the distance between macro BS and Rx Reader is 35m which is the minimum distance between macro BS and UE in traditional simulation.

- CW in DL spectrum, aggressor: D2R, victim: NR DL

| Interference power |  |  |
| --- | --- | --- |
| Tag ouput power (device 2a) | -7.290772601 | dBm |
| Interference power@Tag Tx | -62.8717345 | dBm |
| Interference power@UE Rx | -85.8717345 | dBm |
| Noise power@UE | -112.4472749 | dBm |
| Interference total | -62.84997493 | dBm |
| Signal power of NR UL |  |  |
| Received power@UE | -44.98947231 | dBm |
| SINR@UE before interference added | 67.45780264 | dB |
| SINR@UE after interference added | 17.86050262 | dB |
| Throughput loss | 39.97530828 | % |

For this scenario, the worst case is same for both D1T1-A1 and D1T1-A2 since only Tag output power matters.

- R2D in DL spectrum, aggressor: NR DL, victim: R2D

| Interference power |  |  |
| --- | --- | --- |
| Inteference power @macro BS Tx | -86.40151601 | dBm |
| Inteference power @Tag Rx | -51.40151601 | dBm |
| Noise power@Tag | -97.44727495 | dBm |
| Interference total | -51.40003496 | dBm |
| Signal power of R2D |  |  |
| Received power@Tag | -19.82253864 | dBm |
| SINR@Tag | 31.57749632 | dB |

To maximize the interference, the distance between macro BS and Tag is 35m and for this scenario, D1T1-A1 and D1T1-A2 are same.

- R2D in DL spectrum, aggressor: R2D, victim: NR DL

| Interference power |  |  |
| --- | --- | --- |
| interference power@Reader Tx | -80.78688431 |  |
| interference power@UE Rx | -68.78688431 |  |
| Noise power@ UE | -112.4472749 |  |
| Interference total | -68.5209847 | dBm |
| Signal power of R2D |  |  |
| Received power@UE | -44.98947231 | dBm |
| SINR@UE before interference added | 67.45780264 | dB |
| SINR@UE after interference added | 23.53151239 | dB |
| Throughput loss | 21.36817323 | % |

- R2D in UL spectrum, aggressor: NR UL, victim: R2D

| Interference power |  |  |
| --- | --- | --- |
| Interference power@UE Tx | -40.72020093 | dBm |
| Interference power@Tag Rx | -32.5809619 | dBm |
| Interference total | -31.96082764 | dBm |
| Signal power of R2D |  |  |
| Received power@ Tag | -33.2907726 | dBm |
| SINR@Tag | -1.329944959 | dB |

- R2D in UL spectrum, aggressor: R2D, victim: NR UL

| Interference power |  |  |
| --- | --- | --- |
| Noise power@BS | -116.4472749 | dBm |
| Interference power @ Reader Tx | -105.6530725 | dBm |
| Interference power @ BS Rx | -110.6530725 | dBm |
| Interference toal | -104.1932962 | dBm |
| Signal power of R2D |  |  |
| Received power@BS | -108.2995965 | dBm |
| SINR@BS before interference added | 8.147678434 | dB |
| SINR@BS after interference added | -4.106300328 | dB |
| Throughput loss | 7.10212688 | % |

In this case, we assume the Reader still transmit with maximum output power while the UE transmits with minimum output power.

In all, after the analysis above, we identify that there is no co-existence issue for these cases which is marked as green in following table.

Observation 3: For option 1-1, there is no co-existence issue for the cases marked as green in the table below:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | Device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |

For option1-2, the legacy NR UE move into the indoor, and the main difference is that the penetration loss is removed which make the interference more severe in some cases, so we only check whether the green cases are still free of co-existence issue. To save some pages, the calculations will not be listed here, and the details can be found in Annex.

Observation 4: For option 1-2, there is no co-existence issue for the cases marked as green in the table below:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | Device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |

Based on the analysis above, we propose:

Proposal 3: For option 1-1, the following cases in D1T1 can be marked as no co-existence issue and no need to perform the simulation:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR DL | Device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |

For option 1-2, the following cases in D1T1 can be marked as no co-existence issue and no need to perform the simulation:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: DL | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |
|  | R2D: DL CW2D and D2R: DL | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |

The last scenario is D1T1-B, where the CW is transmitted from outside topology. In our understanding, when CW is outside of topology, it has more flexibility to mitigate the interference, and D1T1-A1/D1T1-A2 is enough for co-existence study.

Observation 5: When CW is outside of topology, the interference can be mitigated by adjust the CW distribution. From co-existence study perspective, D1T1-A1/D1T1-A2 are enough.

Proposal 4: For D1T1-B (the CW is transmitted from outside topology), no need to perform co-existence simulation to save efforts

  - Coexistence assumption for D2T2

For D2T2, the interference is related to the distribution of intermediate UE which is hard to find a reasonable worst case to be analyzed, and the simulation is necessary no matter for option 2-1 or option 2-2. Therefore, in this part, we focus on the simulation assumption for D2T2.

First issue is the assumption of intermediate UE. In our understanding, the main advantage of D2T2 compared to D1T1 is the mobility of intermediate UE, which means the same coverage can be accomplished with less UEs, i.e., trading time for inventory efficiency. In the simulation, such characteristic should be reflected.

Observation 6: Compared to D1T1, the advantage of D2T2 is using less UEs to achieve same coverage while the inventory time will be enlarged.

Based on the analysis above, the intermediate UE can be disturbed uniformly and the total number can be same as D1T1, but for each snapshot, only a subset of UEs are activated. Furthermore, for each snapshot, there is only one nearest Tag are communicated with each activated UE.

Proposal 5: For the intermediate UE, the following assumptions are applied to D2T2 co-existence simulation:

- The intermediate UE uniformly distributed over the horizon area.

  - In InH-office, the total number is 12; In InF-DL, the total number is 18.

- For each #snapshot, only a subset of intermediate UEs are activated simultaneously

  - The number of activated UE is randomly between 0 and total UE number

- For each activated UE, only the nearest Tag is counted into co-existence evaluation.

Another issue is the distribution of legacy NR UE. In traditional co-existence evaluation, the activated UE is described as same as the number of BS beam, and same approach can be a starting point for legacy NR UE.

Observation 7: In traditional co-existence assumption, the activated UE number is assumed same as the number of BS beam.

Proposal 6: It is assumed that the legacy NR UE is uniformly distributed over the horizon area in the simulation, and the UE number is same as the number of BS beam.

- Conclusion

In this contribution, we provide our views on the co-existence issue.

Observation 1: The Reader received performance degradation due to CW itself can be ignored.

Observation 2: The IM3 produced by CW and D2R will overlap with D2R which will bring SNR degradation.

Proposal 1: For the D2R, only the interference of IM3, which produced by D2R and CW, is considered as the impact of CW in the co-existence evaluation, and the impact of CW itself can be ignored.

Proposal 2: The CDF of CW power at legacy NR receiver (including BS/UE) should be provided in co-existence evaluation and further check the degradation due to the receiver blocking.

Observation 3: For option 1-1, there is no co-existence issue for the cases marked as green in the table below:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | Device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |

Observation 4: For option 1-2, there is no co-existence issue for the cases marked as green in the table below:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | Device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | Reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |

Proposal 3: For option 1-1, the following cases in D1T1 can be marked as no co-existence issue and no need to perform the simulation:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR DL | Device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |
|  | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |

For option 1-2, the following cases in D1T1 can be marked as no co-existence issue and no need to perform the simulation:

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
|  | R2D: DL CW2D and D2R: DL | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |
|  | R2D: DL CW2D and D2R: DL | NR DL | Device |
|  |  | NR DL | Reader |
|  | R2D: UL CW2D and D2R: UL | Device | NR UL |

Observation 5: When CW is outside of topology, the interference can be mitigated by adjust the CW distribution. From co-existence study perspective, D1T1-A1/D1T1-A2 are enough.

Proposal 4: For D1T1-B (the CW is transmitted from outside topology), no need to perform co-existence simulation to save efforts

Observation 6: Compared to D1T1, the advantage of D2T2 is using less UEs to achieve same coverage while the inventory time will be enlarged.

Proposal 5: For the intermediate UE, the following assumptions are applied to D2T2 co-existence simulation:

- The intermediate UE uniformly distributed over the horizon area.

  - In InH-office, the total number is 12; In InF-DL, the total number is 18.

- For each #snapshot, only a subset of intermediate UEs are activated simultaneously

  - The number of activated UE is randomly between 0 and total UE number

- For each activated UE, only the nearest Tag is counted into co-existence evaluation.

Observation 7: In traditional co-existence assumption, the activated UE number is assumed same as the number of BS beam.

Proposal 6: It is assumed that the legacy NR UE is uniformly distributed over the horizon area in the simulation, and the UE number is same as the number of BS beam.

- References

- R4-2407008, LS on Ambient-IoT evaluation scenarios and assumptions, CMCC

- R4-2406714, WF on Ambient IoT in NR,	CMCC, RAN4#110bis

- Annex