3GPP TSG-RAN WG4 Meeting # 111 	R4-2408095

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for intermediate UE

Agenda Item:	10.13.3.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on the intermediate UE RF requirement.

- Discussion

  - System parameters

- Operating band

This depends on operator request, same as AIoT device, maybe we can take band n8 as the example band.

- Channel bandwidth

For the intermediate UE, since the in-band operation is prioritized, the channel bandwidth can be same as NR.

- Minimum receiver bandwidth

This requirement is also mentioned in [1]. Traditionally, we assume the bandwidth is same for Tx and Rx, but the spectrum for backscattering signal may different from OFDM, e.g., single sideband or double sideband, so RAN4 also need to discuss whether this requirement is needed or not.

  - Tx requirements

- Transmit output power

In [2], the MOP (maximum output power) for reader is defined as below:

However, the intermediate UE is hard to achieve such high output power, so we prefer the intermediate UE to follow the NR UE power class.

- Output power dynamic

For this part, only the new time mask for R2D need to be discussed. In [1], 4 types of time are defined:

RAN4 can take this as starting point and further discuss whether these requirement need to be defined.

- Transmit signal quality

The frequency error for Reader is defined in [2] as ±10 ppm which can be the starting point. The EVM also need to be defined, but the details need more discussion.

- Unwanted emissions

For the SEM, the unwanted emission for Reader in [2] can be starting point.

It is noted the SEM above is more stringent than UE IBE when only 1PRB is allocated, so RAN4 may not need to define a new IBE for AIoT signal. The definition of occupied bandwidth can be same as NR and the ACLR depends on co-existence study.

The SE is also defined in [2] which can be the starting point.

  - Rx requirements

In [2], for the Reader, 50% success rate is used as metric for the Rx requirement. RAN4 should first discuss what metric will be used for Rx requirement definition. All the discussion seems to be inefficient before we get the conclusion on this issue.

Proposal 1: It is suggested that success rate is used as performance metric for Rx requirement definition for intermediate UE.

Based on the analysis above, we have following proposal:

Proposal 2: The following table can be the starting point for intermediate UE RF requirement discussion:

| RF Requirement for intermediate UE |  |  |  |
| --- | --- | --- | --- |
| System parameter | Operating band |  | Band n8 as example band in SI |
|  | Channel bandwidth | Transmission bandwidth configuration | 180 kHz, FFS larger bandwidth for higher data rate |
|  |  | Guard band | FFS |
|  | Minimum receiver bandwidth |  | RAN4 need to discuss whether this requirement is need considering the spectrum of backscatter signal |
| Tx requirement | Transmit output power | Maximum output power | Follow NR UE power class |
|  | Output power dynamic | Transmit time mask | FFS following requirement: Transmit-to-Receive Turn-Around Time; Receive-to-Transmit Turn-Around Time; Transmit Power-On Ramp; Transmit Power-Down Ramp; |
|  | Transmit signal quality | Frequency error | ±10 ppm |
|  |  | EVM | FFS, ASK/PSK are considered |
|  |  | IBE | FFS whether new IBE is needed since the SEM in [2] is more stringent |
|  | Output RF spectrum emissions | Occupied bandwidth | Same as NR |
|  |  | SEM | SEM in [2] as starting point |
|  |  | ACLR | Depends on co-existence study |
|  |  | SE | SE in [2] as starting point |
| Rx requirement | RAN4 should first discuss and decide what metric will be used to represent the Rx performance, e.g., BLER, success rate, etc. |  |  |

- Conclusion

In this contribution, we provide our views on the RF requirement for AIoT BS.

Proposal 1: RAN4 discuss whether 50% success rate is also used for Rx requirement definition for AIoT BS.

Proposal 2: The following table can be the starting point for AIoT device RF requirement discussion:

| RF Requirement for AIoT device |  |  |  |
| --- | --- | --- | --- |
| System parameter | Operating band |  | Band n8 as example band in SI |
|  | Channel bandwidth | Transmission bandwidth configuration | 180 kHz, FFS larger bandwidth for higher data rate |
|  |  | Guard band | FFS |
|  | Minimum receiver bandwidth |  | RAN4 need to discuss whether this requirement is need considering the feasibility of RF filter |
| Tx requirement | Transmit output power | Maximum output power | -25 dBm/100kHz for frequency <900MHz; -18 dBm/100kHz for frequency >900MHz; FFS whether/how to convert radiated requirement to conducted requirement |
|  | Output power dynamic | Transmit OFF power | Same as NR, -40 dBm |
|  |  | Transmit time mask | FFS following requirement: Transmit-to-Receive Turn-Around Time; Receive-to-Transmit Turn-Around Time; Transmit Power-On Ramp; |
|  | Transmit signal quality | Frequency error | FFS |
|  |  | EVM | FFS, ASK/PSK are considered |
|  |  | IBE | FFS |
|  | Output RF spectrum emissions | Occupied bandwidth | Same as NR |
|  |  | SEM | Unwanted emission in [2] as starting point |
|  |  | ACLR | Depends on co-existence study |
|  |  | SE | Same as NR |
| Rx requirement | RAN4 should first discuss and decide what metric will be used to represent the Rx performance, e.g., BLER, success rate, etc. |  |  |

- References

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard

- ETSI EN 302 208 V3.4.1

- R4-240xxxx, Preliminary evaluation on AIoT coexistence, vivo, RAN4#111