# 3GPP TSG-RAN WG4 Meeting #111	R4-2408128

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	vivo

Title: 	Discussion on RF requirements for RedCap and eRedCap UE supporting FR1-NTN

Agenda Item:	10.15.2.1

Document for:	Approval

Introduction

In the last meeting, the work plan and several requirements for (e)RedCap NTN in FR1 Full Duplex FDD bands were decided for NR NTN phase 3. The WFs were captured in [1]. In this contribution, we will continue discussing the remaining issues in the last meeting and the UE RF requirements for (e)RedCap NTN in Half Duplex mode.

Discussion

Remaining issues in the last meeting

In the last meeting, we had following open issues not being fully discussed. In this contribution, we would like to share our views on these open items.

  - Whether the NTN RedCap and eRedCap UE supports the enhanced channel raster.

  - Whether to continue using maxNumberMIMO-LayersPDSCH to indicate more than 1Rx antenna port support or a new UE capability on number of Rx support which is independent of DL MIMO layer support needs to be introduced for NR NTN RedCap and eRedCap UEs.

  - Whether the pre-compensate function could be supported by NTN RedCap and eRedCap UE.

  - Whether any update is needed for NTN Redcap and eRedcap with respect to regulatory requirements with considering the feasibility of Redcap and eRedcap implementations. (Sony)

  - Whether other Rx requirements than REFSENS from NR NTN can be reused for NTN Redcap and eRedcap.

For open item 1, the FR1-NTN bands already support the enhanced channel raster as shown in Table 5.4.2.3-2. From our understanding, if RedCap UE or eRedCap UE are to support FR1 NTN bands, it can naturally support enhanced channel raster. And there will no extra RF or specification impact if NTN redcap and eRedCap UE supports the enhanced channel raster.

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n254 | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

For Open item 3, We think the frequency pre-compensate function needs to be supported if RedCap or eRedCap UE intends to support NTN to address the large Doppler effects. However, this function should be done in RAN1 and RAN2，instead of RAN4. Unless we see an impact on UE RF requirement impact, we suggest not to discuss it further.

For Open item 4, regulation is an important aspect for developing FR1 NTN requirements. For RedCap and eRedCap supporting NTN, it inherits requirements for NTN along with the regulation considerations. Only for 1Rx, REFSENS requirements needs to be evaluated due to the reduce Rx capability. From our understanding, if there is regulation update for NTN, NTN RedCap or eRedCap UE can inherit the requirements accordingly. We don’t need extra work for this issue.

One import aspect for NTN RedCap UE and eRedCap UE is reduced Rx capability. REFSENS requirements need to accommodate 1Rx since REFSENS is RX number dependent. For other Rx requirements of NTN, it is not impacted by 1Rx. Therefore, we suggest to reuse other Rx requirements from NTN for RedCap or eRedCap NTN.

UE RF requirements for (e)RedCap NTN in HD mode

According to the WID, HD-FDD can be started in Q3 if RAN1 confirmed there are no essential changes needed for their support.

- For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1]

  - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4]

However, there was an agreement in the last meeting about this work plan:

Sub-topic 1-3: HD-FDD (e)RedCap

Agreement: Start the discussions on the RF requirements for NTN RedCap with HD-FDD in the next meeting (RAN4#111).

We continue discussing the UE RF requirements for RedCap or eRedCap NTN in HD mode. For UE RF requirements for (e)RedCap NTN in Half-Duplex mode, the only focus is on REFSENS requirements. There are two considerations for HD REFSENS:

- Compared to FD-FDD, RF front end my adopt filters with switch for HD-FDD, instead of using duplexer; the insertion loss for HD-FDD may be smaller than FD-FDD due to the difference in RFFE.

- For HD-FDD, receiving DL will not have the interference from UL transmission;

Base on these two considerations, the REFSENS may need tightening for HD, compared with FD-FDD. However, the tightening values need to be studies case by case. In the discussion of REFSENS for RedCap UE, we had following agreements for deriving the requirements:

- The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in the table below.

Table 1. HD-FDD 5MHz REFSENS tightening from FD-FDD

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70, n91~n94 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

- HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS and n=2 for 30kHz SCS.and n=4 for 60kHz SCS

For 2Rx FR1-NTN FDD bands, the REFSENS is defined as follows:

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.8 |
| n256 | 30 |  | -96.6 | -94.6 | -94.0 |
|  | 60 |  | -97.0 | -94.9 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -99.5 | -96.3 | -94.5 |  |
| n254 | 30 |  | -96.6 | -94.6 |  |
|  | 60 |  | -97.0 | -94.9 |  |

According to above table 1, RFSENS for HD need to tightened by [0] dB for band n255 and [0.5]dB for band n256 and n254.

Then the REFSENS for FR1-NTN HD-FDD bands can be calculated as:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -94.3 |
| n256 | 30 |  | -97.1 | -95.1 | -94.5 |
|  | 60 |  | -97.5 | -95.4 | -94.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Proposal 1: The 2Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -94.3 |
| n256 | 30 |  | -97.1 | -95.1 | -94.5 |
|  | 60 |  | -97.5 | -95.4 | -94.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

For 1Rx, with additional Delta_1R 2.5dB, we can derive 1Rx REFSENS for FR1-NTN HD-FDD bands.

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.5 | -94.3 | -92.5 | -91.8 |
| n256 | 30 |  | -94.6 | -92.6 | -92.0 |
|  | 60 |  | -95.0 | -92.9 | -92.2 |
|  | 15 | --97.5 | -94.3 | -92.5 | -91.3 |
| n255 | 30 |  | -94.6 | -92.6 | -91.5 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
|  | 15 | -97.5 | -94.3 | -92.5 |  |
| n254 | 30 |  | -94.6 | -92.6 |  |
|  | 60 |  | -95.0 | -92.9 |  |

Proposal 2: The 1Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.5 | -94.3 | -92.5 | -91.8 |
| n256 | 30 |  | -94.6 | -92.6 | -92.0 |
|  | 60 |  | -95.0 | -92.9 | -92.2 |
|  | 15 | --97.5 | -94.3 | -92.5 | -91.3 |
| n255 | 30 |  | -94.6 | -92.6 | -91.5 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
|  | 15 | -97.5 | -94.3 | -92.5 |  |
| n254 | 30 |  | -94.6 | -92.6 |  |
|  | 60 |  | -95.0 | -92.9 |  |

For the Uplink configuration for reference sensitivity for HD-FDD, we suggest to reuse the configuration for FD-FDD.

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  | 24 | 36 | [36] | FDD |
|  | 60 |  | 10 | 18 | [18] |  |
|  | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  | 24 | 36 |  | FDD |
|  | 60 |  | 10 | 18 |  |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |

Conclusion

This contribution discusses some general issues for RedCap or eRedCap UE supporting FR1-NTN bands and the REFSENS requirements for FR1-NTN HD-FDD bands. The following proposals are made:

Proposal 1: The 2Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -94.3 |
| n256 | 30 |  | -97.1 | -95.1 | -94.5 |
|  | 60 |  | -97.5 | -95.4 | -94.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Proposal 2: The 1Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.5 | -94.3 | -92.5 | -91.8 |
| n256 | 30 |  | -94.6 | -92.6 | -92.0 |
|  | 60 |  | -95.0 | -92.9 | -92.2 |
|  | 15 | --97.5 | -94.3 | -92.5 | -91.3 |
| n255 | 30 |  | -94.6 | -92.6 | -91.5 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
|  | 15 | -97.5 | -94.3 | -92.5 |  |
| n254 | 30 |  | -94.6 | -92.6 |  |
|  | 60 |  | -95.0 | -92.9 |  |

Proposal 3: For the Uplink configuration for reference sensitivity for HD-FDD, it is suggested reuse the configuration for FD-FDD.

- References

[1] R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm, RAN4#110bis.