# 3GPP TSG-RAN WG4 Meeting #111	R4-2408309

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.4.1

Source:	China Telecom

Title:	Discussion on RRM core requirements maintenance for FR2 SCell activation delay reduction

Document for:	Discussion

- Introduction

In RAN4 #110bis meeting, RRM core requirements maintenance for FR2 SCell activation delay reduction were discussed and the WF [1] was approved.

In this paper, we present our views on RRM core requirements maintenance for FR2 SCell activation delay reduction issues captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 1-3: L3 reporting in multiple SCell activation - FFS - Option 1(HW): - For single SCell activation, the measurement report of the to-be-activated SCell should be valid, and the requirements applied the SCell is activated based on the report of the to-be-activated SCell (e.g. TCI configuration). - For multiple CC activation in the same FR2 band when more than one to-be-activated SCell are configured with servingCellMO, considering following two options: - Option 1-1: Only the latest result is considered as valid when the results from the same FR2 band are different (in terms of SSB index), and UE only reports the latest result to NW. - Option 1-2: Results from different Cells in the same FR2 band are also valid even the results are different. UE reports the results to NW, and it is up to NW which result to be used for SCell activation. - Option 2(Ericsson): - For multiple CC activation when more than one to-be-activated SCell are configured with servingCellMO, it is up to UE implementation to report CC of SCell to be activated result or the other CC (if it is latest) or both. - Option 3(Moderator): - For single SCell activation, the measurement report of the to-be-activated SCell should be valid, and the requirements applied the SCell is activated based on the report of the to-be-activated SCell (e.g. TCI configuration). (from Huawei option 1) - For multiple CC activation in the same FR2 band when more than one to-be-activated SCell are configured with servingCellMO, - It is up to UE implementation to report CC of SCell to be activated result or the other CC (if it is latest) or both. (from Ericsson option 2) - Results from different Cells in the same FR2 band are also valid even the results are different. UE reports the results to NW, and it is up to NW which result to be used for SCell activation. (from Huawei option 1-2)

For L3 reporting in single SCell activation, there is no ambiguity that the measurement report of the to-be-activated SCell should be valid, and the requirements applied the SCell is activated based on the report of the to-be-activated SCell (e.g. TCI configuration). For L3 reporting in multiple SCell activation, the multiple CC activation are in the same FR2 band when more than one to-be-activated SCell are configured with servingCellMO, UE reports the valid measurement results to NW, in our views, it’s up to UE implementation to report CC of SCell to be activated result or the other CC (if it is latest) or both. Therefore, UE might report measurement result from CC of SCell to be activated or other CC or both CCs, after UE reports the valid measurement results to NW, in our views, it’s up to NW to decide which result to be used for SCell activation. As a consequence, option 3 from Moderator can be a good consensus for this issue.

Proposal 1: For multiple CC activation in the same FR2 band when more than one to-be-activated SCell are configured with servingCellMO, it is up to UE implementation to report CC of SCell to be activated result or the other CC (if it is latest) or both, and after UE reports the results to NW, it is up to NW which result to be used for SCell activation.

- Conclusion

The contribution presents our views on RRM core requirements for FR2 SCell activation delay reduction, with the following proposals:

Proposal 1: For multiple CC activation in the same FR2 band when more than one to-be-activated SCell are configured with servingCellMO, it is up to UE implementation to report CC of SCell to be activated result or the other CC (if it is latest) or both, and after UE reports the results to NW, it is up to NW which result to be used for SCell activation.

- References

- R4-2406339, WF for [110bis][207] NR_RRM_enh3_part1, Apple, RAN WG4 Meeting #110bis, April 2024.