# 3GPP TSG-RAN WG4 Meeting #111	R4-2408310

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.4.3

Source:	China Telecom

Title:	Discussion on RRM performance requirements for FR2 SCell activation delay reduction

Document for:	Discussion

- Introduction

In RAN4 #110bis meeting, RRM performance requirements for FR2 SCell activation delay reduction were discussed and the WF [1] was approved.

In this paper, we present our views on RRM performance requirements for FR2 SCell activation delay reduction issues captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 2-1: How to configure M to have valid L3 report for FG31-1 TC Agreement: - Calculate the M values based on UE capability in the corresponding sections. Agreement: DCI transmission timing: cover two case: - Case 1: n+3ms+THARQ+M-k2 - FFS: UE pass condition is either UE to L1 or L3 report. - Case 2: n+[7]ms+ THARQ - where k2=1 with type A mapping and startSymbolAndLength (SLIV) = 42 (L=4, and S=0). DL scheduling for data starts from n+3ms. Issue 2-2: whether to verify requirement without L3 reporting for FG31-1 TC Agreement: - For all FG31-1 TCs, only verify the activation delay when L3 report is triggered. - FFS UE reporting L1 is also a pass condition.

For M configuration to have valid L3 report for FG31-1 TC, it was agreed in last meeting that DCI transmission timing cover two case, and Sub-test 1 and Sub-test 2 are defined in the endorsed CR [2]. In Sub-test 2, from n + 3 ms + M – k2, TE continuously schedules the PUSCH to UE on PCell, while M is defined in 8.3.17 and k2 = 1. During M period, L1 measurement result might be reported before receiving TCI activation command, then configuration message for TCI will be based on either L1-RSRP reporting or L3-RSRP reporting when UE report both L3-RSRP reporting and L1-RSRP reporting before receiving TCI activation command. Therefore, UE reporting L1 can also be a pass condition to verify the case that UE report both L3-RSRP reporting and L1-RSRP reporting before receiving TCI activation command.

Proposal 1: UE reporting L1 can also be a pass condition to verify the case that UE report both L3-RSRP reporting and L1-RSRP reporting before receiving TCI activation command.

- Conclusion

The contribution presents our views on RRM performance requirements for FR2 SCell activation delay reduction, with the following proposals:

Proposal 1: UE reporting L1 can also be a pass condition to verify the case that UE report both L3-RSRP reporting and L1-RSRP reporting before receiving TCI activation command.

- References

- R4-2406339, WF for [110bis][207] NR_RRM_enh3_part1, Apple, RAN WG4 Meeting #110bis, April 2024.

- R4-2406508, Draft Big CR to TS 38.133 on performance requirements for Even Further RRM enhancement for NR and MR-DC, Apple, OPPO, RAN WG4 Meeting #110bis, April 2024.