# 3GPP TSG-RAN WG4 Meeting #111                                   R4-2408313

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.24.1

Source:	China Telecom

Title:	Discussion on RRM core requirements maintenance for NES

Document for:	Discussion

- Introduction

In RAN4 #110bis meeting, RRM core requirements for inter-band SSB-less SCell activation for NES was discussed and the WF [1] was approved. In this paper, we present our views about RRM core requirements for NR network energy saving.

- Discussion

Core requirements maintenance

Status in the WF [1]:

Issue 1-1-5: Intra-band non-contiguous CA - Proposals - Option 1 For FR1 intra-band non-contiguous CA, reuse the SSB-less SCell activation delay requirement defined for FR1 collocated inter-band CA with the same RTD side condition i.e. RTD within CP (Nokia, Intel) - Option 1a: Introduce the optional with capability signalling with per FS granularity for UE supporting intra-band NCCA SSB-less SCell operation in Rel-18. (Intel) - Option 2: The delay requirements of SSB-less SCell activation for intra-band non-contiguous CA can be same as that for legacy intra-band contiguous CA provided the UE impelmentation architecture are same for the two scenarios. The exact side conditions for intra-band non-contiguous CA can be further discussed. (ZTE) - Option 3: RAN4 will not consider SSBless SCell operation for intra-band NCCA in R18. (QC) - Option 4: (Apple, Huawei) - For FR1 intra-band NCCA with SSB-less operation, EPRE difference at UE side shall be NOT larger than 6dB - For FR1 intra-band NCCA with SSB-less operation, RTD between the target SCell and the intra-band NCCA collocated reference serving cell can be within CP - Option 5: (CMCC) - Define two sets of requirement and side condition, reuse the intra-band contiguous CA case and inter-band CA case respectively. - Define two UE capabilities for intra-band NCCA scenario, which corresponding to two set of requirements respectively, the granularity could be per FS indication - Option 6: - Specify only assuming separate chains and reuse largely the requirements specified for inter-band cases; - Specify UE optional capability signalling for intra-band NCCA SSB-less SCell operations in a similar way as for inter-band cases; - UE with single chain implementation does not indicate support for intra-band NCCA SSB-less SCell operations and does not need to meet the requirements. - Option 7: - Specify two sets of requirements respectively assuming single and separate chains, where one of the sets reuse largely the requirements specified for inter-band cases; - Specify UE optional capability signalling for two different implementations separately; - Different UE implementations are required to meet different sets of requirements correspondingly. - Other options are not precluded

Regarding FR1 Intra-band non-contiguous CA with SSB-less operation, it was agreed in previous meeting that further study whether we can reuse or not the SSB-less activation delay requirement specified for FR1 inter-band CA, and the study does not impact the completion timeline of the WI. In our understanding, UE implementation in FR1 Intra-band non-contiguous CA can be based on single RF chain or separate RF chains, which is different from UE implementation in FR1 inter-band CA based on separate RF chains, therefore, we think the SSB-less activation delay requirement specified for FR1 inter-band CA cannot be directly reused for FR1 Intra-band non-contiguous CA with SSB-less operation.

Observation 1: Regarding FR1 Intra-band non-contiguous CA with SSB-less operation, it was agreed in previous meeting that further study whether we can reuse or not the SSB-less activation delay requirement specified for FR1 inter-band CA, and the study does not impact the completion timeline of the WI.

Observation 2: UE implementation in FR1 Intra-band non-contiguous CA can be based on single RF chain or separate RF chains, which is different from UE implementation in FR1 inter-band CA based on separate RF chains.

Proposal 1: The SSB-less activation delay requirement specified for FR1 inter-band CA cannot be directly reused for FR1 Intra-band non-contiguous CA with SSB-less operation.

- Conclusion

The contribution presents our views on RRM core requirements for NR network energy saving, with the following observations and proposals:

Observation 1: Regarding FR1 Intra-band non-contiguous CA with SSB-less operation, it was agreed in previous meeting that further study whether we can reuse or not the SSB-less activation delay requirement specified for FR1 inter-band CA, and the study does not impact the completion timeline of the WI.

Observation 2: UE implementation in FR1 Intra-band non-contiguous CA can be based on single RF chain or separate RF chains, which is different from UE implementation in FR1 inter-band CA based on separate RF chains.

Proposal 1: The SSB-less activation delay requirement specified for FR1 inter-band CA cannot be directly reused for FR1 Intra-band non-contiguous CA with SSB-less operation.

- References

- R4-2406301, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN WG4 Meeting #110bis, April 2024.