3GPP TSG-RAN WG4 # 111	R4-2408631

Fukuoka, Japan, May 20 – May 24, 2024

Agenda item:	7.11.2.2

Source:	MediaTek Inc.

Title:	Simulation results of Advanced receiver to cancel inter-user interference for MU-MIMO

Document for:	Discussion

# Introduction

In RAN#103, the status report of Rel-18 WI of NR demodulation performance evolution, the work objective for advanced receiver to cancel inter-user interference for MU-MIMO is defined [1].  The work objective is to evaluate and specify advanced receiver to cancel inter-user interference for MU-MIMO. This work is split into two phases where the first phase studies the performance gain, reference receiver assumption, interference modelling, testability, required signalling overhead, as well as impact on other WGs. The initial receiver candidates are E-MMSE-IRC and R-ML. In the second phase it is expected to specify PDSCH demodulation requirements under MU-MIMO scenario with advanced receiver.

# Simulation parameters and results

In this chapter we are going to discuss simulation parameters in Chapter 2.1, simulation results in Chapter 2.2 and simulation analysis in Chapter 2.3.

# 2.1 Simulation parameters

In this chapter we are going to show updated simulation results with short analysis.

We have provided simulation results of agreed FDD and TDD scenarios with 2Rx and 4Rx. Also, we are assuming the same scrambling ID for DMRS sequence for all UEs. We have simulated only single co-scheduled UE scenarios for this meeting. Also, we have assumed full bandwidth allocation of all UEs and the same QCL assumption in all DMRS ports. The evaluation metric is SNR at 70% of maximum throughput.

We have assumed co-scheduled UE modulation order information as known and detected by UE.

In this document we compare performance of 3 receiver types as agreed in WF [2], namely

- R-ML with genie modulation order information (Rel-18 advanced receiver)

- R-ML with modulation order blind detection (Rel-18 advanced receiver)

- MMSE-IRC (Rel-17 reference receiver)

# 2.2 Simulation results

In previous meeting we agreed extensive simulation plan for this meeting. All agreed tests are run, and simulation results are shown in Table 2-1 for known modulation order and in Table 2-2 for Modulation Order Blind Detection (MOBD). We provide 2 simulation results for known modulation order tests and 3 simulation results for MOBD tests. Genie MO R-ML is using know modulation order information, provided with DCI signalling, for R-ML receiver. MOBD R-ML is using blind detection of modulation order information for R-ML receiver. MMSE-IRC is following agreed Rel-17 reference receiver implementation.

Table 2-1: Simulation results of MU-MIMO scenarios with known modulation order.

| Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Genie MO R-ML | MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 15kHz SCS | 10MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS13 | QPSK | 13.6 | 20.7 |
| 2 |  |  |  | 2T4R |  |  |  |  |  | 12.2 | 22.9 |
| 3 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM | 16.9 | 17.2 |
| 4 | TDD 30kHz SCS | 40MHz | 1+1 | 2T2R |  | TDLC300-100 | ULA medium | MCS 13 | QPSK | 14.3 | 23.0 |
| 5 |  |  |  | 2T4R |  |  |  |  |  | 13.1 | 32.9 |
| 6 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 17 | 16QAM | 17.2 | 17.8 |

Table 2-2: Simulation results of MU-MIMO scenarios with modulation order blind detection.

| Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Genie MO R-ML | MOBD R-ML | MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | FDD 10kHz SCS | 10MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | 13.6 | 15.5 | 20.7 |
| 8 |  |  |  | 2T4R |  |  |  |  |  | 12.2 | 15.1 | 22.9 |
| 9 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM | 20.7 | 22.0 | 29.2 |
| 10 |  |  |  | 2T4R |  |  |  |  |  | 20.0 | 21.7 | 32.4 |
| 11 |  |  |  | 2T2R |  |  | ULA Low | MCS 13 | QPSK | 11.4 | 12.5 | 16.1 |
| 12 |  |  |  | 2T4R |  |  |  |  |  | 6.8 | 7.4 | 9.0 |
| 13 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK | 10.9 | 11.8 | 12.8 |
| 14 |  |  |  |  |  |  | XP medium |  |  | 12.1 | 13.3 | 14.5 |
| 15 | TDD 30kHz SCS | 40MHz | 1+1 | 2T2R | Orthogonal | TDLC300-100 | ULA medium | MCS 13 | QPSK | 14.3 | 15.8 | 23.0 |
| 16 |  |  |  | 2T4R |  |  |  |  |  | 13.1 | 15.2 | 32.9 |
| 17 |  |  |  | 2T2R |  |  |  | MCS 17 | 16QAM | 21.1 | 22.3 | 30.7 |
| 18 |  |  |  | 2T4R |  |  |  |  |  | 20.8 | 22.6 | 39.9 |
| 19 |  |  |  | 2T2R |  |  |  | MCS 13 | QPSK | 11.8 | 12.6 | 17.2 |
| 20 |  |  |  | 2T4R |  |  |  |  |  | 7.1 | 7.5 | 9.2 |
| 21 |  |  | 2+2 | 4T4R |  | TDLA30-10 | ULA Low | MCS 13 | QPSK | 11.2 | 12.0 | 13.1 |
| 22 |  |  |  |  |  |  | XP medium |  |  | 12.5 | 13.5 | 14.9 |

# 2.3 Simulation analysis

We can do several observations from these simulations. These are repeated also in discussion document.

Observations of known modulation order tests

Observation #1: All Rank 1+1 tests have enough gains over baseline receiver and SNR is at reasonable level. Tests are sufficient in our view.

Observation #2: All Rank 2+2 tests have only negligible gains over baseline receiver and SNR is at reasonable level. Test usefulness is questionable but 4Rx is properly tested in 2T4R test.

Observations of modulation order blind detection tests

Observation #3: All Rank1+1 tests with MOBD (7, 8, 15, 16) matching agreed known modulation order tests (1, 2, 4, 5) have enough gains over baseline receiver and SNR is at reasonable level. Tests are sufficient in our view.

Observation #4: Rank1+1 tests with MOBD using low antenna correlation with 4 receiver antennas have limited gains over baseline receiver compared to other options.

Observation #5: All Rank 2+2 tests with MOBD have only negligible gains over baseline receiver and SNR is at reasonable level. Test usefulness is questionable but 4Rx can be properly tested in 2T4R test.

# Conclusion

In this paper we provided simulation results with analysis and observations on the advanced receiver to cancel inter-user interference for MU-MIMO. The following observations are made:

Observations of known modulation order tests

Observation #1: All Rank 1+1 tests have enough gains over baseline receiver and SNR is at reasonable level. Tests are sufficient in our view.

Observation #2: All Rank 2+2 tests have only negligible gains over baseline receiver and SNR is at reasonable level. Test usefulness is questionable but 4Rx is properly tested in 2T4R test.

Observations of modulation order blind detection tests

Observation #3: All Rank1+1 tests with MOBD (7, 8, 15, 16) matching agreed known modulation order tests (1, 2, 4, 5) have enough gains over baseline receiver and SNR is at reasonable level. Tests are sufficient in our view.

Observation #4: Rank1+1 tests with MOBD using low antenna correlation with 4 receiver antennas have limited gains over baseline receiver compared to other options.

Observation #5: All Rank 2+2 tests with MOBD have only negligible gains over baseline receiver and SNR is at reasonable level. Test usefulness is questionable but 4Rx can be properly tested in 2T4R test.

# Reference

- RP-240348, “Status report for NR demodulation requirement evolution”, China Telecom

- R4-2406114, “WF on advanced receiver for MU-MIMO scenario”, China Telecom