3GPP TSG-RAN WG4 Meeting #111    	R4-2408720

Fukuoka, Japan, 20th – 24th April, 2024

Title:	Larger future specification structure enhancements for UE RF

Agenda item:	12.1.1.3

Source:	Nokia,

Document for:	Discussion

# 1	Background

There is ongoing RAN task in RAN4 which is called Specification quality improvement (RP-240782) and related to that a dedicated AI “Larger specification structure enhancement” is allocated. In this contribution we discuss how we think that UE RF specifications should be improved when the time scope is beyond REL20. These solutions that we discuss in this contribution are made based on lessons learned on 4G and especially 5G. Without a better word we will call these future enhancements as 6G specifications from now on.

# 2 	Discussion

We divide the discussion into three chapters. First, we discuss how to define single band operation, then we discuss 6G CA and DC and lastly 6G interworking with other radios.

## 2.1	Single band specifications

### 2.1.0	General

Current way of defining single band operation in terms of what requirements are defined is quite good and no big problems are identified. Collection of current requirements and commentary for suitability for 6G is listed in Table 1. Table 1 content mainly is targeted for FR1 spectrum bands, but some aspects are of course applicable for FR2 spectrum bands.

Table 1: Single band operation UE RF requirements

|  | Requirement | 6G Spec | Details |
| --- | --- | --- | --- |
| System parameters | Operating bands | yes | Follow the principle of NR. Assing a prefix to band number similarly as in NR where "n" was used. For 6G for example "s" could be used. If band is same as NR or LTE or 3G then same number is used. I=B1=n1=s1. |
|  | UE CH bandwidth | yes | What should be avoided is the need to add new channel bandwidths after initial 6G release. Hence, at minimum, NR FR1 channel bandwidths need to be defined from the start of 6G. |
|  | Asymmetric CH BWs | yes | Built in from the beginning of 6G |
|  | Channel spacing | yes | Needs discussion how to define |
|  | Channel raster | yes | Needs discussion how to define Channel raster is defined in RAN4, though RAN1/RAN2 specs use different reference frequencies. Applicability of channel raster or enhanced channel raster needs to be clarified in a better way if it is about SIB1, UE specific CBW and/or BWP. The relationship with point A and frequency offset should be also clarified. |
|  | Sync raster | yes | Needs discussion how to define |
|  | Tx-Rx separation | yes | Similarly, as in NR |
| Transmitter requirements | Maximum output power | yes | Power class 2 should be the default power class. Support power boosting (beyond the nominal PC class) for low-PAPR waveforms from initial 6G release |
|  | MPR | yes | NR MPR concept is good baseline, but it should be further optimized |
|  | A-MPR | yes | NR A-MPR concept is not optimum. It needs rethinking in order to minimize A-MPR. |
|  | Pcmax | yes | Simplification compared to NR Pcmax should be made. |
|  | dTib | no | Relaxation is not allowed in order to maximize output power |
|  | Min power | yes | Similarly as in NR |
|  | OFF power | yes | Similarly as in NR |
|  | ON/OFF time mask | yes | Similarly as in NR |
|  | Power control | yes | Similarly as in NR |
|  | Frequency error | yes | Similarly as in NR |
|  | Error Vector Magnitude | yes | Similarly as in NR |
|  | Carrier leakage | yes | Some enhancement compared to NR to increase output power |
|  | IQ Image | yes | Some enhancement compared to NR to increase output power |
|  | In-band emissions | yes | Room for improvements exist. |
|  | Occupied bandwidth | maybe | If not needed can be skipped, depends on regulation |
|  | Spectrum emission mask | yes | Similarly as in NR |
|  | Additional spectrum emission mask | yes | Similarly as in NR based on regulation |
|  | ACLR | yes | Similarly as in NR |
|  | UTRA ACLR | no | Not defined in 6G. |
|  | General spurious emissions | yes | Similarly as in NR |
|  | Spurious emissions for UE co-ex | yes | Similarly as in NR |
|  | Additional spurious emissions | yes | Similarly as in NR based on regulation |
|  | Transmit intermodulation | maybe | Needs discussion if needed in 6G |
| Receiver requirements | Reference sensitivity | yes | Similarly as in NR |
|  | ΔRIB,c | no | If relaxation is specified, then single carrier REFSENS needs to be tightened from NR. |
|  | Maximum input level | yes | Similarly as in NR |
|  | Adjacent channel selectivity | yes | Similarly as in NR |
|  | In-band blocking | yes | Blocking requirements are needed. Are those divided as in-band and out of band and narrow band can be discussed or is one set of requirements enough |
|  | Out-of-band blocking | yes | Blocking requirements are needed. Are those divided as in-band and out of band and narrow band can be discussed or is one set of requirements enough |
|  | Narrow band blocking | maybe | Blocking requirements are needed. Are those divided as in-band and out of band and narrow band can be discussed or is one set of requirements enough |
|  | Spurious response | yes | Could be combined with blocking requirement |
|  | Wide band Intermodulation | yes | Similarly as in NR |
|  | Spurious emissions | yes | Similarly as in NR |

Next, we will talk through also some key requirements if we think that some deviation from LTE/NR style is needed.

### 2.1.1 Operating band numbers

NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G use same band number if frequency arrangement is same but NR has prefix on “n”. For 6G we could re-use prefix concept and just pic a new letter, for example “s”.

| Band number space | 4G FR1-1 |  | 5G FR1-1 |  | 6G FR-1 and FR1-2 |  | 5G/6G FR2-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band # | Duplex | Band # | Duplex | Band # | Duplex | Band # | Duplex |
| Legacy FR-1 FDD | 1 - 32 | FDD | n1 - n32 | FDD re-farm | s1 - s32 | FDD re-farm | N/A |  |
| Legacy FR-1 TDD | 33 - 64 | TDD | n33 - n64 | TDD re-farm | s33 - s64 | TDD re-farm |  |  |
| New 4G/5G/6G FR1-1 and FR1-2 | 65 - 256 | all duplex modes | n65 - n256 | all duplex modes | s65 - s256 | all duplex modes + re-farm |  |  |
| 5G/6G NTN | N/A |  | <--n256 |  | <--s256 |  | <-n512 |  |
| 5G/6G FR2-1 | N/A |  |  |  |  |  | n257 -> s257 -> | TDD |

### 2.1.2	UE channel bandwidths

The need of listing individual applicable channel bandwidths needs to be discussed. Earlier it has been seen necessary to bound some limits for amount on necessary specifications hence fully arbitrary bandwidths are not allowed. What should be avoided is the need to add new channel bandwidths after initial 6G release as this has been somewhat problematic in NR. Hence at least NR FR1 channel bandwidths widths need to be specified i.e.3, 5, 10, 15…90, 95 and 100 MHz, in increments of 5 MHz (with 3MHz being the exception to the rule needed for specific low-bandwidth deployments). What to define beyond 100 MHz needs discussion but for FR1 up to 200 MHz can be foreseen. Additionally, no other table should list channel bandwidths, but reference the only table that contains the definitions.

### 2.1.3	Asymmetric CH BWs

There is clearly an operator need for asymmetric CH BWs based on experience of NR and LTE. This should be made possible in 6G without a need for asymmetric channel bandwidth combination sets.

### 2.1.4	Maximum output power i.e. Power classes

Power class 2 should be the default power class. Support power boosting (beyond the nominal PC class) for low-PAPR waveforms from initial 6G release as native feature. Power Class framework should be enhanced at least to enable better clarity/knowledge at the gNB.

### 2.1.5	MPR/A-MPR

NR MPR concept with inner and outer is quite good although the table listing MPR values gives excessive reduction especially for CP-OFDM. Further work on maximizing output power is deemed necessary for 6G.

A-MPR concept is largely inherited from 4G and is not good anymore. It is too complex and still do not maximize the output power, so the A-MPR concept must be enhanced.

One big enhancement on improving the quality of MPR and especially A-MPR is that number of possible waveforms is reduced from NR. DFT-S-OFDM is for coverage and CP-OFDM is for MIMO. There is no need to have such a big overlap with DFT-S-ODFM and CP-OFDM in terms of SNR/PAPR. Hence DFT-S-ODFM should be specified only for low PARP waveforms such as PI/2-PBSK and QPSK and not 16-QAM and higher.

### 2.1.6	Pcmax

Very complex in LTE and NR partly because new features/functionalities are added afterwards which breaks the fluent structure of the clause. Improvement needed.

### 2.1.7	dTib (and the band edge relaxation, note3)

Should not be defined in 6G because of specification simplicity and UE performance point of view. More effort is needed from UE implementation to meet the requirements without relaxation to output power in single CC mode in order not to reduce coverage.

### 2.1.8	Carrier leakage and IQ-Image

Important for modulator impairments. Improvement over NR is necessary, similarly as from LTE to NR.

### 2.1.8	ALCR/SEM

Spectrum emission mask is needed for regulator purposes and ACLR is needed for co-existence. UTRA ACLR should not be defined anymore.

### 2.1.9	REFSENS and dRib

This can be seen as a package. From specification simplicity and UE performance point of view dRib should be avoided. If dRib is not defined then there is less pressure to tighten the baseline REFSENS. If dRib is allowed, then big improvement to LTE/NR REFSENS must happen.

### 2.1.10	In-band blocking, out-of-band blocking, narrow-band blocking and spurious response

Discussion should happen if In-band blocking, out-of-band blocking, narrow band blocking and spurious response clauses can be combined into one clause in 6G.

## 2.2	Carrier aggregation and dual connectivity

### 2.2.1	General

NR carrier aggregation and dual connectivity requirements are defined in same specifications as single band requirements i.e. 38.101-1 and 38.101-2. This is good arrangement as carrier aggregation clauses refer often to single carrier requirement clauses and dual connectivity clauses refers to carrier aggregation requirement clauses.

Current way of defining CA/DC operation in terms of what requirements are defined is quite good in some respects but there are also some big problems identified. Collection of current requirements and commentary for suitability for 6G is listed in Table 2.

|  | Requirement | 6G Spec | Details |
| --- | --- | --- | --- |
| System parameters | Operating bands for CA | yes | Is useful for necessary band combination restrictions etc. |
|  | NR CA bandwidth classes | yes | Unless new naming convention is established for CA Configurations |
|  | Channel spacing for CA | yes | Needs discussion how to define |
|  | Channel raster | yes | Refers to single CC |
|  | Sync raster | yes | Refers to single CC |
|  | Tx-Rx separation | yes | Refers to single CC |
|  | CA Configurations Intraband | yes | No BCSs, BW capabilities are signalled. |
|  | CA configurations Interband | no/maybe | Band combinations and BW capabilities per band are signalled. |
| Transmitter requirements | Maximum output power | yes | Needs discussion how to define |
|  | MPR | yes | NR MPR concept is good baseline, but it should be further optimized |
|  | A-MPR | yes | NR A-MPR concept is not optimum. It needs rethinking to minimize A-MPR. |
|  | Pcmax | yes | Simplification compared to NR Pcmax should be made. |
|  | dTib | no | Relaxation is not allowed in order to maximize output power |
|  | Min power | yes | Refers to single CC |
|  | OFF power | yes | Refers to single CC |
|  | ON/OFF time mask | yes | Refers to single CC |
|  | Power control | yes | Refers to single CC |
|  | Frequency error | yes | Refers to single CC |
|  | Error Vector Magnitude | yes | Similarly, as in NR |
|  | Carrier leakage | yes | Some enhancement compared to NR in order to increase output power |
|  | IQ Image | yes | Some enhancement compared to NR in order to increase output power |
|  | In-band emissions | yes |  |
|  | Occupied bandwidth | maybe | If not needed can be skipped, depends on regulation |
|  | Spectrum emission mask | yes | Similarly, as in NR |
|  | Additional spectrum emission mask | yes | Similarly, as in NR based on regulation |
|  | ACLR | yes |  |
|  | UTRA ACLR | no | Not defined in 6G. |
|  | Spurious emissions | yes | Similarly, as in NR based on regulation |
|  | Spurious emissions for UE co-ex | yes | Similarly, as in NR i.e. listing regulator requirements only or just define as normative text. |
|  | Additional spurious emissions | yes | Similarly, as in NR based on regulation |
|  | Transmit intermodulation | maybe | Needs discussion if needed in 6G |
| Receiver requirements | Reference sensitivity | yes | Similarly, as in NR |
|  | ΔRIB,c | no | If relaxation is specified then single carrier REFSENS needs to tightened from NR. |
|  | MSD due to UL harmonic interference | no | Not defined in 6G. |
|  | MSD due to intermodulation interference due to 2UL CA | no | Not defined in 6G. |
|  | MSD due to cross band isolation for CA | no | Not defined in 6G. |
|  | Lower-MSD requirements for inter-band CA | no | Not defined in 6G. |
|  | Maximum input level | yes | How to define depends on if CA bandwidth classes are used |
|  | Adjacent channel selectivity | yes | How to define depends on if CA bandwidth classes are used |
|  | In-band blocking | yes | Blocking requirements are needed. Are those divided as in-band and out of band can be discussed or is one set of requirements enough and depends on if CA bandwidth classes are sued |
|  | Out-of-band blocking | yes | Blocking requirements are needed. Are those divided as in-band and out of band can be discussed or is one set of requirements enough and depends on if CA bandwidth classes are sued |
|  | Narrow band blocking | maybe | Needs discussion if needed in 6G |
|  | Spurious response | yes | Could be combined with blocking requirement |
|  | Wide band Intermodulation | yes | Similarly, as in NR |
|  | Spurious emissions | yes | Similarly, as in NR |
|  | Power imbalance for CA | yes | Part of 6G specifications from the beginning |

### 2.2.2	Operating bands for CA

Is a useful tool to have as conditions or restrictions can be added for a band combination level instead of CA configuration level.

### 2.2.3		CA bandwidth classes

Was useful and usable in LTE, was needed in NR but it turned out that due to many reasons in the end far too many CA bandwidth classes had to be defined due to limitation on UE and BS hardware capabilities. This led to long tables of CA configurations. Especially bad the situation is in 38.101-2 as spectrum blocks are wide, for example 800 MHz or 1600 MHz but CA configurations must be specified with 100 MHz carriers. This leads to long list of necessary fallbacks that must be captured into specifications.

Would be good if simpler solution is established for capturing requirements for individual CA configurations.

### 2.2.4 	Bandwidth combination set

Bandwidth combination set were introduced in REL10 for LTE due to chipset limitations to support some larger channel bandwidth combinations. It has also been used to limit possible bandwidth combinations in order to reduce testing burden. This may have worked in a beginning but later and especially in NR where new/additional channel bandwidths were introduced also new BCSs had to be introduced. Additionally, due to the concept of fallback band combinations in RAN2, UE has to anyway always support compatible bandwidths in all of its fallback BCs than it does with the parent BC, which makes the process of defining the BCSs in specifications somewhat error-prone. To overcome before mentioned issues a BCS4/5 concept was introduced i.e. if UE signals BCS4 or 5 it supports all bandwidth combinations which are relevant to the bands in question. Note some additional maximum/minimum signalling is associated with BCS4/5.

In 6G no BCSs should be defined, instead UE (minimum/maximum bandwidth) capabilities are signalled to the network.

### 2.2.5	Maximum output power for CA

Similarly, as for single band operation PC2 should be the baseline per band. No artificial band combination power classes should be introduced, instead power classes apply per band and can be used to the maximum given that SAR is met. Power Class framework should be enhanced at least to enable better clarity/knowledge at the gNB.

### 2.2.6	MPR/A-MPR/Pcmax/dTib

What was written for single band operation hold also here with an emphasis that MPR and A-MPR needs to be defined better in order not to restrict output power unnecessarily.

### 2.2.7	REFSENS and dRib

This can be seen as a package. From specification simplicity and UE performance point of view dRib should be avoided. If dRib is not defined, then there is less pressure to tighten the baseline REFSENS. If dRib is allowed then big improvement compared to LTE/NR REFSENS must happen.

### 2.2.8		REFSENS exception

NR specifications have many different forms of reference sensitivity exceptions which we will call MSD from now on. There are many MSD tables which are in following clauses in 38.101-1: MSD due to UL harmonic interference, MSD due to intermodulation interference due to 2UL CA, MSD due to cross band isolation for CA, Lower-MSD requirements for inter-band CA. There are also some MSDs that need separate relaxation value for higher UE power classes.

Defining MSD is complicated and time consuming. It has also a price tag which is not small as a lot of effort is put into defining MSD number which needs also to be tested. This MSD number is based usually on an allocation that produces worst-case MSD. These allocations are for example 1RB+1RB which are not used in real deployments. For all other allocations UEs perform better than defined MSD, note that also for allocation that is exactly defined for MSD test case UEs will typically perform much better than the requirement. We do not see any value on having this number in the specifications.

### 2.2.9	Power imbalance for CA

RAN4 has defined non-collocated requirements for some overlapping bands combinations. It is foreseen that this kind of optional capabilities are needed from the start for 6G. By doing it from the beginning confusing signalling extensions can be avoided.

## 2.3 Multi-RAT interworking

### 2.3.1	General

It is not known what kind of multi-RAT connectivity options will be standardized for 6G. From RAN4 specifications point of view especially the NSA band combinations have been difficult from various aspects merely because of the huge number of combinations including EN-DC, NE-DC, FR1-FR2 CA. Big number of combinations have created issues and workload because of numerous WIs, avalanche of TRs, request process, block approval process, post meeting process etc. Some NSA combinations are  also technically very challenging (Intraband EN-DC) to define although benefits are highly questionable. Quite many NSA combinations also need tedious MSD work.

We know for a fact that number of NR SA and NSA combinations are counted in tens of thousands at the time of writing. If RAN4 would need to combine all those with new 6G bands that would multiply number of combinations into hundreds of thousands. Not even the ongoing effort for database solution would save RAN4 from stagnation.

In our opinion no NSA RAN4 requirements are needed for 6G.  Current NR requirements including EN-DC apply for NR part in case of NSA with 6G and 6G SA requirements would apply for 6G part but no NR+6G requirements such as MSD, BCS, dTib, dRib etc are defined.

# 3	Conclusion

In this contribution we have discussed larger future specification structure enhancements for UE RF.

Observation 1: Current way of defining single band operation in terms of what requirements are defined is quite good and no big problems are identified.

Observation 1-1: NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G use same band number if frequency arrangement is same but NR has prefix on “n”. For 6G we could re-use prefix concept and just pic a new letter, for example “s”.

Observation 1-2: At least NR FR1 channel bandwidths need to be specified i.e. 3, 5, 10, 15…90, 95 and 100 MHz, in increments of 5 MHz (with 3MHz being the exception to the rule needed for specific low-bandwidth deployments). What to define beyond 100 MHz needs discussion but for FR1 up to 200 MHz can be foreseen. Additionally, no other table should list channel bandwidths, but reference the only table that contains the definitions.

Observation 1-3: There is clearly an operator need for asymmetric CH BWs based on experience of NR and LTE. This should be made possible in 6G without a need for asymmetric channel bandwidth combination sets.

Observation 1-4: Power class 2 should be the default power class. Support power boosting (beyond the nominal PC class) for low-PAPR waveforms from initial 6G release as native feature.

Observation 1-5: NR MPR concept with inner and outer is quite good although the table listing MPR values gives excessive reduction especially for CP-OFDM. Further work on maximizing output power is deemed necessary for 6G. A-MPR concept is largely inherited from 4G and is not good anymore. It is too complex and still do not maximize the output power, so the A-MPR concept must be enhanced.

Observation 1-6: Very complex in LTE and NR partly because new features/functionalities are added afterwards which breaks the fluent structure of the clause. Improvement needed.

Observation 1-7: Important for modulator impairments. Improvement over NR is necessary, similarly as from LTE to NR.

Observation 1-8: Spectrum emission mask is needed for regulator purposes and ACLR is needed for co-existence. UTRA ACLR should not be defined anymore.

Observation 1-9: This can be seen as a package. From specification simplicity and UE performance point of view dRib should be avoided. If dRib is not defined then there is less pressure to tighten the baseline REFSENS. If dRib is allowed, then big improvement to LTE/NR REFSENS must happen.

Observation 1-10: Discussion should happen if In-band blocking, out-of-band blocking, narrow band blocking and spurious response clauses can be combined into one clause in 6G.

Observation 2: NR carrier aggregation and dual connectivity requirements are defined in same specifications as single band requirements i.e. 38.101-1 and 38.101-2. This is good arrangement as carrier aggregation clauses refer often to single carrier requirement clauses and dual connectivity clauses refers to carrier aggregation requirement clauses.

Observation 2-1: Operating bands for CA is a useful tool to have as conditions or restrictions can be added for a band combination level instead of CA configuration level.

Observation 2-2: 	CA bandwidth classes: would be good if simpler solution is established for capturing requirements for individual CA configurations.

Observation 2-3: In 6G no BCSs should be defined, instead UE (minimum/maximum bandwidth) capabilities are signalled to the network.

Observation 2-4: Similarly, as for single band operation PC2 should be the baseline per band. No artificial band combination power classes should be introduced, instead power classes apply per band and can be used to the maximum given that SAR is met.

Observation 2-5: MPR/A-MPR/Pcmax/dTib: What was written for single band operation hold also here with an emphasis that MPR and A-MPR needs to be defined better in order not to restrict output power unnecessarily.

Observation 2-6: REFSENS and dRib can be seen as a package. From specification simplicity and UE performance point of view dRib should be avoided. If dRib is not defined, then there is less pressure to tighten the baseline REFSENS. If dRib is allowed, then big improvement compared to LTE/NR REFSENS must happen.

Observation 2-7: Defining MSD is complicated and time consuming. It has also a price tag which is not small as a lot of effort is put into defining MSD number which needs also to be tested. This MSD number is based usually on an allocation that produces worst-case MSD. These allocations are for example 1RB+1RB which are not used in real deployments. For all other allocations UEs perform better than defined MSD, note that also for allocation that is exactly defined for MSD test case UEs will typically perform much better than the requirement. We do not see any value on having this number in the specifications.

Observation 2-8: Power imbalance for CA from non-collocated requirements for some overlapping bands combinations should be defined from the beginning so that confusing signalling extensions can be avoided.

Observation 3: We know for a fact that number of NR SA and NSA combinations are counted in tens of thousands at the time of writing. If RAN4 would need to combine all those with new 6G bands that would multiply number of combinations into hundreds of thousands. Not even the ongoing effort for database solution would save RAN4 from stagnation. In our opinion no NSA RAN4 requirements are needed for 6G. Current NR requirements including EN-DC apply for NR in case of NSA with 6G and 6G SA requirements would be specified but no NR+6G requirements such as MSD, BCS, dTib, dRib etc are defined.

# 4	References

[1]	RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103