3GPP TSG-RAN WG4 Meeting #111				                              R4-2408832

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	10.9.2.3

Source: 	OPPO

Title: 	on Intra-band non-contiguous CA RX requirements

Document for:	Approval

- Introduction

In RAN4#110bis meeting, the intra-band non-contiguous CA has been discussed. In this paper, the RX requirement for intra-band non-contiguous CA has been discussed.

- Discussion

Based on current TS 38.101-1 intra-band non-contiguous CA requirement compared to the intra-band contiguous CA requirement, below table is summarized for each requirement.

Table 1 RX requirement.

| Requirement | Spec structure | Requirement for intra-band non-contiguous CA |
| --- | --- | --- |
| Reference sensitivity | New subclause added | Based on the band combination |
| Maximum input level | New subclause added | For this uplink configuration, The UE shall meet the requirements for each sub-block as specified in Table 7.4-1 and Table 7.4A.1-1 for contiguous part for one component carrier and two component carriers per sub-block, respectively. |
| ACS | New subclause added | For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.5 and 7.5A.1 for one component carrier and two component carriers per sub-block, respectively. |
| Blocking | New subclause added | In-band blocking: For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clause 7.6.2 and 7.6A.2.1 for one component carrier and two component carriers per sub-block, respectively. Out-of-band blocking: the UE shall meet the requirements for each sub-block as specified in clauses 7.6.3 and 7.6A.3.1 for one component carrier and two component carriers per sub-block, respectively. Narrow-band blocking For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.6.4 and 7.6A.4.1 for one component carrier and two component carriers per sub-block, respectively.. |
| Spurious response | New subclause added | For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clauses 7.7 and 7.7A.1 for one component carrier and two component carriers per sub-block, respectively. |
| Intermodulation characteristics | New subclause added | For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in clause 7.8.2 and 7.8A.2.1 for one component carrier and two component carriers per sub-block, respectively. |

It can be observed that for all the RX requirements, the requirement for general and intra-band contiguous CA apply for one component carrier and two component carriers per sub-block, respectively.

Observation 1: For intra-band non-contiguous CA, for all the RX requirements, the requirement for general and intra-band contiguous CA apply for one component carrier and two component carriers per sub-block, respectively.

In such case, for intra-band non-contiguous SL CA RX requirements, similar approach apply.

Proposal 1: For intra-band non-contiguous SL CA, for all the RX requirements, the requirement for general and intra-band contiguous CA apply for one component carrier and two component carriers per sub-block, respectively.

3	Conclusions

In this contribution, we give further analysis on the intra-band non-contiguous sidelink CA RX requirements. The observations and proposals are as below.

Observation 1: For intra-band non-contiguous CA, for all the RX requirements, the requirement for general and intra-band contiguous CA apply for one component carrier and two component carriers per sub-block, respectively.

Proposal 1: For intra-band non-contiguous SL CA, for all the RX requirements, the requirement for general and intra-band contiguous CA apply for one component carrier and two component carriers per sub-block, respectively.

4 References

[1] R4-2406611 	WF on NR_SL_ intraB_CA_ITS, OPPO, Meta