3GPP TSG-RAN WG4 Meeting #111				                              R4-2408834

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	10.9.2.1

Source: 	OPPO

Title: 	on system parameter for intra-band non-contiguous and contiguous CA

Document for:	Approval

- Introduction

In RAN4#110bis meeting, the intra-band non-contiguous CA has been discussed. In this paper, the system parameter has been discussed.

- Discussion

For the operating bands, the intra-band non-contiguous CA band table is in the same sub-clause of intra-band contiguous CA for NR. Similar approach can be reused for intra-band non-contiguous SL CA.

Proposal 1: To capture the intra-band contiguous and non-contiguous SL CA bands under the same sub-clause.

For the channel bandwidth, during the last RAN4 meeting, the channel bandwidth for intra-band non-contiguous CA has been agreed. For the intra-band contiguous CA PC2, there is no official agreement. From our understanding, currently the channel bandwidth for PC3 intra-band contiguous CA can be reused for PC2.

Proposal 2: For PC2 intra-band contiguous CA, reuse the same channel bandwidth of PC3 intra-band contiguous CA.

For channel arrangement, currently the requirement in subclause 5.4A has not differentiate contiguous and non-contiguous CA. In such case, similar approach can apply to sidelink CA and no update for sidelink CA subclause is needed.

Proposal 3: No update for sidelink CA subclause is needed for intra-band non-contiguous CA.

3	Conclusions

In this contribution, we give further analysis on the intra-band non-contiguous sidelink CA RX requirements. The observations and proposals are as below.

Proposal 1: To capture the intra-band contiguous and non-contiguous SL CA bands under the same sub-clause.

Proposal 2: For PC2 intra-band contiguous CA, reuse the same channel bandwidth of PC3 intra-band contiguous CA.

Proposal 3: No update for sidelink CA subclause is needed for intra-band non-contiguous CA.

4 References

[1] R4-2406611 	WF on NR_SL_ intraB_CA_ITS, OPPO, Meta