3GPP TSG-RAN WG4#111	R4-2408856

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	6.18.2

Source: 	Qualcomm France

Title: 	CA_n41A-n66A PC2 MSD

Document for:	Approval

# Introduction

Cross-band MSD for PC2 CA_n41A-n66A is discussed in this contribution.

- Discussion

There is no cross-band MSD specified for n41 in PC3, however there is still some interference level looming in, which should be carefully evaluated when going to PC2.

The following test point was proposed for PC2 n41-n66 [1]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n41 | 1760 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 0.2 | >ACLR2 |

We did analysis for this, but except for using maximum supported BW for n66 UL with single CC UL RB allocation:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n41 | 1760 | 45 | 15 | 240 (RBstart=2) | 2501 | 10 | TBD | >ACLR2 |

We got the following results for PC3, PC2 1TX, and PC2 2TX cases:

One can see that RX IP2 is dominating for 1TX case; we note that IP2 interference increase 2-fods with power increase, i.e. RX IP2 interferer is 6dB higher in 1TX PC2 than in 1TX PC3. We propose to specify MSD requirements only for PC2. The PC3 MSD is very small and would be zero if any margin in REFSENS were assumed.

- Conclusion

MSD for PC2 CA_n41A-n66A was provided with the following results, which should be accounted in defining requirements for this combination:

PC2 1TX:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n41 | 1760 | 45 | 15 | 240 (RBstart=2) | 2501 | 10 | 1.2 | >ACLR2 |

PC2 2TX:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n41 | 1760 | 45 | 15 | 240 (RBstart=2) | 2501 | 10 | 1.4 | >ACLR2 |

- Reference

[1] R4-2404383, “TP for TR38.850: FDD PC2 for n66 with DL CA_n41A-n66A”, T-Mobile USA