3GPP TSG-RAN WG4 Meeting #111	R4-2408973

# Fukuoka, Japan, 20 - 24 May, 2024

Title: 	Discussion on SAN demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	7.16.8.1

Document for:	Discussion

Background

During RAN4#110b meeting, WF [1] on NR_NTN_enh_SAN_UE_demod was approved. In this contribution, we share our views on SAN demodulation requirements for NR NTN enhancements.

Discussion

MCS

- Agreement - MCS 2 and [MCS 12]

Based on our simulation results in our accompany paper, under the MCS 12, the SNR is about 10dB considering the impairment margin. We think the such SNR is achievable comparing to the link budget study in TR 38.821 as Table 2.1-1 shows.

Table 2.1-1 Calibration results on UL transmissions

|  | UL Coupling Loss |  |  | UL Geometry SIR |  |  | UL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| SC1 | 109.2 | 113.5 | 117.8 | -6.9 | -1.3 | 4.4 | -7.0 | -1.5 | 1.0 |
| SC2 | 109.2 | 113.5 | 117.8 | 3.6 | 8.3 | 12.9 | 3.1 | 7.6 | 12.2 |
| SC3 | 109.2 | 113.5 | 117.8 | 4.5 | 9.3 | 14.1 | 3.5 | 8.1 | 12.7 |
| SC4 | 137.9 | 140.2 | 142.5 | -4.6 | -1.0 | 3.3 | -9.8 | -7.3 | -5.0 |
| SC5 | 138.0 | 140.3 | 142.5 | 8.0 | 8.9 | 9.2 | 1.5 | 3.3 | 4.9 |
| SC6 | 96.1 | 97.4 | 98.8 | -3.9 | -1.1 | 2.6 | -3.9 | -1.1 | 2.6 |
| SC7 | 96.1 | 97.4 | 98.8 | 6.1 | 8.3 | 10.4 | 6.1 | 8.3 | 10.4 |
| SC8 | 96.1 | 97.4 | 98.8 | 6.9 | 9.3 | 11.6 | 6.9 | 9.3 | 11.5 |
| SC9 | 123.7 | 125.3 | 127.1 | -4.0 | -0.8 | 3.2 | -4.1 | -1.1 | 2.7 |
| SC10 | 123.7 | 125.4 | 127.1 | 6.9 | 9.0 | 11.1 | 5.2 | 7.1 | 9.0 |
| SC11 | 102.2 | 103.4 | 104.8 | -4.0 | -1.1 | 2.6 | -4.0 | -1.2 | 2.5 |
| SC12 | 102.2 | 103.4 | 104.8 | 5.9 | 8.2 | 10.4 | 5.9 | 8.2 | 10.3 |
| SC13 | 102.2 | 103.4 | 104.8 | 6.7 | 9.2 | 11.6 | 6.7 | 9.2 | 11.5 |
| SC14 | 129.7 | 131.4 | 133.1 | -4.0 | -0.8 | 3.2 | -4.5 | -1.7 | 1.5 |
| SC15 | 129.7 | 131.4 | 133.1 | 6.9 | 9.0 | 11.1 | 2.3 | 4.1 | 5.8 |
| SC16 | 117.2 | 121.5 | 125.8 | -6.7 | -1.2 | 4.5 | -7.6 | -2.6 | 2.5 |
| SC17 | 117.2 | 121.5 | 125.8 | 3.7 | 8.3 | 12.9 | 0.9 | 5.3 | 9.6 |
| SC18 | 117.2 | 121.5 | 125.8 | 4.5 | 9.3 | 14.1 | 0.3 | 4.7 | 9.0 |
| SC19 | 143.4 | 145.7 | 148.1 | -4.6 | -0.9 | 3.5 | -13.9 | -11.6 | -9.3 |
| SC20 | 143.4 | 145.7 | 148.1 | 6.3 | 9.1 | 11.8 | -13.6 | -11.2 | -8.9 |
| SC21 | 100.1 | 105.4 | 111.0 | -8.3 | -1.6 | 4.9 | -8.3 | -1.6 | 4.9 |
| SC22 | 100.1 | 105.4 | 111.0 | 2.2 | 8.0 | 13.7 | 2.1 | 7.9 | 13.7 |
| SC23 | 100.1 | 105.4 | 111.0 | 3.0 | 9.0 | 14.9 | 3.0 | 8.9 | 14.8 |
| SC24 | 129.8 | 131.5 | 133.4 | -3.9 | -0.7 | 3.5 | -4.5 | -1.6 | 1.7 |
| SC25 | 129.8 | 131.6 | 133.4 | 7.1 | 9.3 | 11.6 | 2.1 | 4.0 | 5.9 |
| SC26 | 106.0 | 111.4 | 117.1 | -8.5 | -1.6 | 5.0 | -8.5 | -1.7 | 4.9 |
| SC27 | 106.0 | 111.4 | 117.1 | 2.0 | 8.0 | 13.9 | 1.9 | 7.9 | 13.8 |
| SC28 | 106.0 | 111.4 | 117.1 | 2.9 | 9.0 | 15.1 | 2.7 | 8.7 | 14.8 |
| SC29 | 135.8 | 137.6 | 139.5 | -4.0 | -0.7 | 3.5 | -6.0 | -3.6 | -1.2 |
| SC30 | 135.8 | 137.6 | 139.5 | 7.2 | 9.4 | 11.7 | -2.8 | -0.9 | 0.9 |
| NOTE: Geometry SINR = -10log10(I/C + N/C), where C, I and N equals the carrier, interferer and noise power levels measured over the configured signal bandwidth. |  |  |  |  |  |  |  |  |  |

Confirm to use MCS 2 and MCS 12 for SAN demodulation requirements for normal PUSCH with CP-OFDM for above 10 GHz bands.

Proposals

In this contribution, we discuss on SAN demodulation requirements for NR NTN enhancements. Our observations and proposals are:

- Confirm to use MCS 2 and MCS 12 for SAN demodulation requirements for normal PUSCH with CP-OFDM for above 10 GHz bands.

Reference

R4-2406024, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110b, Huawei, HiSilicon