3GPP TSG-RAN WG4 Meeting # 111	R4-2409014

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussions on UE performance requirements for Rel-18 eRedCap

Source: 	Huawei, HiSilicon

Agenda item:	6.21.3.1

Document for:	Discussion

Background

In last meeting, WF on [110][125] NR_redcap_enh_demod was agreed, in this contribution we provide our views on open issues.

- Discussions

In last meeting RAN4 agreed to use following CSI-RS and PDSCH PRB size configuration:

CSI-RS configuration for CSI reporting requirements - Configure BWP/PDSCH/CSI-RS PRB size as follows: CQI reporting tests (static/fading); PMI reporting test. FDD CBW/SCS=10MHz/15kHz; BWP: 52 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH:15 RBs (PRB 0 to 14)); BWP: 52 RBs CSI-RS: 28RBs (PRB 0 to 27) PDSCH: 25RBs (PRB 0 to 24). TDD CBW/SCS=20MHz/30kHz; BWP: 51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH: 7RBs (PRB 0 to 6); BWP: 51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH: 15RBs (PRB 0 to 14). Note: BWP size is same as the transmission bandwidth in TS38.101-1 5.3.2..

All existing CSI test have almost same PRB size for PDSCH and CSI-RS, but for eRedCap CSI test, due to the peak date rate limitation, PDSCH PRB size should be smaller than CSI-RS, which may cause potential performance degradation, especially for TDD CQI test. In order to check the performance degradation, we run the simulations for CQI and PMI tests and the results have been captured in clauses 3.1 to 3.3.

CQI Static conditions

RAN4 agreed to evaluate CQI performance under static conditions to check whether to apply the same test points as Rel-17 RedCap CQI reporting requirements in static condition.

CQI reporting test in static condition - Interested companies are encouraged to provide the simulation results whether to apply the same test points as Rel-17 RedCap CQI reporting requirements in static condition. - SNR=5/6dB and SNR=11/12dB for FDD/HD-FDD/TDD with 1Rx. - SNR=8/9dB and SNR=14/15dB for FDD/HD-FDD/TDD with 2Rx.

The simulation results are summarized in Table 3-1:

Table 3-1: Simulation results for CQI under AWGN conditions

| SNR (dB) | 1Rx UE |  |  |  |  | 2Rx UE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Median CQI | Rate of reported CQI in range of median CQI +/- 1 | BLER at median CQI | BLER at Median CQI - 1 | BLER at Median CQI + 1 | Median CQI | Rate of reported CQI in range of median CQI +/- 1 | BLER at median CQI | BLER at Median CQI - 1 | BLER at Median CQI + 1 |
| 4 | 6 | 100% | 0 | 0 | 0.99 | 6 | 100% | 0 | 0 | 1 |
| 6 | 7 | 100% | 0.01 | 0 | 0.91 | 7 | 100% | 0.01 | 0 | 1 |
| 8 | 8 | 100% | 0 | 0 | 0.91 | 8 | 100% | 0 | 0 | 1 |
| 10 | 9 | 100% | 0 | 0 | 0.74 | 9 | 100% | 0 | 0 | 0.93 |
| 12 | 10 | 100% | 0 | 0 | 0.71 | 10 | 100% | 0 | 0 | 0.72 |
| 14 | 11 | 100% | 0.01 | 0 | 1 | 11 | 100% | 0 | 0 | 0.78 |
| 16 | 12 | 100% | 0 | 0 | 0.91 | 12 | 100% | 0 | 0 | 0.96 |

CQI Fading conditions

RAN4 agreed to evaluate CQI performance under fading conditions to check whether to apply the same test points as Rel-17 RedCap CQI reporting requirements in static condition.

CQI reporting test in fading condition - Interested companies are encouraged to provide the simulation results whether to apply the same test points and requirements as Rel-17 RedCap CQI reporting requirements in fading condition. - SNR=9/10dB and SNR=15/16dB for FDD/HD-FDD/TDD with 1Rx. - SNR=6/7dB for FDD/HD-FDD/TDD with 2Rx. - Requirements: - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%. - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05 - Average BLER with follow CQI shall be greater than or equal to 0.02.

The simulation results are captured in Table 3-2.

Table 3-2: Simulation results for CQI under fading channel

| SNR(dB) | FDD |  |  |  |  |  |  |  | TDD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1Rx |  |  |  | 2RX |  |  |  | 1Rx |  |  |  | 2RX |  |  |  |
|  | Median CQI | BLER | Alpha | Gama | Median CQI | BLER | Alpha | Gama | Median CQI | BLER | Alpha | Gama | Median CQI | BLER | Alpha | Gama |
| 4 | 6 | 0.24 | 0.39 | 1.24 | 8 | 0.31 | 0.43 | 0.94 | 6 | 0.30 | 0.46 | 1.02 | 8 | 0.35 | 0.45 | 0.86 |
| 5 | 7 | 0.25 | 0.39 | 1.21 | 8 | 0.26 | 0.38 | 1.10 | 7 | 0.30 | 0.47 | 0.98 | 9 | 0.42 | 0.47 | 0.87 |
| 6 | 8 | 0.23 | 0.41 | 1.17 | 9 | 0.28 | 0.42 | 1 | 8 | 0.30 | 0.45 | 0.94 | 9 | 0.35 | 0.46 | 0.85 |
| 7 | 8 | 0.24 | 0.39 | 1.14 | 9 | 0.23 | 0.37 | 1.13 | 8 | 0.31 | 0.45 | 0.92 | 9 | 0.28 | 0.40 | 0.94 |
| 8 | 9 | 0.22 | 0.42 | 1.12 | 10 | 0.23 | 0.42 | 1.15 | 9 | 0.30 | 0.47 | 0.91 | 10 | 0.33 | 0.48 | 0.95 |
| 9 | 9 | 0.23 | 0.40 | 1.13 | 11 | 0.22 | 0.45 | 1.14 | 9 | 0.30 | 0.44 | 0.92 | 11 | 0.40 | 0.51 | 1.00 |
| 10 | 10 | 0.20 | 0.43 | 1.20 | 11 | 0.21 | 0.44 | 1.14 | 9 | 0.29 | 0.35 | 0.94 | 11 | 0.32 | 0.48 | 1.00 |
| 11 | 10 | 0.22 | 0.40 | 1.19 | 12 | 0.19 | 0.51 | 1.14 | 10 | 0.28 | 0.47 | 0.99 | 12 | 0.42 | 0.58 | 1.08 |
| 12 | 11 | 0.20 | 0.43 | 1.15 | 12 | 0.19 | 0.47 | 1.14 | 11 | 0.25 | 0.48 | 0.99 | 12 | 0.35 | 0.60 | 1.06 |
| 13 | 11 | 0.20 | 0.40 | 1.15 | 12 | 0.18 | 0.46 | 1.13 | 11 | 0.23 | 0.48 | 1.04 | 12 | 0.28 | 0.53 | 1.07 |
| 14 | 12 | 0.19 | 0.47 | 1.14 | 13 | 0.17 | 0.56 | 1.13 | 12 | 0.22 | 0.58 | 1.06 | 13 | 0.32 | 0.66 | 1.06 |
| 15 | 12 | 0.19 | 0.45 | 1.11 | 13 | 0.17 | 0.58 | 1.10 | 12 | 0.20 | 0.56 | 1.07 | 13 | 0.27 | 0.70 | 1.06 |
| 16 | 13 | 0.19 | 0.56 | 1.10 | 14 | 0.15 | 0.67 | 1.13 | 13 | 0.20 | 0.63 | 1.05 | 13 | 0.23 | 0.66 | 1.07 |
| 17 | 13 | 0.17 | 0.56 | 1.08 | 14 | 0.15 | 0.72 | 1.09 | 13 | 0.16 | 0.66 | 1.05 | 14 | 0.26 | 0.71 | 1.04 |

We observe that due to the large span of PDSCH and CSI-RS frequency allocation, for most SNR points of TDD, the throughput gain of follow CQI is quite limited and for some SNR points, the negative gain can be observed. Therefore, we don’t think it’s feasible to define CQI requirements under fading channel.

Observation 1: For most SNR points of TDD, the throughput gain of follow CQI is quite limited and for some SNR points, the negative gain can be observed.

PMI Fading conditions

RAN4 agreed to evaluate PMI performance to check whether to apply the same test points as Rel-17 RedCap.

PMI reporting tests - Interested companies are encouraged to provide the simulation results whether to apply the same test points and requirements as Rel-17 RedCap PMI reporting requirements for FDD/HD-FDD/TDD with 1Rx/2Rx. - Set the test metric as γ=tuetrnd, where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding based on type I Single Panel codebook. - γ=1.3.

The simulation results are shown in Figure 4-1 and Table 4-1:

Figure 4-1: PMI simulation results

Table 3-3: Summary of simulation results

| Gama | FDD | TDD |
| --- | --- | --- |
| 1Rx | 2.02 | 1.80 |
| 2Rx | 2.25 | 1.97 |

Based on the simulation results, we think it is feasible to reuse Rel-17 RedCap PMI test setup and test metric.

We propose the following:

Proposal 1: RAN4 to define CQI requirements under AWGN conditions and PMI requirements by reusing the Rel-17 RedCap test setup and test metric.

Proposal 2: RAN4 shall not define CQI requirements under fading conditions.

- Conclusion

In this contribution we provide our simulation results for eRedCap. The proposals and observations are:

Observation 1: For most SNR points of TDD, the throughput gain of follow CQI is quite limited and for some SNR points, the negative gain can be observed.

Proposal 1: RAN4 to define CQI requirements under AWGN conditions and PMI requirements by reusing the Rel-17 RedCap test setup and test metric.

Proposal 2: RAN4 shall not define CQI requirements under fading conditions.

- Reference

[1]    RP-2402870 Way forward on [110][325] NR_redcap_enh_demod. Ericsson