3GPP TSG-RAN WG4 Meeting # 111	R4-2409047

# Fukuoka, Japan,  20th May - 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | draft-CR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5: Chapter 10.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-5-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To apply the test metrics change for EISrefsens. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To include figures for EISrefsens test metrics; To update the text to describe the EISrefsens test metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements and test configuration for EISrefsens mismatches. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

10.3	OTA reference sensitivity level

10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel.

10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as [specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 10.3.2-1 and Table 10.3.2-2]. The requirement is verified with the test metric of EIS (Link=RX spherical coverage grid, Meas=Link Angle).

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) |  | UL/DL RB allocation |  | OTA reference sensitivity level, EISREFSENS (dBm) |  |
| --- | --- | --- | --- | --- | --- |
| 50, 100, 200, 400 |  | Full RB allocation NRB as specified in sub-clause 5.3.2 |  | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |  |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

Table 10.3.2-2: EISREFSENS_50MHz value per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | -122 |
|  |  | 3 | -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | -122 |

VSAT spherical coverage grid of test metric for peak reference sensitivity verification should cover the spherical area within the supported elevation angles by VSAT declaration. The peak reference sensitivity verification can be simplified to the Rx beam peak direction only for Type 1 or 4 VSAT if the supported lowest elevation angle(s) of such VSAT can be achieved by mechnicaly rotating its aperture and beam peak to this lowest elevation angle(s).

The supported lowest elevation angle declared by VSAT should be lower than at most [60] degree. And this supported lowest elevation angle can be declared by VSAT considering its capability of electronic and/or mechanical beam or aperture steering.

The illustration of the supported lowest elevation angle and the spherical coverage grid are shown in the figure below.

Figure 10.3.2-1: Spherical coverage grid and lowest supported elevation angle

<End of Change>