3GPP TSG-RAN WG4 Meeting #111                                        R4-2409175

Fukuoka, Japan, May 20 - 24, 2024

Source: 	Huawei, HiSilicon

Title: 	On SRS IL imbalance issue

Agenda Item:	10.1.1.4.4

Document for:	Approval

Introduction

During RAN4#110bis, SRS IL imbalance issue has been discussed. Following conclusions are reproduced from [1].

- Topic 4: SRS IL imbalance issue - Sub-topic 4-1: General considerations for SRS IL imbalance issue Way forward: Further discuss the topic of SRS IL imbalance issues with the following proposals used as a starting point. Whether a solution is agreeable, depends on further discussion of the following candidate solutions and/or identified issues. - Proposal 1: Study the issues of insertion loss imbalance across SRS ports. - Proposal 2: Further discuss SRS IL for 6RX type of devices taking into consideration: - SRS insertion loss requirements for 6RX UEs - SRS IL impact on performance - SRS insertion loss compensation - UE assistance on SRS insertion loss (power imbalance) - Others - Proposal 3: Companies should, independent of their view, provide constructive technical inputs on SRS TX power imbalance topic. - Sub-topic 4-2: Candidate solutions for the SRS IL imbalance issue Way forward: Further discuss the following options considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation. - Option 1: Specify UE behaviour and requirements for scenarios, when UE has sufficient power to compensate the power imbalance (Case 2) and require UE to perform SRS IL compensation up to the maximum power capabilities. - Case 1 (non near max Tx power): In this scenario the SRS transmission power (PSRS) is below PCMAX_L,f,c. Based on TS 38.213 the UE is required to compensate any insertion loss and no further changes in the specification are needed. Per discussion during RAN4 meetings there seem to be no clear understanding whether all existing UEs do perform the SRS IL compensation under these conditions. So, it is recommended to consider specific requirements / conformance requirements in future to guarantee proper UE implementations. - Case 2 (near max Tx power): In this scenario the actual required SRS transmission power is higher than PCMAX_L,f,c, but is still below the max transmission power PCMAX_H,f,c. Based on the interpretation of TS 38.101-1 above, in this case the UE behaviour is undefined and UE may or may not perform SRS IL compensation. - Case 3 (max Tx power): In this scenario the actual required SRS transmission power is equal to PCMAX_H,f,c. (i.e. hypothetical scenario). In this case UE is not capable to perform any SRS IL compensation. - Option 2: At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively. - Option 3: Not specifying reporting of SRS IL offsets due to IL imbalance. - Option 4: IL imbalance reporting mechanism for SRS AS including both static reporting and dynamic reporting. - Static reporting is up to UE implementation, and UE needs to indicate the power compensation behaviour to NW if UE reports statically. - Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. - Option 5: IL imbalance reporting mechanism for SRS AS including both the configured maximum output power per SRS resource and the power headroom per SRS resource. - The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission. - Other options are not precluded. Consider some clarifications according to the RAN1 and RAN4 specs. - RAN1 specification on power control doesn’t require or imply that UE shall apply the same PCMAX for all AS-SRS transmission occasions - Under the minimum requirement ∆TRxSRS defined in RAN4 specification, it is up to UE implementation whether to apply exact value for corresponding AS-SRS transmission occasions.

In this contribution, we would like to share our views regarding this topic.

Discussion

During last meeting, SRS IL issue has been discussed extensively but with limited convergence. Given that this issue itself has been justified, we tend to provide our preliminary thinking with further clarifications.

- Specification examination and target scenario identification

For a start, below figure can be used for better illustration on the SRS IL issue.

Figure 1. Free space channel targeted PMI estimation error due to imbalanced power delivery at UE antenna connector

As clearly defined in TS 38.213, the actual power delivered per SRS occasion is the minimum value between PCMAX and another calculated one, and it should be clarified that the PCMAX utilized here is a configured value according to the definition in TS 38.101-1 instead of the range [PCMAX, L, PCMAX, H].

If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasion i as [dBm] where, - PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i - PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId --- Omit the rest part for simplicity ---

Proposal 1: Clarify that for PSRS calculation as defined in TS 38.213, PCMAX is a value determined by UE according to the definition in TS 38.101-1, where ∆TRxSRS has already been counted respective to each SRS occasion, but not the range [PCMAX, L, PCMAX, H] itself.

Look closely at the PSRS definition, the network can do power control based on SRS resource set level configuration along with SRS power control adjustment state for each SRS occasion:

- Target power PO_SRS,b,f,c

- Scaling factor αSRS,b,f,c for the estimated path loss of each SRS resource set

- Power up or down controlled by hb,f,c for each SRS occasion

Moreover, PSRS equals to PCMAX as the UE determined for each SRS occasion if it is the smaller one. Thus we think SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

In our understanding, at least Case #3 is the target scenario to consider SRS IL reporting because it is not the network duty to consider compensation for the path loss.

Observation 1: SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

Proposal 2: The following scenario can be considered as the target for SRS IL reporting:

- For all SRS resources within the set, all of PSRS equals to PCMAX

  - In this scenario, the estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge

- Whether or not to consider effect of loss imbalance across Rx paths

During Rel-18 discussion, concern regarding Rx IL imbalance was raised. Unlike antenna switching SRS transmission, DL reception would not require Rx switching. Consequently, IL link budget for each Rx path should be within the same level given that it can be realized by similar PCB trace pattern and RF component selection.

Observation 2: Unlike antenna switching SRS transmission, DL reception would not require Rx switching. Consequently, IL of each Rx path should be within the same level given that it can be realized by similar PCB trace pattern and RF component selection.

For better illustration, we can take one proposed architecture in [4] e.g., Figure 1 for analysis. It can be calculated that the IL imbalance among Rx branches is about 0.3~0.5dB @3.5GHz which is negligible. Unfortunately, there is no example has been provided for us to better understand if there could be any exceptional but reasonable implementation. Besides, we believe it is rational that per branch REFSENS has never been studied by RAN4.

Observation 3: RAN4 has never discussed about per branch REFSENS since no exceptional but reasonable RF implementation can be provided to prove the necessity of it.

Proposal 3: Tx/Rx-Rx imbalance should not be considered for the discussion on SRS IL reporting.

- Other aspects if SRS IL reporting can be considered

- Reporting granularity

In our understanding, the reporting granularity should be SRS resource, because RAN1 spec defines a UE behaviour as below.

- 7.3 Sounding reference signals For SRS, - if a UE is provided tdm for an SRS resource with 8 ports in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports on each symbol for SRS transmission. - else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS.

In this sense, the transmitted power should be divided equally between SRS ports for one SRS resource.

Proposal 4: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient.

- Threshold for reporting frequency control

Similar as the threshold defined for legacy PHR reporting, if dynamic reporting can be considered for SRS IL reporting, then a network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency.

Proposal 5: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency.

Conclusion

In this contribution we discussed on the SRS IL reporting. We have the following observations and proposals:

Observation 1: SRS transmission power determination can be categorised as following cases.

| Case #1 | For all SRS resources within the set, none of PSRS equals to PCMAX | The estimated path loss could be relatively small, e.g. cell centre |
| --- | --- | --- |
| Case #2 | For all SRS resources within the set, some of PSRS equals to PCMAX | The estimated path loss could still be not so high while some of the diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre |
| Case #3 | For all SRS resources within the set, all of PSRS equals to PCMAX | The estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge |

Observation 2: Unlike antenna switching SRS transmission, DL reception would not require Rx switching. Consequently, IL of each Rx path should be within the same level given that it can be realized by similar PCB trace pattern and RF component selection.

Observation 3: RAN4 has never discussed about per branch REFSENS since no exceptional but reasonable RF implementation can be provided to prove the necessity of it.

Proposal 1: Clarify that for PSRS calculation as defined in TS 38.213, PCMAX is a value determined by UE according to the definition in TS 38.101-1, where ∆TRxSRS has already been counted respective to each SRS occasion, but not the range [PCMAX, L, PCMAX, H] itself.

Proposal 2: The following scenario can be considered as the target for SRS IL reporting:

- For all SRS resources within the set, all of PSRS equals to PCMAX

  - In this scenario, the estimated path loss could be high and all diversity branches becomes MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge

Proposal 3: Tx/Rx-Rx imbalance should not be considered for the discussion on SRS IL reporting.

Proposal 4: Given that specification defines the Tx power should be equally distributed across SRS ports for each SRS transmission, per SRS resource IL reporting would be sufficient.

Proposal 5: If dynamic reporting can be considered for SRS IL reporting, network configurable threshold related to e.g. historical change of PSRS can be considered in order to give the network authority for handling SRS IL reporting frequency.

- References

- R4-2406585, “WF on requirements for 6Rx”, AT&T, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2406718, “LS on 3T6R and 4T6R antenna switching SRS”, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-240xxxx, “On 6Rx reference sensitivity requirements for FR1 UE”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-25, 2024.

- R4-2216437, “R18 Discussion on 8Rx FWA”, OPPO, 3GPP TSG-RAN WG4 Meeting #104bis-e, Electronic Meeting, October 10 – 19, 2022.