3GPP TSG-RAN WG4 Meeting #111                                R4-2409305

Fukuoka, Japan, 20-24 May, 2024

Source:	KDDI, LGE, Murata, Samsung

Title:	TP for TR 38.898: DC_1-42_n77

Agenda item:			6.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_1-42_n77 according to the request in [1].

The corresponding 3-band PC3 combo has already been specified.

The 2-band PC2 DL_1A_n77A with UL_1A_n77A has already been specified in [2]..

There is no pending fallbacks.

2. Reference

[1] RP-240241 Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

[2] 3GPP TS 38.101-3 V18.5.1 (2024-03)

# 3. Text Proposal

<Start of Text Proposal>

5.x 	DC_18-42_n77

5.x.1	Configuration for DC

Table 5.79.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_18A-42A_n77A14,15,16 DC_18A-42C_n77A15,16 | DC_18A_n77A14 |
| NOTE1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

5.x.2	Maximum output power for DC

The maximum output power requirement for PC2 UL DC_18_n77 is defined as Table 6.2B.1.3-1 in TS38.101-3 [2]. So, this section can be omitted.

5.x.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. There is no MSD issue for this configuration because LTE Band 42 and NR Band n77 are synchronized, and that simultaneous transmission of LTE Band 42 and NR Band n77 are not supported.

5.x.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

<End of Text Proposal>