3GPP TSG-RAN WG4 Meeting # 111	R4-2409430

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda Item:	10.4.2.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on FR1 NTN devices OTA

Document for:	Approval

# Introduction

In RAN#103, the WID on TRP, TRS and MIMO OTA testing enhancement Phase 3 was approved in [1] where the following objectives of test method for FR1 NTN devices.

- Study and define test methodology and radiated performance metric for FR1 NTN devices (including NR NTN and IoT NTN) - Study the usage scenarios and develop enhanced test methodology - Study and specify the proper performance metric - Develop preliminary Measurement Uncertainty (MU) (RAN5)

In RAN4#110bis, WF on Rel-19 TRP, TRS and MIMO OTA was approved in [2] in which open issues for FR1 NTN OTA testing were captured.  In this paper, we provide our views on OTA test method and performance metric for FR1 NR NTN and IoT NTN devices.

# 2. Discussion

## 2.1 UE type

For NR-NTN device, in Rel-17 NR NTN discussion, handheld UE was assumed [3]. In [2], another UE type, i.e., VSAT-like device was also mentioned. While it is not clear about the definition of VSAT-like device. For example, VSAT could be either fixed VSAT or mobile VSAT. And usually, VSAT has many types of UE form factor, e.g., larger DUT sizes, different antenna configurations, etc. Therefore, it is difficult to study the OTA test method and performance metric before there is a clear definition of VSAT-like device.

Observation 1: VSAT could be either fixed VSAT or mobile VSAT and has many types of UE form factor. It is difficult to study the OTA test method and performance metric before there is a clear definition of VSAT-like device.

Proposal 1: RAN4 to prioritize handheld UE for FR1 NR NTN OTA discussion.

Proposal 2: It is encouraged interested companies to provide input including definition, DUT size, antenna design, etc., to other UE type if any.

For IoT NTN, the form factors of DUT are various and IoT devices could be used for many scenarios including industry, shipping ports, airports, utility monitoring, etc. Therefore, RAN4 needs to discuss the target form factor and corresponding size, antenna design, etc., considering different IoT NTN use cases.

Proposal 3: RAN4 should discuss the target UE form factors and corresponding size, antenna design, etc., for IoT NTN OTA testing.

## 2.2 UE performance metric

In [2], the issue on whether different satellite orbits should be considered for UE performance metric was raised. NTN UEs can support either NGSO or GSO or both.

For GSO scenario, the relative position between UE and satellite is almost static. Therefore, UE antenna radiated performance is the main target for verification. In addition, omni-radiated performance such as TRP and TRS are not suitable for NTN device OTA testing any more since usually device needs to point to the satellite with the help of software before starting the communication with satellite. Therefore, conical TRP/EIRP for Tx and conical TRS/EIS for Rx can be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f.

Proposal 4: For GSO scenario, conical TRP/EIRP for Tx and conical TRS/EIS for Rx can be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f.

While for NGSO scenario, the relative position between UE and satellite is changing. Hence, both UE antenna radiated performance and capability of tracking satellite beam should be verified in this scenario. To verify the UE’s capability of tracking satellite beam, CDF-like performance metric with hemisphere scan can be considered as the starting point.

Proposal 5: For NGSO scenario, CDF-like performance metric with hemisphere scan can be considered as the starting point.

## 2.3 Other aspects

For NTN UE, antenna design could be based on circular polarization or cross polarization. Therefore, in NTN OTA testing, the test method should not limit UE implementation on pol., and whether the antenna is shared or separate from TN antenna. And from TE PoV, the test setup needs to decide  support different UE implementation on the antenna design.

Proposal 6: In NTN OTA testing, the test method should not limit UE implementation on pol., and whether the antenna is shared or separate from TN antenna. From TE PoV, the test setup should support different UE implementation on the antenna design.

Other factors that affect the NTN UE OTA test method include whether the devices use Transmit Antenna Selection (TAS) function and the number of Tx antennas (1Tx and/or 2Tx). UE vendors are encouraged to provide feedback on the above other aspects that affect NTN radiated performance.

Proposal 7: UE vendors are encouraged to provide feedback on the other aspects including TAS, number of Tx antenna, etc, that affect NTN radiated performance.

# 3.	Conclusion

In this paper, we provide our views on OTA test method for FR1 NTN devices. The following proposals were made:

Observation 1: VSAT could be either fixed VSAT or mobile VSAT and has many types of UE form factor. It is difficult to study the OTA test method and performance metric before there is a clear definition of VSAT-like device.

Proposal 1: RAN4 to prioritize handheld UE for FR1 NR NTN OTA discussion.

Proposal 2: It is encouraged interested companies to provide input including definition, DUT size, antenna design, etc., to other UE type if any.

Proposal 3: RAN4 should discuss the target UE form factors and corresponding size, antenna design, etc., for IoT NTN OTA testing.

Proposal 4: For GSO scenario, conical TRP/EIRP for Tx and conical TRS/EIS for Rx can be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f.

Proposal 5: For NGSO scenario, CDF-like performance metric with hemisphere scan can be considered as the starting point.

Proposal 6: In NTN OTA testing, the test method should not limit UE implementation on pol., and whether the antenna is shared or separate from TN antenna. From TE PoV, the test setup should support different UE implementation on the antenna design.

Proposal 7: UE vendors are encouraged to provide feedback on the other aspects including TAS, number of Tx antenna, etc, that affect NTN radiated performance.

# 4.	References

- RP-240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3

- R4-2406086, Way Forward for [110bis][338] TRP_TRS_MIMO_OTA, vivo, CAICT

- TR 38.863