3GPP TSG-RAN WG4 Meeting#111                              R4-2409617

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.16.5.2

Source: 	ZTE Corporation,Sanechips

Title: 	Further discussion on Rx RF requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In the previous RAN4 meeting, we reached good progress on the VSAT RF requirements, however there are still some remaining issues left for further discussions. In this contribution, we would like to share some further views on these remaining Rx requirements.

Online agreement Issue 1-1: Maximum TRP Online Agreement: - For type 1 and type 4 with mechanical steering antenna (e.g. parabolic antenna) - 35dBm - For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array) - 35dBm, 40dBm or 45dBm for further decision in this meeting. Offline agreement: For maximum TRP: - For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array) - [43dBm] for further ACLR evaluation; if there is coexistence issue identified for 43dBm next meeting, then set 40dBm as maximum TRP power; - The maximum EIRP 76.2 is not changed, the antenna gain for other TRP values could further adjusted; Offline agreement: For the minimum peak EIRP requirement: - To state in RAN4 spec to declare the supported elevation angles to comply the minimum peak EIRP requirement and leave the test for RAN5 discussion. Issue 1-2: Minimum output power WF: Not to define the transmitter Minimum controlled output power requirement; Issue 1-3: Transmit ON-OFF power - Recommended for further discussion: - Option 1: reusing FR2 BS OFF power requirement -36dBm/MHz Online Agreement: - Agree on Option 1. Issue 1-4: Transmit ON-OFF time mask Online Agreement: - Transient period for FR2-NTN is 5us. Issue 1-5: Transmit freq error - Recommended Agreement: - Follow the agreement reached in R4-232197 with fixed doppler shift. - Time varying doppler shift will be discussed in Rel-19 RAN4 NTN WID. Issue 1-6: Transmit modulation quality - Recommended Agreement: - Follow the agreement reached in R4-2317648, R4-2321974 Issue 1-7: SEM Online Agreement: - Agree on proposal 4. - FFS on definition of P_(rated,UE) Offline agreement for NS indication for regional requirements Step 1: to make the regulatory requirement clear and applicable to which bands; Step 2: to further discuss how to arrange the regulatory requirement in the specification next meeting; Issue 1-8: Power control requirement Online Agreement: - No requirement for power control for Ka band for NR-NTN UE Issue 1-9: MPR requirement for type 2 and type 5 to meet OFF-axis EIRP requirement Offline agreement: For type 2 an type 5, the power reduction for OFF-axis EIRP requirement might be needed and the exact value is left for the implementation; Issue 2-1 Minimum EIS requirement Online Agreement: - for type 3 UE, to specify minimum EIS as -115.6dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz; - for type 1/2/4/5 UE, to specify minimum EIS as -122dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz Issue 2-2: Maximum input power Online Agreement: - For type 3, - [-101]dBm as maximum input power with 64QAM - For type 1/2/4/5: - -109.6dBm with 16QAM or [64QAM]. - FFS for QPSK - The exact MCS or coding rate for FRC of maximum input power need further discussion and confirmation. Issue 2-5: Polarization characteristics Online Agreement: - Either RHCP or LHCP is supported by Ka band VSAT in Rel-18 Open for further discussion Issue 1-11: Antenna performance - Proposal 1: Specify Antenna performance requirements for band n511 and n510 based on FCC 47 CFR 25.209. [Ericsson, R4-2404939] - Proposal 2: Antenna point accuracy should not be introduced into the spec until RAN4 fully discuss such requirements in detail. [Huawei, R4-2405339] Recommended WF: - Further discuss the proposal 1 - To clarify anything is not clear for antenna point accuracy. Issue 1-7: SEM Online Agreement: - Agree on proposal 4. - FFS on definition of P_(rated,UE) - FFS for the boundary for SEM and spurious emission requirements. Issue 1-7: SEM Online Agreement: - Agree on proposal 4. - FFS on definition of P_(rated,UE) - FFS for the boundary for SEM and spurious emission requirements. Issue 2-2: Maximum input power - FFS on Whether the maximum input power should be aligned with ACS interfering signal power level; Issue 2-3: ACS - FFS on the test parameters for ACS requirement Issue 2-4: Antenna off-axis performance - Proposal 1: Specify Rx antenna off-axis performance requirements for band n512 based on ETSI EN 301 360, EN 301 459, EN 303 699, EN 303 978. [Ericsson, Thales, R4-2404940] - Recommended WF: Need further discussions online.

Discussion

- 2.1. The maximum input power

During the last RAN4 meeting, we reached the following consensus for maximum input power, however there are still some further issues to be further discussed. In the following section, we want to share some further analysis on the following issues based on assumption SAN Tx power as 62dBm EIRP for LEO at peak direction and 92dBm EIRP for SAN at peak direction, however the applicable MCS or modulation order need to be further checked.

Agreement: For type 3, for the maximum input power requirement, for Ka-band VSAT, only to define the requirement up to [16/64QAM]. - FFS on the coding rate when defining the FRC configuration for [16/64QAM] maximum input power. - FFS on the value for maximum input power level. - Note: for 64QAM modulation order, the lower orbits compared with 600KM might be considered . For type 1/2 capable of connecting with both GSO and LEO, for the maximum input power requirement, for Ka-band VSAT, only to define the requirement up to [16/64QAM] under the LEO assumption as maximum input power - FFS on the coding rate when defining the FRC configuration for [16/64QAM/] maximum input power. - FFS on the value for maximum input power level. For type 4/5 capable of connecting with GSO, for the maximum input power requirement, for Ka-band VSAT, only to define the requirement up to [16/64QAM/] under the GSO assumption as maximum input power - FFS on the coding rate when defining the FRC configuration for [16/64QAM/] maximum input power. - FFS on the value for maximum input power level.

|  | GSO |  | LEO1200 |  | LEO600 (set 1) |  | LEO600 (set 2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| h | 35786.00 | 35786.00 | 1200.00 | 1200.00 | 600.00 | 600.00 | 600.00 | 600.00 |
| B | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 | 6378.00 |
| r | 42164.00 | 42164.00 | 7578.00 | 7578.00 | 6978.00 | 6978.00 | 6978.00 | 6978.00 |
| Elevation_degree | 30.00 | 90.00 | 45.00 | 90.00 | 45.00 | 90.00 | 45.00 | 90.00 |
| Elevation_radian | 0.52 | 1.57 | 0.79 | 1.57 | 0.79 | 1.57 | 0.79 | 1.57 |
| D2 | 38611.64 | 35786.00 | 1579.95 | 1200.00 | 814.83 | 600.00 | 814.83 | 600.00 |
| f_MHz | 17000.00 | 17000.00 | 17000.00 | 17000.00 | 17000.00 | 17000.00 | 17000.00 | 17000.00 |
| Pathloss_FreeSpace | 208.78 | 208.12 | 181.02 | 178.63 | 175.27 | 172.61 | 175.27 | 172.61 |
| Atmosphere_loss | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| NF [dB] | 2.50 | 2.50 | 2.50 | 2.50 | 2.50 | 2.50 | 6.00 | 6.00 |
| BW [MHz] | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 | 50.00 |
| Target SNR for DL 64QAM | 6.00 | 9.50 | 6.80 | 12.00 | 12.50 | 18.00 | 9.00 | 15.00 |
| IM | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| SAN_AntennaGain | 55.50 | 55.50 | 55.50 | 55.50 | 55.50 | 55.50 | 55.50 | 58.50 |
| SAN_TxPower [dBm] | 88.99 | 91.99 | 61.99 | 64.99 | 61.99 | 64.99 | 61.99 | 64.99 |
| MaxInputPower | -121.79 | -118.13 | -121.03 | -115.64 | -115.28 | -109.62 | -115.28 | -109.62 |
| Gain_element | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 |
| Rx_Antenna_size_min | 1067.42 | 1028.78 | 1076.93 | 1031.00 | 1064.22 | 1026.13 | 1064.22 | 1151.33 |

Issue 2-2: Maximum input power Online Agreement: - For type 3, - [-101]dBm as maximum input power with 64QAM - For type 1/2/4/5: - -109.6dBm with 16QAM or [64QAM]. - FFS for QPSK - The exact MCS or coding rate for FRC of maximum input power need further discussion and confirmation. Issue 2-2: Maximum input power - FFS on Whether the maximum input power should be aligned with ACS interfering signal power level; Issue 2-3: ACS - FFS on the test parameters for ACS requirement

In the last RAN4 meeting, there were some discussions whether maximum input power should be aligned with interference signal power of Case 2 ACS requirement similar as the legacy UE. From our understanding, this is not needed since the maximum input power from satellite should be much lower than the potential interference signal from the terrestrial network. To test the maximum input power with non-reasonable power level will mislead the VSAT design. Secondly, interference signal of Case 1 ACS requirement will have two test purposes which is somehow slightly different from legacy UE:

- The adjacent channel selectivity;

- To regulate the receiver dynamic range requirement which is similar as ACS Case 2 requirement and legacy maximum input power;

- Therefore the receiver dynamic range requirement is still tested by ACS requirement and not tested by maximum input power anymore which is something new to VSAT compared with legacy specification.

- Proposal 1:

- For the type 3, to specify the maximum input power as -101dBm for all channel bandwidth and applicable modulation order as 64QAM

- For the type 1,2,4,5, to specify the maximum input power as -109.6Bm for all channel bandwidth and applicable modulation order as 16QAM and [64QAM]

- 2.2. ACS

- In the last RAN4 meeting, we reached the consensus for ACS requirement for VSAT UE. Accordingly, the test parameter for ACS requirement could be defined as following.  In addition, it should be noted that Case 1 has already the maximum potential interference signal, therefore the test parameters for Case 2 is not needed since the maximum input power is much lower than interfering signal power level.

Issue 1-1: NTN VSAT UE ACS Agreement: - The summary of co-existence study results of VSAT UE ACS will be capture in TR 38.863 Chapter 6. The suggested ACS value for UE could be different and captured in Chapter 8 with some notes elaborating reasons. - NTN VSAT UE ACS value is 25dBc.

- Proposal 1: to define the ACS requirement for VSAT as following without Case 2 ACS requirement defined:

Table 7.5-1: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | dB | 25 | 25 | 25 | 25 |

Table 7.5-2: Test parameters for adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | REFSENS + 37.5 dB | REFSENS +37.5 dB | REFSENS +37.5 dB | REFSENS +37.5 dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern as described in Annex A.3.2 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Clause 7.3.2, which are applicable to different UE power classes. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

Conclusions

In this contribution, we want to share some further views on VSAT RF requirements for NTN in Ka-band.and proposals are made as following:

- Proposal 1:

- For the type 3, to specify the maximum input power as -101dBm for all channel bandwidth and applicable modulation order as 64QAM

- For the type 1,2,4,5, to specify the maximum input power as -109.6Bm for all channel bandwidth and applicable modulation order as 16QAM and [64QAM]

- Proposal 2: to define the ACS requirement for VSAT as following without Case 2 ACS requirement defined:

Table 7.5-1: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | dB | 25 | 25 | 25 | 25 |

Table 7.5-2: Test parameters for adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | REFSENS + 37.5 dB | REFSENS +37.5 dB | REFSENS +37.5 dB | REFSENS +37.5 dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern as described in Annex A.3.2 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Clause 7.3.2, which are applicable to different UE power classes. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |  |  |  |

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- https://www.kingsat-tech.com/product/enhanced-vsat-ku-band-3-axis-stabilized-and-4-axis-tracking114.html

- https://www.mitsubishielectric.com/news/2020/0206.html

- R4-2217792, Email discussion summary for [104-bis-e][140] NR_NTN_enh_UERF, ZTE, Noted

- R4-2217743, WF on NTN UE in Ka-band, ZTE, Approved

- http://satstar.net/satellites/space2.html

- https://www.satbeams.com/satellites

- http://satstar.net/beams/inmar5f3_sb11.html

- R4-2216515,NTN enhancements – Regulatory aspects, Ericsson, Noted

- R4-2220573	WF on NTN UE RF requirements, ZTE, approved.

- R4-2220119, Topic summary for [105][139] NR_NTN_enh_UERF, ZTE

- https://www.etsi.org/deliver/etsi_en/301400_301499/301459/02.01.01_60/en_301459v020101p.pdf

- RF Spectrum FCC 47CFR25.138, 47CFR25.202,

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.202

- https://www.digisat.org/enterprise-75cm-professional-ka-band-terminal

- https://www.satsig.net/eut2beac.htm

- R4-2303533,WF on NTN UE RF requirements for Ka band, ZTE, Approved.

- R4-2306624, WF on NTN UE RF requirements in Ka-band. ZTE, approved.

- https://www.eutelsat.pl/files/contributed/satellites/pdf/esog120.pdf

- R4-2403649,WF on NR-NTN UE RF requirements, ZTE, Samsung, Approved.

- R4-2401116, big CR for NTN UE, agreed.

- R4-1703046, Transient time – impact ON OFF time mask, Qualcomm, Noted

- R4-1809791, Network Performance Analysis for Power Control Tolerance, Apple.

- R4-2406134	Way Forward for [110bis][307] NR_NTN_enh_Part3, Samsung, Approved.