3GPP TSG-RAN WG4 Meeting #111											R4-24XXXXX

Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:			4.8

Source:	Moderator (Ericsson)

Title:	Topic summary for [111][301] BSRF_Maintenance

Document for:	Information

# Introduction

The scope of this topic summary is BS RF maintenance agenda items. Topics are divided according to the agenda:

Up to Rel-17 maintenance for LTE and NR:

- BS RF requirements and BS conformance testing	(4.2)

- UE/BS EMC requirements	(4.3)

Rel-18 maintenance for LTE and NR:

- NB-IoT/eMTC core & perf. requirements for NTN: 
SAN RF requirement and conformance testing 	(5.2.2.2)

- Other Rel-18 non-spectrum related WIs: BS RF requirements	(5.2.8.2)

Rel-18 on-going work Items for LTE:

- IoT NTN enhancements: SAN RF requirements maintenance	(8.3.1)

# Topic #1: BS RF requirements and BS conformance testing (4.2)

## Companies’ contributions summary

Discussion papers and LS

| T-doc number | Company | Title/Proposals |
| --- | --- | --- |
| R4-2407121 (Sub-topic 1-1) | Union Inter. Chemins de Fer | Clarification of multiple carrier operation for n100/n101 Proposal 1: Agree that from CEPT Report 74 [3] it is concluded that the term “multiple carrier” applies to the use of more than one wideband carrier (LTE or NR including NB-IoT) in a single frequency band. Proposal 2: Agree that, as the term “multiple carrier” in ECC DEC(20)02 [2] only applies to a single frequency band, the DEC(20)02 [2] does allow transmission of 1 carrier in band n100 and simultaneous transmission of 1 carrier in band n101 at the BS transmit power levels as per the DEC(20)02 [2]. Hence using CA/DC with 1 carrier in band n100 plus 1 carrier in band n101 is allowed at the BS transmit power levels as per the DEC(20)02 [2] without the need for the implementation of a coordination procedure or other mitigation measures. |
| R4-2407216 | Charter, Ericsson, Nokia | (NR_newRAT-Core) TDOC for Total power dynamic range (TPDR) Proposal: To change Table 9.4.3.3-2 for FR2-2 OTA (rounding of dynamic range numbers). |
| R4-2407384 | SoftBank Corp., Deutsche Telekom, NTT DOCOMO, KDDI | (NR_NTN_Solutions) Considerations on HAPS additional operating bands Proposal: Add HAPS operating bands to Rel-17 and Rel-18 TS38.104 clause 5.2 |
| R4-2409466 (Sub-topic 1-1) | Vodafone, Orange, Telecom Italia, Deutsche Telekom | Consideration of multiple carrier operation for n100/n101 Proposal 1: Add multi-carrier clarification to Rel-17 BS specifications TS 38.104 and TS 38.141-1, wishing to allow multiple carriers shall implement coordination procedure or other mitigation measures. |
| R4-2409533 (Sub-topic 1-1) | Huawei, HiSilicon | Follow-up discussion on multiple carrier operation for n100/n101 bands Proposal 1: Update TS 38.104 and TS 38.141-1 with the wording reusing Decides 4 text of ECC(20)02, in order to keep RAN4 specification fully transparent to the ECC regulation. Proposal 2: Send LS to ETSI TC RT (and possibly also ECC technical bodies involved) asking for clarification on the multiple carrier wording. |
| R4-2409538 (Sub-topic 1-1) | Huawei, HiSilicon | Draft LS on multiple carrier operation for n100/n101 bands |

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2407122 (Sub-topic 1-1) | Union Inter. Chemins de Fer | Clarification on multiple carrier operation for n100/n101, R17 Summary of change: Addition of clarification of multiple carrier operation, extending the existing text on coordinated deployments. |
| R4-2407123 (Sub-topic 1-1) | Union Inter. Chemins de Fer | Clarification on multiple carrier operation for n100/n101, R17 Summary of change: Addition of clarification of multiple carrier operation, extending the existing text on coordinated deployments. |
| R4-2407151 | Charter, Ericsson, Nokia | (NR_newRAT-Core) CR for Total power dynamic range (TPDR) Summary of change: Values in Table 9.4.3.3-2 are updated to be consistent with other values for OTA total power dynamic range using the same rounding. |
| R4-2407385 | SoftBank Corp., Deutsche Telekom, NTT DOCOMO, KDDI | (NR_NTN_solutions) CR for TS 38.104: Modification of HAPS operationg bands description Summary of change: Add description of NR operating bands for HAPS. |
| R4-2407420 | Nokia | (AAS_BS_LTE_UTRA-Core) CR to TS 37.105 on correction for single RAT E-UTRA operation Summary of change: Align the OTA spurious emission requirement for single RAT E-UTRA operation with the corresponding requirement is TS 36.104. |
| R4-2407424 | Nokia | (AAS_BS_LTE_UTRA-Core) CR to TS 37.145-2 on correction for single RAT E-UTRA operation Summary of change: Align the OTA spurious emission requirement for single RAT E-UTRA operation with the corresponding requirement is TS 36.141. |
| R4-2407463 | CATT | (NR_IAB-Perf) CR for TS 38.176-2, Correction on notes for scaling factor for IAB-MT type 1-O Summary of change: - Add Note 1 for IAB-MT type 1-O OTA ACLR absolute limit in Table 6.7.3.5.1-2. - Add Note 1 for IAB-MT type 1-O OTA CACLR absolute limit in Table 6.7.3.5.1-5. - Add Note 3(Y = - 9 + 10log10(NTXU,OTApercell) dB) in Table 6.7.5.5.5.1-1. |
| R4-2407502 | CATT | CR for TS 38.174, Correction on specification suffix information for IAB Summary of change: To add the suffix none and A for IAB. |
| R4-2408197 | NEC | CR to 38.108: Minimum requirements for unwanted emissions for SAN Summary of change: For SAN type 1-H, rename the subclause titles for basic limits and add new subclauses for minimum requirements. For SAN type 1-O, add text to specify the minimum requirements. Typo correction in the OOBE basic limits table. |
| R4-2408198 | NEC | CR to 38.108: Minimum requirements for unwanted emissions for SAN (rel-18) Summary of change: For SAN type 1-H, rename the subclause titles for basic limits and add a new subclause for minimum requirements. For SAN type 1-O, add text to specify the minimum requirements and correct a typo in the OOBE basic limits table. For SAN type 2-O, “basic limit” is replaced by “limit” |
| R4-2408203 | NEC | CR to 38.176-1: Correction of test tolerance value for IAB-MT total power dynamic range (rel-16) Summary of change: In table C.1-1, “±1.0 dB for 40MHz < f ≤ 100MHz” was corrected to “±1.0 dB for 40MHz < BW ≤ 100MHz”. |
| R4-2408472 | Ericsson | (TEI17) CR to 37.104 on Removal of Additional spurious emission limits for GSM (BC2) Summary of change: The “Additional minimum requirement for BC2 (Category B)” for spurious emissions is removed. |
| R4-2408474 | Ericsson | (TEI17) CR to 37.141 on Removal of Additional spurious emission limits for GSM (BC2) Summary of change: The “Additional minimum requirement for BC2 (Category B)” for spurious emissions is removed. |
| R4-2409067 | Nokia, Skyworks Solutions, Inc. | (NR_unlic_enh-Core) CR to TS 38.104 NR-U Nominal channel spacing Summary of change: To avoid introduction of [ ] in a frozen TS, the agreed changes of WF R4-2406677 are implemented without brackets. |
| R4-2409070 | Nokia, Qualcomm Inc. | (NR_IAB-Core) CR to TS 38.174: maintenance to IAB requirements Summary of change: Removal of brackets for power control requirements for legacy IAB-MT. |
| R4-2409395 | Huawei, HiSilicon | (NR_NTN_solutions-Core) CR to TS 38.108: Channel raster to resource element mapping Summary of change: Update the text in 5.4.2.2 to make it clear for Channel raster to resource element mapping. |
| R4-2409436 | Keysight Technologies UK Ltd | (NR_ext_to_71GHz-Perf) CR to 37.941: 71 GHz Extension, update on missing Rx MU Summary of change: 10.2.3.4, comment related C1-11 is missing Modification originally in EIS table in 10.2.3 and MU/TT for EIS in 10.2.7 and 10.2.8 were done by other CR (in different format) so that these skipped. Table 10.5.4-3 MU table for FR2 to include FR2-2 values and clarify on frequency range Table 10.6.4-2 MU table for FR2 to include FR2-2 values and clarify on frequency range Table 14.2.4-1 MU table for FR2 in include FR2-2 values and clarify on frequency range |
| R4-2409448 (Sub-topic 1-1) | Vodafone,Orange, Telecom Italia, Deutsche Telekom | CR to TS 38.104: Clarification on multiple carrier operation for n100/n101, Rel 17 Summary of change: Addition of clarification for multiple carrier operation, extending the existing text on the coordinated deployments. |
| R4-2409465 (Sub-topic 1-1) | Vodafone, Orange, Telecom Italia, Deutsche Telekom | CR to TS 38.141-1: Clarification on multiple carrier operation for n100/n101, Rel 17 Summary of change: Addition of clarification for multiple carrier operation, extending the existing text on the coordinated deployments. |
| R4-2409519 | Huawei, HiSilicon | CR to TS 38.104: Clarifications on RMR terminology and related operating bands Summary of change: NR operating bands table updated with a new Note on RMR bands and their applicability. Missing definition and abbreviations added. |
| R4-2409530 | Huawei, HiSilicon | CR to TS 38.104: Alignment with UE specification for n30 and n77 notes in Table 5.2-1 Summary of change: Note 5 and 6 added in Table 5.2-1. |
| R4-2409531 | Huawei, HiSilicon | CR to TS 38.104: Alignment with UE specification for n30 and n77 notes in Table 5.2-1 Summary of change: Note 5 and 6 added in Table 5.2-1 |
| R4-2409534 (Sub-topic 1-1) | Huawei, HiSilicon | CR to TS 38.104: Clarification on multiple carrier operation for n100/n101 Summary of change: Addition of clarification for multiple carrier operation, extending the existing text on the coordinated deployments.. |
| R4-2409536 (Sub-topic 1-1) | Huawei, HiSilicon | CR to TS 38.141-1: Clarification on multiple carrier operation for n100/n101 Summary of change: Addition of clarification for multiple carrier operation, extending the existing text on the coordinated deployments.. |
| R4-2409548 | Huawei, HiSilicon | CR to TS 38.108: Corrections for non-regenerative payload and gateway Summary of change: - Update scope to clarify on non-regenerative payload. - Update definitions, to align terminology with (non-)regenerative payload (instead of bend pipe, or NTN payload RF). - Update SAN definition to be generic enough to contain both non-regenerative and regenerative payload options. - Align usage of the “satellite-gateway” term. - Update SAN figures to align gateway terminology (gateway vs. satellite-gateway) and payload terminology (remove “NTN payload RF” and replace it with the “satellite payload”). |
| R4-2409550 | Huawei, HiSilicon | CR to TS 38.181: Corrections for non-regenerative payload and gateway Summary of change: - Update scope to clarify on non-regenerative payload. - Update definitions, to align terminology with (non-)regenerative payload (instead of bend pipe, or NTN payload RF). - Update SAN definition to be generic enough to contain both non-regenerative and regenerative payload options. - Align usage of the “satellite-gateway” term. - Update SAN figures to align gateway terminology (gateway vs. satellite-gateway) and payload terminology (remove “NTN payload RF” and replace it with the “satellite payload”). |
| R4-2409554 | Huawei, HiSilicon | CR to TS 38.106: removal of environmental annex Summary of change: Removal of annex A. |
| R4-2409557 | Huawei, HiSilicon | CR to TS 38.181: test condition corrections Summary of change: - Introduction of missing declarations in table 4.6-1. - Simplified and clarified Annex B structure, based on declarations added to clause 4.6. - Correction of the wording in the Informative annex (shall can) |
| R4-2409559 | Huawei, HiSilicon | CR to TS 38.181: Correction for OTA ICS test setup Summary of change: Correction for OTA ICS test setup, to fix cross-reference and remove [].. |
| R4-2409562 | Huawei, HiSilicon | CR to TS 38.181: removal of [] for MU/TT values Summary of change: Removal of outstanding [] from MU and TT values. Other editorial corrections. |
| R4-2409675 | NTT DOCOMO, Inc. | (NR_bands_R17_BWs-Core) CR for Tx intermodulation core requirements in certain region Summary of change: Added NOTE in table 6.7.2.1-1, 6.7.3.1-1 and 9.8.2-1 for BS type 1-C/1-H/1-O Co-location Tx intermodulation regional requirements for Band n79.. |
| R4-2409676 | NTT DOCOMO, Inc. | (NR_bands_R17_BWs-Core) CR for Tx intermodulation requirements in certain region Summary of change: Added NOTE in table 6.7.5.1.1-1 and table 6.7.5.2.1-1 for BS type 1-C/1-H Co-location Tx intermodulation regional requirements for Band n79. |
| R4-2409677 | NTT DOCOMO, Inc. | (NR_bands_R17_BWs-Core) CR for OTA Tx intermodulation requirements in band n79 Summary of change: Added NOTE in Table 6.8.5.1-1 for BS type 1-O OTA Tx intermodulation regional requirements for Band n79. |
| R4-2409779 | Huawei, HiSilicon | CR to TS 38.141-1: Clarification on RMR terminology Summary of change: Missing definition and abbreviations added.. |
| R4-2409780 | Huawei, HiSilicon | CR to TS 38.108: Correction of the FR1-NTN frequency range Summary of change: Removal of outstanding [], and other editorial corrections to follow drafting rules.. |

## Open issues summary

### Sub-topic 1-1: Multiple carrier operation for n100/n101

RAN4#110bis considered the topic of Multiple carrier operation for n100/n101, and agreed on a WF in R4-2406624:

“During RAN4#111, RAN4 to reflect the “multiple carriers using wideband technologies” wording and related considerations on “coordination procedure or other mitigation measures” from Decides 4 in ECC/DEC/(20)02, into Rel-17 versions of TS 38.104 and TS 38.141-1, where the exact wording to be implemented in related CRs is FFS.”

Several discussion papers (R4-2407121, R4-2409466, R4-2409533) make proposals for how to address this topic and three sets of CRs are proposed (Cat F in R4-2407122, R4-2407123, R4-2409448, R4-2409465, R4-2409534, R4-2409536). There is also a Draft LS proposed in R4-2309538.

Two open issues are identified.

Issue 1-1: How to include the multiple carrier clarification in specs

- Proposals

  - Option 1: Add multi-carrier clarification to TS 38.104 and TS 38.141-1 (Rel-17), based on text in ECC Decision (20)02.

  - Option 2: Add multi-carrier clarification to TS 38.104 and TS 38.141-1 (Rel-17), based on text in ECC Decision (20)02. Also add Note(s) that 1 carrier in n100 and 1 carrier in n101 are not considered multicarrier cases.

- Recommended WF

  - TBA

Issue 1-2: LS response

- Proposals

  - Option 0: No LS.

  - Option 1: Send an LS response to ETSI TC RT asking for feedback on the motivation and background for the multiple carrier scenario.

- Recommended WF

  - TBA

# Topic #2: UE/BS EMC requirements (4.3)

## Companies’ contributions summary

Discussion papers

| T-doc number | Company | Title/Proposals |
| --- | --- | --- |
| R4-2407801 | ZTE Corporation, Sanechips | Discussion on NR repeater EMC performance criteria Proposal 1: Edit the performance criteria of NR repeater, by improving the power accuracy test level from ± 2dB/± 3dB to ± 1dB. |

Submitted CRs (Cat A CRs not listed)

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2407855 | ZTE Corporation, Sanechips | (NR_repeaters-Perf) CR to TS 38.114 EMC performance criteria R17 Summary of change: Edit the test level of NR repeater’s performance criteria to align with LTE repeater. |
| R4-2408240 | ZTE Corporation, Sanechips | (NR_newRAT) CR to TS 38.113 BS EMC maintenance R15 Summary of change: Editing the reference that is pointed to incorrect clauses in RF Spec. Updating the test configuration tables to align with RF Spec. Deleting some empty void tables and clauses. |

## Open issues summary

-

# Topic #3: NB-IoT/eMTC core & perf. requirements for NTN: SAN RF requirement and conformance testing (5.2.2.2)

## Companies’ contributions summary

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2407466 | CATT | (LTE_NBIOT_eMTC_NTN_req-Core)CR for TS 36.108, Correction on antenna connector for RF requirements Summary of change: - 1) Remove symbol Pmax,c,AC. - 2) Change “antenna connector” to “TAB connector for SAN type 1-H” in sub-clause 6.5.4.1. - 3) Remove “at the antenna connector” in sub-clause 7.5.2. |
| R4-2407467 | CATT | (LTE_NBIOT_eMTC_NTN_req-Core)CR for TS 36.108, Correction on general SAN transmitter spurious emission limits Summary of change: - Change Prated,c,sys to Prated,t,sys in Table 6.6.5.2.1-1. - Change Prated,c,TRP to Prated,t,TRP in Table 9.7.5.2.2-1. |
| R4-2407468 | CATT | (LTE_NBIOT_eMTC_NTN_req-Perf)CR for TS 36.181, Correction on antenna connector for demodulatin requirements Summary of change: - Remove symbol Pmax,c,AC. - Remove one of two symbols Prated,c,TRP. - Change “antenna connectors” to “TAB connectors”. - Change D54 to D44, and Change Dxx to D44 for OTA REFSENS receiver target reference direction. - Remove conducted initial conditions in sub-clause 11.5.1.4.1. |
| R4-2407469 | CATT | (LTE_NBIOT_eMTC_NTN_req-Perf)CR for TS 36.181, Correction on general SAN transmitter spurious emission limits Summary of change: - Change Prated,c,sys to Prated,t,sys in Table 6.6.5.5-1. - Change Prated,c,TRP to Prated,t,TRP in Table 9.7.5.5-1. |
| R4-2407470 | CATT | (LTE_NBIOT_eMTC_NTN_req-Perf)CR for TS36.181, Correction on Number of RX antennas in header row of tables for radiated demodulation test requirements Summary of change: Change “Number of RX antennas” to “Number of demodulation branches”. |
| R4-2409539 | Huawei, HiSilicon | CR to TS 36.108: Band-agnostic OBUE requirement Summary of change: Removal of band number, being replaced by reference to clause 5.2. |
| R4-2409540 | Huawei, HiSilicon | CR to TS 36.181: Band-agnostic OBUE requirement Summary of change: Removal of band number, being replaced by reference to clause 5.2. |

## Open issues summary

-

# Topic #4: Other Rel-18 non-spectrum related WIs: 
BS RF requirements (5.2.8.2)

## Companies’ contributions summary

Discussion papers

| T-doc number | Company | Title/Proposals |
| --- | --- | --- |
| R4-2409120 | NTT DOCOMO, Inc. | (TEI18) Discussion on definition of FR2 home class BS Proposal 1: It’s suggested to regard home class as one type of LA with the note in the specification TS38.104 as follows. |

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2408081 | Ericsson | (NR_ext_to_71GHz-Core) CR to TS 38.104: Correction of FR2-2 OTA total power dynamic range in sub-clause 9.4.3 Summary of change: - Values in Table 9.4.3.3-2 are updated to be consistent with other values for OTA total power dynamic range. - [] removed. |
| R4-2409119 | NTT DOCOMO, Inc. | (TEI18) CR for TS38.104 define FR2 home class as one of kind of LA BS Summary of change: Regard FR2 HBS as one kind of LA and clarify that all LA requirements are also applicable for such low power BS with following updates: - Add some note in 4.4 for BS class definition. |

## Open issues summary

-

# Topic #5: IoT NTN enhancements: SAN RF requirements maintenance (8.3.1)

## Companies’ contributions summary

Submitted CRs

| T-doc number | Company | Title / Summary of change |
| --- | --- | --- |
| R4-2408199 | NEC | CR to 36.108: Minimum requirements for unwanted emissions for SAN Summary of change: For SAN type 1-H, rename the subclause titles for basic limits and add new subclauses for minimum requirements. For SAN type 1-O, add text to specify the minimum requirements. |

## Open issues summary

-