3GPP RAN WG4 Meeting #112	R4-2411335

Maastricht, NL, August 19 – August 23, 2024

Title: 	Discussion on Core requirements of carrier phase positioning

Source: 	CATT

Agenda item:	6.1.1.2

Document for:	Discussion

Introduction

In RAN4#111 meeting, fruitful progress regarding the core requirements of sidelink and carrier phase positioning was achieved which are captured in the WF [1]. In this paper, we will further discuss the related open issues.

Discussion

Carrier phase positioning

In our understanding, the issue 2-1-2 in WF [1] has been addressed by the agreements in issue 2-1-1. Both issues are copied below for reference.

Issue 2-1-2: Measurement period requirements for DL RSCP/DL RSCPD with multiple PFLs configured

Proposals: - Option 1: - When multiple PFLs are configured in the assistance data and DL RSCPD is requested with RSTD, the measurement period is given by TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation. - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window. - When multiple PFLs are configured in the assistance data and DL RSCP is requested with UE Rx-Tx, the measurement period is given by TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation. - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window. - Option 2: - When multiple PFLs are configured where RSCPD is supposed to be measured in PFL j, support option 1 with the following updates (taking RSCPD measurement as the example) - For UE supporting both FG 41-2-3 and FG 42-2-8，TRSCPD with RSTD, j is the measurement period for both RSCPD and RSTD in PFL j by taking time window into account. - For UE supporting FG 41-2-3 only, TRSCPD with RSTD, j=max(TRSCPD, j,TRSTD, j), where TRSCPD, j is the measurement period for RSCPD in PFL j by taking time window into account, and T RSTD, j is the legacy measurement period for RSTD in PFL j without time window. - For UE not supporting FG 41-2-2 or FG 41-2-8, TRSCPD with RSTD, j=TRSTD, j.

Issue 2-1-1: Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD

Agreements: For UE configured for CPP measurement with legacy measurement with multiple PFLs, when LMF does not configure measurement time window(s) for a PFL - UE performs DL RSCPD/RSCP measurement on a single PFL that is common between the reference TRP and the target TRP. - Existing requirements for RSTD/UE Rx-Tx without time window apply. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and does not support FG 41-2-3: - UE performs DL RSCPD/RSCP measurement on the indicated PFL by the network. - Existing legacy requirements without time window apply for both CP measurements and legacy measurements. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3, but does not support FG 41-2-8/41-2-9: - Existing legacy requirements without time window apply for legacy measurements. - CPP measurement requirements apply for CP measurements for the PRS resources associated to the time window have occasions only within the measurement time window. For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3 and FG 41-2-8/41-2-9: - CPP measurement requirements apply for both legacy measurements and CP measurements. FFS: Whether and/or how to define the measurement period requirements for DL RSCPD/RSCP measurements when aperiodic time window is configured.

The only leftover is how to define the measurement period requirements for DL RSCPD/RSCP measurements when an aperiodic time window is configured.

In our understanding, since the time window is one shot, there is no need to denote the measurement period by an equation. The measurement period requirements for such case which is limited by the duration of the aperiodic time window can be captured in the text format in corresponding clauses, e.g., when an aperiodic time window is configured, the measurement period is upper-bounded by the duration of this time window. A draft CR is submitted [2].

Proposal 1: When an aperiodic time window is configured, the measurement period requirements of CPP is upper-bounded by the duration of this time window.

Conclusions

This paper discussed the core requirements for CPP measurements, and the following proposals are provided:

CPP:

Proposal 1: When an aperiodic time window is configured, the measurement period requirements of CPP is upper-bounded by the duration of this time window.

References

[1] R4-2410193, WF on SL positioning and carrier phase positioning, CATT, RAN4#111.

[2] R4-2411333, Draft CR on core requirements for CPP, CATT, RAN4#112.