# 3GPP TSG-RAN WG4 Meeting #112	R4-2411450

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.4.2

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#111 meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2]. In this contribution, we continue the discussion on the open issues in [1].

# General aspects

Issue 1-1-1: Core requirements to be specified for LP-WUR measurement

- Proposals - P1: RAN4 shall focus on following cases (Samsung CMCC vivo ZTE): - Case 1: LP-SS for OOK based LP-WUR; - Case 2: Existing PSS/SSS for OFDM based on LP-WUR; - P1-1: Suspend the discussion of OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state, until RAN1 achieve the consensus about whether it can be target for sync and RRM measurement. (Samsung CMCC) - P1-2: It is not necessary for RAN4 to define measurement requirements for OFDM-based LP-SS RRM measurement at least in this stage. (ZTE) - P2: New requirements may need be specified for the interruption time between LP-WUS received paging indication to MR ready for Paging monitoring (Samsung xiaomi Docomo) - P2-1: RAN4 to define requirements on UE paging monitoring to make sure that UE does not miss paging when entry/exit condition for LP-WUS monitoring is met (HW) - P3: RAN4 to discuss what is the impact of different architecture options on LP-WUR requirement and study the need to define requirements separately for supported architecture types (Nokia) - P4: The feasibility of OFDM-based LP-SS measurement shall be discussed, a LS could be sent to RAN1 on RAN’4 conclusion. (ZTE) Background: RAN4 110bis agreement - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS Recommendations: - Update the first bullet of RAN4 110bis’s agreement on issue 2-2-1 as - Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Suspend the discussion of OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state until more RAN1 progress is available

In last meeting RAN4 has no conclusion on the revision of the RAN4#110bis agreement, and at least the OOK based LP-WUR serving cell measurement shall be prioritized. RAN1 still didn’t conclude on the overlaid OFDM sequence for LP-SS, and in last RAN1 meeting, only the followings were achieved.

Agreement Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS - Down select among gold sequence, m sequence and ZC sequence - FFS the overlaid OFDM sequence is time or frequency domain sequence. - FFS how to reuse the existing sequences - Note: Strive to minimize the impact on OOK detection performance - If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS

With such situation in RAN1, we are fine with the recommendation from the moderator in last meeting that OFDM-based LP-WUR serving cell measurement requirement based on LP-SS at Idle/Inactive state can be discussed after more RAN1 made concrete conclusion to support overlaid OFDM sequence for LP-SS.

Proposal 1: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- FFS on measurement requirements for OFDM based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state after RAN1 has concrete conclusion.

Issue 1-1-2: Cases/states to be considered for RRM relaxation

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

In case#1, when UE fully offload from MR to LR, we assume measurement results from LP-WUR will decide whether the MR shall be waken up or not, however, the MR will be turned off or in ultra-deep sleep mode. The FFS part is mainly about whether the higher priority measurement shall still be kept on MR or not. Current higher priority measurement in RAN4 has two main delays: 1min and 1 hour, as followings.

In order to save the power of UE, if MR still needs to wake up every 1min, we didn’t see much gain for this feature, but we are open to discuss if higher priority measurement on MR can be kept and performed as slow as 1 hour per layer. For simplicity of this feature at UE, we prefer to not consider higher priority layer measurement on MR when the MR is fully offloaded. Thus, the MR neighbor cell measurement shall not be considered for case #1.

But we are still open to discuss the condition for higher priority layer measurement in case#1 if majority companies would like to support it.

Proposal 2: higher priority layer neighbor cell measurement shall also be OFF for case #1.

We think the case #1 is the case when UE has very good quality of serving cell, and UE can fully offload from MR to LR. But if the serving cell quality is not that good as case #1, UE may consider to use MR relaxation for serving and neighbor cell measurement. Case #2 is a little bit weird case to consider, since when the LR is ON and neighbor cell measurement is OFF on MR, it seems not necessary for UE to keep MR awake for a relaxed serving cell measurement. In our view the threshold to trigger corresponding offloading and relaxation can be as following figure,

Figure 1. threshold assumption for different cases

If the serving cell measurement shows above the threshold of neighbor cell measurement (or a new threshold of MR offloading), UE can turn off the MR, and only rely on LR to perform serving cell measurement, and no neighbor cell measurement is needed (including higher or equal/lower priority layers measurement). If the neighbor cell measurement is triggered (e.g., below the threshold of intra-freq or inter-freq measurement triggering condition) but serving cell quality is still above threshold of LP-WUS monitoring threshold, the MR can be ON with relaxation for serving and neighbor cell measurement while the LR is also ON for serving cell measurement, i.e., case #3. Then if UE is below LP-WUS monitoring threshold but above the not-at-cell-edge threshold for RRM relaxation, then UE will be in a legacy power saving mode, which means, LR is OFF and MR is ON with neighbor cell measurement relaxation (no serving cell relaxation). Even further, if UE is below not-at-cell-edge threshold for RRM relaxation, it means UE is in legacy IDLE/Inactive mode status without any RRM relaxation.

In order to support the above logic, we have some assumptions for such threshold design, even though the final threshold decision shall be made by RAN2. Firstly, we assume the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.  Secondly, the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring, as LP-WUS monitoring will require the T/F tracking on LR, e.g., via LP-SS.

Proposal 3: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

- the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.

Based on the assumption in proposal 3 and the illustration in figure 1, we believe the case#2 and case#4 can be removed at least at this stage, but of course we can revisit the cases if RAN1/2 has some conclusions conflicting with RAN4 assumptions. And the following case table can be assumed in RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 4: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

- Proposals - P1: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. (Apple xiaomi oppo LG CT ZTE) - RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. If both LP-WUR and MR are ON, RAN4 to discuss whether UE uses LP-WUR measurement to decide MR relaxation or UE uses both LP-WUR and MR measurement to decide MR relaxation (Apple) - P1-1: RAN4 to specify MR RRM relaxation requirements for serving cell measurements in Idle/Inactive mode when (MTK) - LP-SS based LP-WUR RRM measurement is activated, or - PSS/SSS based LP-WUR RRM measurement is activated Not to consider RRM relaxation based on the hybrid measurements of both (LP-SS and SSS).

One thing to clarify that, LR measurement will be used for UE to decide switch from case#1 to case#3, that is, serving cell measurement on LR will be used to decide (1)if the neighbor cell measurement is triggered and (2)if the neighbor measurement can be relaxed. Previously in R16/17/18, the neighbor cell triggering and relaxation criteria checking is based on the MR measurement, and therefore it means LR measurement result shall be comparable to MR measurement or shall be equivalent to MR measurement with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 5: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation.

Proposal 6: to support proposal 5, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

In case#3, If LR is ON and MR is ON with relaxed measurement, both the MR and LR will perform the serving cell measurement, it’s still unclear that which measurement(s) will be used to check the certain criteria, e.g., LP-WUS monitoring entry/exit criteria or MR ON/OFF criteria.

In WID, another important enhancement is: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. Even though conditions for offloading RRM serving cell measurement from MR to LR and conditions for relaxation of MR serving cell and neighbor cell measurements is mainly decided/designed by RAN2, RAN4 can investigate the mobility performance corresponding to such relaxation, and the mismatch/bias between LP-WUR based measurement result and MR based measurement result. For instance, if LP-SS is used for the measurement to decide when the MR shall be waken up or not, such LP-SS based measurement result at least shall be comparable to the MR based measurement results or some bias offset shall be defined; otherwise, LP-SS triggers MR to wake up for mobility but MR based measurement results may tell that the mobility is not needed after MR wakes up.

In last RAN2 meeting, there was also some discussion that,

Serving cell measurements related For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. Neighboring cell measurements related RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

RAN4 shall specify MR relaxation requirements at idle/inactive mode for UE supporting LP-WUR, and there are still some open issues to discuss for the principle of such relaxation.

Proposal 7: the criteria design for RRM relaxation of UE MR for both serving and neighbor cell measurements can be left to RAN2 to decide

Proposal 8: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

- RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period.

- If both LR and MR are ON, RAN4 to discuss whether UE uses LR measurement or UE uses both LR and MR measurement to check with the relaxation/offloading/LP-WUS-monitoring criteria (especially in case#3).

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

- Proposals - P1-1: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring: (Qualcomm) - The number of measurement samples needed for filtering. - The number of consecutive times the UE needs to meet the entry/exit criteria - P1-2: Define requirements on entry/exit criteria for LP-WUR measurement/ MR RRM measurement relaxation based on RAN1 and RAN2 design (CT) - P1-3: RAN4 to discuss the requirement about exit the LR by serving cell evaluation based on the LP-RSRP/LP-RSRQ simulation result and the requirement about entry the LR by serving cell evaluation based on the SSB RSRP/RSRQ. (Ericsson)

We are open to discuss the requirement for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, but depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria. Our understanding is:

- If only MR is ON for measurement, the delay requirements for entry/exit criteria evaluation shall be same as legacy MR serving cell measurement delay requirement.

- If only LR is ON for measurement, RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

- If both LR and MR is ON for measurement (MR is in relaxation mode), RAN4 to discuss the delay requirements for entry/exit criteria evaluation and also discuss how to combine or select measurement results from MR and LR.

Regarding the time period for UE to meet the criteria, we think it shall be left to RAN2 to decide. Like the low mobility criteria for power saving, RAN2 decided the RSRP/RSRQ variance within a certain consecutive time to meet the criteria.

Proposal 9: Depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria; and followings are considered:

- If only MR is ON for measurement, the delay requirements for entry/exit criteria evaluation shall be same as legacy MR serving cell measurement delay requirement.

- If only LR is ON for measurement, RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

- If both LR and MR is ON for measurement (MR is in relaxation mode), RAN4 to discuss the delay requirements for entry/exit criteria evaluation and also discuss how to combine or select measurement results from MR and LR.

Proposal 10: Regarding the consecutive time period for UE to meet the criteria, it shall be left to RAN2 to decide.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement

- P1: RAN2 is be the main group for criteria (entry/exit conditions) design (Apple oppo vivo) - P2: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition - P3: RAN4 to firstly discuss the differences between entry/exit conditions for LP-WUS monitoring defined in RAN1/RAN2 and conditions for LP-WUR measurement. RAN4 should at least consider LP-SS and PSS/SSS when discussing the entry/exit conditions for LP-WUR measurement. RAN4 to wait for RAN1/RAN2, if the entry/exit conditions for LP-WUS monitoring are introduced by RAN1/RAN2, the thresholds on serving cell measurement result can be further discussed in RAN4. (CATT) - P4: Both serving cell quality and UE mobility should be considered for the criteria(entry/exit conditions) for MR RRM measurement relaxation &LP-WUR measurement (State2) and only LP-WUR measurement (State 3). Whether network configure the high priority frequency carrier measurement or not should also be considered when defining the criteria (entry/exit conditions). (CMCC) - P5: When UE enters the State 3 (Measurement only based on LP-WUR), The serving cell quality should at least be Srxlev > SIntraSearchP and Squal > SIntraSearchQ (CMCC) - P6: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective (LG) - P7: RAN4 to discuss serving cell evaluation for entry/exit conditions for PSS/SSS and LP-SS based LR. PSS/SSS based entry/exit evaluation can be prioritised. (Nokia)

Like in power saving feature, the criteria itself has been defined be RAN2 in TS38.304, and therefore here we also think that RAN2 is the main group for criteria (entry/exit conditions) design. However, for RRM relaxation requirement (e.g., scaling factor), RAN4 can work on the RRM measurement relaxations (e.g., scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Proposal 11: RAN2 shall be the main group for criteria (entry/exit conditions) design

Proposal 12: RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation

- Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (oppo vivo Apple) - RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2. (Apple) - P2: RAN4 should firstly discuss the meaning of MR RRM measurement relaxation on which phases and what purposes are included for RRM relaxation in the entire procedure, i.e. for cell selection/reselection or handover, for serving cell measurement or neighbor cell measurement, etc. (CATT) - P3: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation (LG) - P3a: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition

Similar as the discussion in issue 1-1-5, we also think RAN2 shall be the main group for criteria design.

Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring

- Proposals - P1: RAN4’s involvement on entry/exit conditions for LP-WUS monitoring can be triggered by other groups if necessary (Apple) - P2: For Entry/exit condition for LP-WUS monitoring, the performance of miss detection rate on LP-WUS can be considered as side condition in addition to RSRP and RSRQ (Samsung) - P3-1: The entry/exit conditions for LP-WUR measurement should be consistent with the entry/exit conditions LP-WUS monitoring (xiaomi) - P3-2: The entry/exit condition for LP-WUS monitoring and the entry/exit condition for LP-WUR measurement should be considered jointly.(CMCC) - P3-3: Same criteria and same/different entry/exit conditions can be used for LP-WUS monitoring and LP-WUR RRM measurement. (vivo) - P3-4: RAN4 to confirm that same entry/exit condition for LP-WUS monitoring applies for serving cell RRM measurement offloading (from MR to LP-WUR). (HW) Notes: MR measurements results shall be larger than the threshold and satisfy the related MR relaxation conditions. (ZTE) - P4: If there are other conditions that should be considered such as UE mobility besides serving cell RRM measurement for measurement offloading from MR to LP-WUR, RAN4 needs to provide input to RAN2 (LG) - P5: For the entry/exit conditions for LP-WUS monitoring, the threshold for the serving cell measurement performed by MR or LR shall be decided by RAN4 (ZTE) Recommendations: Suggest to check whether the following top level description: LP-WUR monitoring is triggered no later than one of cases identified in issue 1-1-2

In last RAN2 #126 meeting, it was agreed that,

- RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. - The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2 is currently working on it, and we think RAN4 work can be triggered by LS from other working groups if needed. But as we discussed in proposal 3, we think the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring, as LP-WUS monitoring will require the T/F tracking on LR, e.g., via LP-SS.

Proposal 13: RAN4’s involvement on entry/exit conditions for LP-WUS monitoring can be triggered by other groups if necessary. And RAN4 assumptions can be as in proposal 3.

Issue 1-1-8: LP-WUR status before entering offloading or after exiting offloading

- Proposals - P1: (Apple) - Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement - Alt 2: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON without RRM measurement (Apple) - Alt 3: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is OFF without RRM measurement. (Apple)

In WID, it was stated that: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. As we discussed in previous proposal 4, UE may or may not turn off the MR for serving/neighbor cell measurement, and therefore, two cases can be considered, i.e., case#1 and case #3

- Case #1: MR is OFF or deep-sleep without reception/transmission and LP-WUR is ON for serving cell measurement.

- Case #3: MR is still ON with RRM measurement relaxation on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement.

In addition, before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

Alt 2: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON without RRM measurement

Alt 3: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is OFF without RRM measurement.

We slightly prefer Alt 2/3 understanding, the reason is: this WI is not targeting to enhancement the legacy measurement performance by using bot MR and LR, and therefore it shall not change the legacy UE measurement behavior after exiting the LP-WUS monitoring status. Anyway, we are open to further discussion.

Proposal 14: before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

- Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

- Alt 2: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON without RRM measurement

- Alt 3: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is OFF without RRM measurement.

We slightly prefer alt 2 or 3.

Issue 1-1-9: Impact on specification

We think this issue can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

Proposal 15: this issue 1-1-9 can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

# LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode

Issue 1-2-2: Noise figure and others impact(RF impairment) on SNR target X

Issue 1-2-1: Methodology on specifying LP-WUR RRM requirements at Idle/Inactive mode - Proposals - P1: RAN4 to decide the Es/Iot side condition for LP-WUR based RRM requirement when we have RAN1 conclusion on target SINR for coverage and RAN4 RF conclusion on noise figure (Apple CMCC) - P2: Update the wording of previous agreement as “The outcome of RAN1’s study in Rel-19 WI on SINR target is used as the starting point for RAN4 LP-WUR requirement study”. (vivo) - P3:RAN4 to wait for the RAN1 conclusions on the target SNR (Nokia ZTE) - P3-1: RAN4 shall study the target SNR as baseline since RAN1 has agreed the target SNR shall be reported. (ZTE) - P4: Given any outcome from RAN1 discussion on the target SNR, RAN4 still need to consider other RF impairments to determine the final value for the target SNR. RAN4 to evaluate the RRM performance for LP-SS using the existing side condition of the Idle mode performance requirements (i.e., -4dB for Idle mode in FR1). (MTK) Issue 1-2-2: Noise figure and others impact(RF impairment) on SNR target X - Proposals - P1: Not consider adjustment on the target SNR from RAN1 due to the noise figure of LP-WUR since it has already been considered in RAN1 assumption. (oppo)

In WID, it was agreed that “The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.” However, the legacy RRM requirement is defined based on SSB coverage rather than coverage of PUSCH for msg3. So RAN4 needs more discussion to decide if current SNR of -3dB(mainly for serving cell) or -6dB(mainly for neighbor cell) can be used to represent the equivalent coverage as PUSCH for msg3. As RAN4 agreed in previous meeting, the SINR for RAN4 requirement can be decided based on RAN1 conclusion on coverage and RAN4 RF conclusion on noise figure. RAN4 RRM can wait until we get these inputs from both RAN1 and RAN4 RF.

In last RAN meeting, some agreements have been made:

Agreement The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes: - NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB - NF of LR = NF of MR+ 5dB: the reported SNR value range is [-6.5, 5.58] dB - NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB - Note 1: The above is observed based on the following assumptions used for calibration: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

However, there is no concrete conclusion of NF from RF, therefore we still think RRM session needs to wait the NF assumption from RF session and then we can decide which target SNR can be chosen from RAN1 candidate list [-3dB, -0.5dB, 2dB]

Proposal 16: RAN4 RRM session to decide which target SINR can be chosen from RAN1 candidate list [-3dB, -0.5dB, 2dB] after RAN4 RF conclusion on noise figure.

Issue 1-2-3: Accuracy for LP-WUR measurement

Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (Apple xiaomi LG Ericsson Huawei MTK ZTE) - P2: RAN4 to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline. FFS on whether or how to define dedicated accuracy requirements. (CATT CMCC) - P3: RAN4 to consider a common target accuracy when defining LP-SS based and PSS/SSS based RRM delay requirements for LP-WUR. (xiaomi) - P4: First determine the measurement accuracy based on the legacy RAN4 requirement and target SNR/SINR based on RAN1 Rel-19 study. Determine required measurement samples based on RAN4 simulations. Determine measurement delay based on sampling number and reference signal periodicity. RAN4 to decide whether the legacy EMR Idle mode accuracy requirement and Connected mode accuracy requirement can be reused for LP-WUR measurements later based on the simulation results (CT) - P5: RAN4 to define the dedicated accuracy requirement in the performance section (CT) - P6: The accuracy requirement defined for Redcap with 1Rx can be used as the base when defining requirements for LP-WUR serving cell measurement. (vivo) - P6-1: Further relaxation on the accuracy target based on reference accuracy maybe needed. The amount of relaxation could be based on RAN4’s simulation outcome. (vivo)

Regarding the accuracy requirement, if we follow the legacy accuracy requirement principle, the dedicated accuracy requirements in TS38.133 section 10 is mainly for the measurement results which is reported to network, and the only measurement accuracy requirement for IDLE/Inactive mode is the EMR feature related measurement. For LR based RRM measurement, since UE will not report anything back to network in IDLE/Inactive mode, our preference is still not to define a dedicated accuracy requirement in TS38.133 section 10, but instead we can reflect such accuracy performance in the core requirement sections as a margin, like in the following example:

Proposal 17: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Issue 2-2-4: Measurement metrics

- Proposals - P1: Follow RAN1 agreement that LP-RSRP for OOK-based LP-WUR should be used, wait RAN1’s further progress on LP-RSRQ. (oppo CMCC Ericsson ZTE) - P2: For the definition of LP-RSSI, suggest to use the linear average of total received power in all LP-SS OOK symbols. (CMCC)

In last RAN1 meeting, the agreement is as following:

Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Since the LP-SS RSRP/RSRQ measurement metrics was agreed in RAN1, RAN4 only needs to wait until the entire LP-SS design has been concluded. Normally measurement metrics is defined in TS38.215(RAN1 spec), we think RAN1 can lead to decide the measurement metrics (RSRP/RSRQ/SINR/RSSI)  standardization before RAN4 RRM requirement design during this LP-WUR WI. In our view, one thing RAN4 can discuss is the Rx number for LP-SS measurement metrics. In legacy SSB-based RSRP measurement, it specified that,

For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches.

Our understanding is that the LP-WUR is a quite low power-consumption and low-cost component on the UE, and it shall be assumed as single Rx as a more typical case for requirement design. Thus, to help completing the measurement metrics, RAN4 can discuss whether 1Rx is assumed for LR based RRM first.

Proposal 18: Single Rx is assumed for LR based RRM measurement.

Issue 1-2-5: Time/frequency sync

- Proposals - P1: RAN4 to wait conclusions from RAN1 on timing error and frequency error. (Apple vivo) - P2: RAN4 to discuss time and frequency error when agreeing on the simulation assumption for evaluating LP-WUR measurement performance. (Huawei)

RAN1 had following agreements in their last meeting:

Agreement The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR. - FFS X which is no larger than 20ppm - Initial frequency error assumption: up to company report For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y. - FFS Y which is no larger than 20ppm and lower than X - Initial frequency error assumption: up to company report

Since such Time/frequency sync will not be directly reflected in the core requirement but only reflected in the simulation assumption for evaluation. Thus, we agree with companies to discuss it in simulation assumption based on RAN1’s and RF’s agreements.

Proposal 19: Discuss Time/frequency error in simulation assumption based on RAN1’s and RF’s agreement.

Issue 1-2-7: Other considerations

In previous meeting, RAN1 also discussed the LP-WUS in eDRX, but no conclusion has been made. In our view, besides LP-WUS, we also need to investigate the UE RRM measurement for LP-SS in eDRX, e.g., whether LP-SS measurement shall follow the eDRX/DRX cycle or not. If the LP-SS based measurement is not following eDRX cycle or DRX cycle in IDLE/Inactive mode, i.e., LP-SS based measurement will follow the LP-SS periodicity, the RRM requirement shall only be defined based on the LP-SS periodicity. In our view, DRX is only applied for MR but not for LP-WUR, and therefore, LP-SS measurement in IDLE/Inactive mode shall only follow LP-SS periodicity.

Proposal 20: LP-SS measurement requirement in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Regarding the whether MR and LR are operation on same carrier or not, RAN1 is also discussing such scenarios. Our preference is to use same carrier scenario as baseline for RAN4 discussion and FFS on the other scenarios if RAN1 has agreement.

Proposal 21: to define RRM requirement, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, and FFS on the other scenarios if RAN1 has agreement.

# MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

- Proposals - P1: Regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, RAN4 to discuss (Apple) - whether serving and neighbor cell measurements can share the same criteria for triggering relaxation or not and, - whether serving and neighbor cell measurements can share the same relaxation requirement (e.g., scaling factor) or not. - P2: RAN4 needs to further study serving cell measurement relaxation, and further neighboring cell measurement relaxation by MR (Samsung CATT Nokia) - P2-1 (CATT): What is the difference between MR RRM measurement relaxation and legacy RRM measurement relaxation (e.g., not-at-cell edge or low mobility in R16/R17)? Whether the legacy criteria for RRM relaxation can be reused for relaxing serving and neighbor cell measurements for UE MR ? Whether RRM measurement of MR is used for neighbor cell measurement when the UE is operating with LP-WUR can be discussed after further input from RAN2. After determining the meaning and the criteria (entry/exit conditions) for MR RRM measurement relaxation in RAN2/RAN4, the method and requirements for MR RRM relaxation will be further discussed in RAN4. - P3: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation (CT vivo ZTE) - P2-1: Equal or larger than 8 (CMCC) - P2-2: >=16 (Huawei) - P4: RAN4 to wait other WGs’ progress to clarify the MR RRM relaxation scenario before discussing the relaxation requirement. (Ericsson) - P5: The legacy SINR condition and accuracy are re-used(HW)

In the WID, the MR relaxation scope is as following:

Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

In the power saving feature, the RRM relaxation for IDLE and connected mode is only for neighbor cell measurement, and no relaxation is allowed for serving cell measurement. The neighbor cell measurement relaxation is decided by serving cell measurement results. RAN4 needs to carefully check if both neighbor cell measurement and serving cell measurement shall share the same relaxation criteria or relaxation requirement (e.g., scaling factor). In our view, if the neighbor cell measurement relaxation is based on the serving cell measurement, it implies that neighbor cell measurement shall always have the more relax measurement than serving cell measurement. The rationale is also obvious: only if UE can get reliable serving cell measurement results, then it can use such results to decide whether neighbor cell measurement can be relaxed or not.

Proposal 22: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, neighbor cell measurement shall have the more relaxation than or equivalent relaxation as serving cell measurement.

As proposed by companies, we think 16 could be used as starting point for relaxation, and based on proposal 22, we would like to propose 16 for serving cell measurement relaxation and neighbor cell measurement relaxation.

Proposal 23: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, consider to use 16 for serving cell measurement relaxation and neighbor cell measurement relaxation .

# LP-WUR CONNECTED mode

Issue 1-5-1: LP-WUR at CONNECTED mode

- Proposals - P1: No RRM objectives is for connected mode in this WI. (Apple Docomo oppo LG CT vivo ZTE) - P2: Postponed until more progress (Ericsson) - P3: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress (Samsung)

In the WID, the scope for connected mode is to discuss the procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring, however, like in R16 WUS design, it would not have any RRM impact by using LP-WUS in connected mode. Furthermore, WID mentioned that,

In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

All the RRM/RLM/BFD/CSI measurement would be as same as legacy case, i.e., use MR to perform these functionalities. Thus, no RRM scope shall be needed for connected mode in this WI.

Proposal 24: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

# Others

Issue 1-6-1: eDRX related

- Proposals - P1: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode: how to enter and exit offloading status if eDRX is configured with PTW. (Apple) - P2: eDRX can be discussed after further input from RAN1 (Samsung)

As discussed previously, if the LP-SS based measurement is not following eDRX cycle or DRX cycle in IDLE/Inactive mode, i.e., LP-SS based measurement will follow the LP-SS periodicity, then when UE decides to turn on MR inside a PTW window (if PTW is used), UE needs to check whether rest of the PTW window is sufficient for MR based RRM measurement or evaluation time (MR based RRM measurement/evaluation shall be contained in one PTW window). Even though it’s a bit early to discuss such details, we think it’s still worthwhile to discuss how to perform offloading based on LP-SS based measurement when eDRX is configured.

Figure 2. example for MR offloading(based on LP-SS based measurement) when eDRX is configured with PTW

Proposal 25: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode:

- how to enter and exit offloading status if eDRX is configured with PTW.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- FFS on measurement requirements for OFDM based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state after RAN1 has concrete conclusion.

Proposal 2: higher priority layer neighbor cell measurement shall also be OFF for case #1.

Proposal 3: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

- the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.

Proposal 4: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 5: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation.

Proposal 6: to support proposal 5, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 7: the criteria design for RRM relaxation of UE MR for both serving and neighbor cell measurements can be left to RAN2 to decide

Proposal 8: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

- RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period.

- If both LR and MR are ON, RAN4 to discuss whether UE uses LR measurement or UE uses both LR and MR measurement to check with the relaxation/offloading/LP-WUS-monitoring criteria (especially in case#3).

Proposal 9: Depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria; and followings are considered:

- If only MR is ON for measurement, the delay requirements for entry/exit criteria evaluation shall be same as legacy MR serving cell measurement delay requirement.

- If only LR is ON for measurement, RAN4 to discuss the delay requirements for entry/exit criteria evaluation.

- If both LR and MR is ON for measurement (MR is in relaxation mode), RAN4 to discuss the delay requirements for entry/exit criteria evaluation and also discuss how to combine or select measurement results from MR and LR.

Proposal 10: Regarding the consecutive time period for UE to meet the criteria, it shall be left to RAN2 to decide.

Proposal 11: RAN2 shall be the main group for criteria (entry/exit conditions) design

Proposal 12: RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Proposal 13: RAN4’s involvement on entry/exit conditions for LP-WUS monitoring can be triggered by other groups if necessary. And RAN4 assumptions can be as in proposal 3.

Proposal 14: before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

- Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

- Alt 2: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON without RRM measurement

- Alt 3: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is OFF without RRM measurement.

We slightly prefer alt 2 or 3.

Proposal 15: this issue 1-1-9 can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

Proposal 16: RAN4 RRM session to decide which target SINR can be chosen from RAN1 candidate list [-3dB, -0.5dB, 2dB] after RAN4 RF conclusion on noise figure.

Proposal 17: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Proposal 18: Single Rx is assumed for LR based RRM measurement.

Proposal 19: Discuss Time/frequency error in simulation assumption based on RAN1’s and RF’s agreement.

Proposal 20: LP-SS measurement requirement in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Proposal 21: to define RRM requirement, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, and FFS on the other scenarios if RAN1 has agreement.

Proposal 22: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, neighbor cell measurement shall have the more relaxation than or equivalent relaxation as serving cell measurement.

Proposal 23: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, consider to use 16 for serving cell measurement relaxation and neighbor cell measurement relaxation .

Proposal 24: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

Proposal 25: RAN4 to discuss followings LP-SS based RRM issue in IDLE/Inactive mode:

- how to enter and exit offloading status if eDRX is configured with PTW.

# References

[1] R4-2410296, WF for RRM core requirements for LP-WUS/WUR, Moderator (vivo), RAN4 #111

[2] R4-2408026, Topic summary for [111][229] NR_LPWUS, RAN4 111, vivo