3GPP TSG-RAN WG4 Meeting #112                                                                R4-2411636

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:	Further study on testability and interoperability issues for AI-CSI

Agenda item:			8.17.4

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Furthermore, the study objectives with corresponding checkpoints in RAN#105 (Sep ‘24) particularly on the two sub-use cases of CSI feedback enhancements are provided [2]. From RAN4 perspective, it is tasked to further analyze the various testing options for two-sided models, i.e., particularly related to the use case of CSI compression among all (sub-)use cases discussed in Rel-18 study item.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

Based on the outcome of the study objectives captured in TR 38.843 [1], RAN4 continued the study in RAN4#110, RAN4#110bis and RAN4#111 with agreements and way forwards achieved in [4][6][8] respectively. In this contribution, we would like to continue the discussion on testability and interoperability issues for AI-CSI (including CSI compression and CSI prediction) use cases.

# 2. Discussion on CSI prediction

## 2.1 Testability on CSI prediction accuracy

Since the topic of CSI prediction has not yet been discussed in RAN4#111, Section 2.1 contain the same proposals from our contribution [7].

In RAN4#110, the agreement was achieved on CSI prediction accuracy metrics, as follows [4]:

Issue 4-1: CSI Prediction Accuracy metrics - Proposals - Option 1: Prediction accuracy can be used as KPI/metric - Option 2: Prediction accuracy cannot be used because the “correct” value is not available - Option 3: Throughput should be the default metric, others should be discussed only if throughput is not feasible - Option 4: Others Agreement: - Agree option 3 for inference only. TBD whether we use relative or absolute throughput. - Monitoring will be discussed separately.

For the detailed options of throughput, the following two options are given in last meeting:

- Option 1: Use relative throughput (e.g. predicted PMI vs random PMI) as the test metric

- Option 2: Use absolute minimum throughput as metric

Especially for relative throughput, many companies propose to use the metrics for existing PMI test, i.e., the throughput with followed PMI (in this case, by following the predicted PMI) as numerator over the throughput with random PMI as denominator.

Firstly, it is possible for UE to use the statistical model-based CSI prediction e.g., Kalman filter, Wiener filter, auto-regression, so whether or not AI/ML-based prediction is used is actually can’t be identified by both Option 1 and Option 2. From that perspective, the relative throughput can be adopted to guarantee the predicted PMI can achieve satisfactory throughput performance.

On the other hand, even by adopting the relative throughput as test metric for inference, the definition of the baseline denominator of relative throughput still needs clarification. In the existing PMI test case, in order to test UE reported PMI is accurate enough, RAN4 choose the random PMI as denominator, which is equivalent to the case of “no UE reporting”. To test the predicted PMI is accurate enough, we need to study what should be the baseline denominator:

Proposal 1: For CSI prediction accuracy metric for inference, relative throughput (i.e., throughput by following predicted PMI over the baseline throughput) can be adopted, but

- FFS the definition of baseline throughput (as denominator):

  - Option 1-1: Throughput achieved by following random PMI (with the same codebook used for the reported predicted PMI)

  - Option 1-2: Throughput achieved by following UE’s last reported PMI given by UE measurement

For the above Option 1-1 by following random PMI for baseline throughput, the final relative throughput performance is determined by both measurement and prediction, since both could contribute to the inaccuracy of predicted PMI.

Option 1-2 is given by the fact that PMI prediction is performed by UE for the time slot without explicit CSI-RS configured, while UE shall still use the measured PMI for the time slot if it contains the CSI-RS configured for PMI reporting. Hence, we can let TE to follow UE’s last reported PMI which is given by UE measurement rather than prediction. By adopting Option 1-2 as baseline throughput, we could rule out the impact from measurement inaccuracy but make the test focus on the UE prediction accuracy.

Proposal 2: For relative throughput used as CSI prediction accuracy metric for inference, Option 1-2 (Throughput achieved by following UE’s last reported PMI given by UE measurement) is adopted for baseline throughput.

## 2.2 Testability on model monitoring for CSI prediction accuracy

For RAN4’s test on the performance monitoring, however, RAN1 is still studying in the potential specification impact, including the definition of monitoring and performance metric:

Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

Obviously, in the study phase, RAN1 has just studied the concept of performance monitoring and identified the specification impact, while the detailed performance monitoring will not be discussed. Therefore, RAN4 could start the discuss on testability issue on the performance monitoring when RAN1 clarify the details for monitoring.

Proposal 3: In the study phase, no RAN4 discussion is needed on the testability issues for performance monitoring (including test method and test metrics to be used).

# 3. Discussion on CSI compression

In RAN4#110, the agreement [4] was achieved on CSI compression, and accordingly the feasibility (including testability, interoperability and other feasibility issues for standardization) of both option 3 and 4 shall be studied further.

Issue 4-2: Testing options for 2-sided model Agreement: RAN4 to further discuss only options 3 and 4

Furthermore, in RAN4#110bis and RAN4#111, the test and reference decoder definitions are further refined as below, captured in WF [6] and [8] respectively:

Agreement from RAN4#110-bis [6]: Test decoder definition: Test decoder(for UE side test): the decoder to be used in RAN4 tests and implemented in TE. it will be captured in the specifications if necessary (for example, for Option 3 it would be explicitly captured in the specifications) test decoder definition covers both Option 3 and Option 4 Agreement from RAN4#111 [8]: Reference encoder: The reference encoder is used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. Reference decoder: The reference decoder is used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE. It could be documented (in TR, WF, etc) or captured in the specifications as necessary. For option 3, for each test, a test decoder needs to be captured in the specs, a reference encoder might be needed to derive the test decoder and/or requirements and/or to validate the test decoder implementation in the TE. the same decoder might be used in multiple tests or each test could have a difference decoder. In option 3 the Reference decoder is the test decoder. For option 4, there might be a need to have reference encoder and/or reference decoder

In this contribution, we continue the study on CSI compression by focusing on the option 3 standardization issues to specify test decoder into 3GPP specification and further discuss on option 4 from the interoperability perspective.

## 3.1 Standardization for Test decoder Option 3

Based on the Release-18 study, test decoder Option 3 is targeted to have a single decoder defined in the specifications for at least a single test for any DUTs. It should be noted that RAN1 Option 1 for inter-vendor training collaboration, i.e. fully standardized reference model (structure + parameters), is the same as RAN4 Test decoder Option 3, in terms of standardization effort to specify reference model (or called test decoder in RAN4). However, the standardization target is obviously different because RAN4 Option 3 is for test decoder used in TE and RAN1 Option 1 is for reference decoder used in gNB.

Based on RAN4#110bis, RAN4 continue to study the feasibility in terms of standardization procedure. In our contribution in RAN4#110bis [5], a flow-chart (copied in Appendix for information) is proposed which could generally be accepted as the common understanding for the standardization procedure to be followed in 3GPP RAN4. In this meeting, RAN4 had extensively discussed how the RAN4 can provide AI-CSI compression results, and how to determine the scenario parameters to generate the dataset.

#### 3.1.1 Purpose of performance alignment for CNN-based evaluation

Based on RAN4#111, the following test decoder derivation procedure Option 1 is agreed, in which companies are required to “bring encoder + decoder set based on agreed parameters” firstly.

Agreement from RAN4#111 [8]: The following test decoder derivation procedure options were discussed: - Option 1 companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria) - Option 2 companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria) RAN4 agrees to work on option 1. Interested companies are invited to bring further feasibility analysis for Option 2

Furthermore, in the post-meeting email discussion after RAN4#111, the baseline system-level parameters (captured in TR38.858 and originally for Rel-18 RAN1 study) are reused to generate the dataset for observed channel model (without labels) and the detailed CNN model parameters for both encoders and decoders are “randomly” picked for verify the feasibility of Option 3 standardization.

In other words, RAN4 pretended to have completed the step 1-5 in Samsung proposed flow-chart [5], and study the feasibility of performance alignment based on specified CNN-structure and the dataset obtained by SLS under the specified scenario.

Observation 1: For the on-going evaluation (which is based on selected CNN-structure and trained over the company SLS-generated dataset under the specified scenario), it should be considered as a feasibility study on standardization procedure, i.e., for Step-6 and 7 in Samsung proposed flow-chart [R4-2405653]:

After clarifying the general purpose of this evaluation, we shall focus on the following issues based on the submitted evaluation results from companies.

Proposal 4: For the on-going evaluation (which is based on selected CNN-structure and trained over the company SLS-generated dataset under the specified scenario), at least the following issues shall be studied:

- Issue-1: Whether or not the performance alignment can be achieved, with the selected CNN-structure and the company SLS-generated dataset under the specified scenario?

- Issue-2: Performance degradation is acceptable if applying the trained model by company-A to the test dataset generated by company-B?

- Issue-3: If the performance degradation in Issue-2 is not acceptable, could aggregating datasets from multiple companies solve the problem?

#### 3.1.2 CNN-based evaluation for CSI compression

In the post-meeting email discussion after RAN4#111, the detailed CNN model parameters for both encoders and decoders are provided as follows. It should be noted that the CNN-based structures for encoder/decoder are given without strong technical justification and not yet favored by most companies, since the purpose of this evaluation is only for the feasibility study on standardization procedure.

Fig 1. Detailed model diagrams of encoder (left) and decoder (right).

Table 1. CNN model parameters for encoder.

| Model architecture parameters for encoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 47M (vs. 1.6M for eTypeII) |
|  | Model size | 448Kbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer after resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer before resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,32) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 2. CNN model parameters for decoder.

| Model architecture parameters for decoder (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | FLOPs | 740M |
|  | Model size | 7.2Mbyte |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layer (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer and direct combination |
|  | CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
|  | Kernel/Channel size within a resnet block | (3,3,128) |
|  | Size of message (feature map) passed across layers | (13,32) |

Table 3. Other model related parameters:

| Activation function | ReLU |
| --- | --- |
| Latent message size | 32 float values |
| Payload size, bits | 64 per layer. |
| Quantization | 2 bits |

When we build up the CNN-based encoder and decoder, we examined the value of FLOPs given in Table 1 and 2, and found that both values of “FLOPs” given are actually for the number of Multiply-Accumulate (MAC) operations, which is defined as a common step that computes the product of two numbers and adds that product to an accumulator [9]. Strictly speaking, it is not the FLOPs (Floating Point Operations), which is used more commonly for neural network computation complexity, and usually 1 MACs can be interpreted as 2 FLOPs.

Proposal 5: The metric of FLOPs shall be adopted used to evaluate neural network complexity, and original CNN complexity values (47M and 740M for encoder/decoder respectively) are actually for MACs, which shall be corrected by FLOPs (9.39*107 and 1.48*109 for encoder/decoder respectively).

Based on the agreed system-level simulation parameters (from TR), which is agreed in the approved WF [8], we have performed the system-level simulation to obtain the dataset, which is randomly split for training and validation. Based on the obtained dataset, the CNN-based model is trained, with the following training parameters considered:

Table 4. Model training parameters

|  | Parameters | Values |
| --- | --- | --- |
| Model training | Loss function | SGCS |
|  | Training/validation datasets | Channel model for training: UMa Channel based on TR 38.901 (details are given in the agreed WF [8]) Total dataset: 25K samples |
|  | Hyperparameters | Learning rate = 0.001, batch size = 200, optimization algorithm = Adam |
| Others | Cross-validation details | Dataset randomly split for training/validation |

#### 3.1.3 Performance analysis

We firstly provide the evaluation results and complexity

Table 5. SGCS Performance for CNN-based Model Trained over Samsung-generated dataset

| Encoder | Decoder | SGCS |
| --- | --- | --- |
| Back-bone: CNN Number of model parameter: 125K FLOPS: 9.39*107 Depth: 6 Res blocks Feature maps: 32 | Back-bone: CNN Number of model parameter: 1.8M FLOPS: 1.48*109 Depth: 6 Res blocks Feature maps: 128 | 0.76 |

Proposal 6: RAN4 shall use the SGCS performance reported from companies to confirm or disprove the performance alignment firstly, with SLS details to generate dataset to be further compared.

To further evaluate the generalization performance (in other words, to evaluate the performance degradation if applying the trained model by company-A to the test dataset generated by company-B, as mentioned in Issue-2 of Proposal 4), we utilize the following external datasets for comparison:

- CAICT shared dataset (Link: https://pan.caict.ac.cn:443/link/96C9DB50C5C075F0F155FC1CF9543B4C)

- OPPO shared dataset (Link: www.wireless-intelligence.com)

Here are some general comparisons between three datasets:

Table 6. Comparison of datasets from three sources

|  | Samsung dataset | CAICT dataset | OPPO dataset |
| --- | --- | --- | --- |
| # of total samples | 25K | 100K | 600K |
| Simulation parameters | Carrier Frequency: 2GHz # of drops: 50 # of UEs per drop: 500 # of slots per UE: 1 | Carrier Frequency: 2.1GHz # of drops: 100 # of UEs per drop: 1000 # of slots per UE: 1 | Carrier Frequency: 4GHz # of drops: 1 # of UEs per drop: 3000 # of slots per UE: 200 slot interval: 100 |

Here we provide the performance evaluation for the generalization performance for models trained in difference datasets.

Table 7. Generalization performance for models trained in difference datasets

| Encoder | Decoder | Testing data Training data | SRI-B dataset | CAICT dataset | OPPO dataset |  |
| --- | --- | --- | --- | --- | --- | --- |
| Back-bone: CNN Number of model parameter: 125K FLOPS: 9.39*107 Depth: 6 Res blocks Feature maps: 32 | Back-bone: CNN Number of model parameter: 1.8M FLOPS: 1.48*109 Depth: 6 Res blocks Feature maps: 128 | Samsung dataset | 0.76 | 0.2995 | 0.298 |  |
|  |  | CAICT dataset | 0.390 | 0.71 | 0.612 |  |
|  |  | OPPO dataset | 0.52 | 0.65 | 0.72 |  |

Based on the results from above Table 7, we can reach the following observations on the model generalization.

Observation 2: The neural network model trained in one dataset could suffer the substantial degradation when the model is tested over other datasets from other sources.

Observation 3: Possible reason for the above Observation 2 is (a) limited model generalization; (b) the training dataset size is not large enough; (c) the distribution for one dataset could be not representative enough.

Furthermore, we also evaluated the performance by limiting the size of three dataset into the similar level, and observed that the performance comparison is similar as Table 7, which can help us to confirm that the reason (b) could not be the major reason.

Proposal 7: For CSI compression scenario, RAN4 can’t directly confirm the generalization performance for the company-wise model trained by using the dataset generated by SLS individually, even based on the detailed simulation parameters.

# 4. Conclusion

In this contribution, we provided our viewpoints on the on the interoperability and testability aspects for AI-CSI use case, accordingly the following observations and proposals are obtained:

AI-CSI prediction:

Proposal 1: For CSI prediction accuracy metric for inference, relative throughput (i.e., throughput by following predicted PMI over the baseline throughput) can be adopted, but

- FFS the definition of baseline throughput (as denominator):

  - Option 1-1: Throughput achieved by following random PMI (with the same codebook used for the reported predicted PMI)

  - Option 1-2: Throughput achieved by following UE’s last reported PMI given by UE measurement

Proposal 2: For relative throughput used as CSI prediction accuracy metric for inference, Option 1-2 (Throughput achieved by following UE’s last reported PMI given by UE measurement) is adopted for baseline throughput.

Proposal 3: In the study phase, no RAN4 discussion is needed on the testability issues for performance monitoring (including test method and test metrics to be used).

AI-CSI compression:

Observation 1: For the on-going evaluation (which is based on selected CNN-structure and trained over the company SLS-generated dataset under the specified scenario), it should be considered as a feasibility study on standardization procedure, i.e., for Step-6 and 7 in Samsung proposed flow-chart [R4-2405653]:

Proposal 4: For the on-going evaluation (which is based on selected CNN-structure and trained over the company SLS-generated dataset under the specified scenario), at least the following issues shall be studied:

- Issue-1: Whether or not the performance alignment can be achieved, with the selected CNN-structure and the company SLS-generated dataset under the specified scenario?

- Issue-2: Performance degradation is acceptable if applying the trained model by company-A to the test dataset generated by company-B?

- Issue-3: If the performance degradation in Issue-2 is not acceptable, could aggregating datasets from multiple companies solve the problem?

Proposal 5: The metric of FLOPs shall be adopted used to evaluate neural network complexity, and original CNN complexity values (47M and 740M for encoder/decoder respectively) are actually for MACs, which shall be corrected by FLOPs (9.39*107 and 1.48*109 for encoder/decoder respectively).

Proposal 6: RAN4 shall use the SGCS performance reported from companies to confirm or disprove the performance alignment firstly, with SLS details to generate dataset to be further compared.

Observation 2: The neural network model trained in one dataset could suffer the substantial degradation when the model is tested over other datasets from other sources.

Observation 3: Possible reason for the above Observation 2 is (a) limited model generalization; (b) the training dataset size is not large enough; (c) the distribution for one dataset could be not representative enough.

Proposal 7: For CSI compression scenario, RAN4 can’t directly confirm the generalization performance for the company-wise model trained by using the dataset generated by SLS individually, even based on the detailed simulation parameters.

# 5. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402390, “Discussion on testability and interoperability issues for AI-CSI”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] R4-2405653, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#110bis, April 2024.

[6] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[7] R4-2409762, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#111, May 2024.

[8] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[9] https://en.wikipedia.org/wiki/Multiply%E2%80%93accumulate_operatio

# 6. Appendix-1: Standardization flow-chart in [R4-2405653]

Initially provided in our contribution in RAN4#110bis [5], the flow-chart in the below figure is generally accepted as the common understanding for the standardization procedure to be followed in order to fully specify the test decoder and corresponding performance requirement for AI/ML-based CSI compression using two-sided model in 3GPP. Here the flow-chart is provided in the Appendix for information to facilitate the discussion for test decoder standardization.

Step-1:  Identify necessary Model Architecture Parameters

Standardization Procedure End

for a certain use case
(e.g., CSI compression for precoding matrix under certain config.)

Model architecture parameters could include: Model type, Model depth, Layer type/size, Quantization, etc.

Model training procedure, loss function, training datasets, hyperparameters, etc.

Step-3:  Companies provide two-sided model design based on their own study/preference

Step-4:  Performance comparison based on different companies’ en/decoder designs

Yes

No

Step-6:  Performance alignment by companies based on agreed model architecture/training parameters

Performance comparison in terms of metrics like NMSE, SGCS, etc.

No

Yes

No

Standardization 
Procedure Start

Step-2:  Identify necessary Model training Parameters

Test decoder is expected to be generated in this step

Reference encoder is assumed, but leave enough implementation flexibility to vendors (similar to Demod alignment for MMSE-IRC)

Step-8:  performance alignment 
for encoder design by companies 
based on assumptions on reference encoder

Yes

RAN4 performance requirement obtained (for certain reference encoder)

Step-10:  Derive RAN4 performance requirement

Step-5: RAN4
agree on two-sided model architecture
 / training parameters?

Step-7: RAN4 agree on test decoder 
(which can be fully specified in spec.)

Step-9: RAN4 achieve performance alignment?

Figure A.1. Standardization procedure to fully specify test decoder and RAN4 performance requirement