3GPP TSG-RAN WG4 Meeting#112		R4-2411867

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.20.3.2

Source: 	Spreadtrum Communications

Title: 	Discussion on RF requirements impact for ambient IoT device

Document for:	Discussion

Introduction

In RAN4#111 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

### Issue 3-3: TX Agreement: - Use the following table as starting point for RF requirements impact study. The table is for information. - Encourage companies to provide views on RF requirements impact for different device types. RF Requirement for A-IoT device- TX part. Tx requirement; Transmit output power; Maximum output power. Output power dynamic; Transmit OFF power. Transmit time mask. Minimum output power. Power control requirement. Transmit ON/OFF power; Transmit OFF power. ON/OFF time mask. Transmit signal quality; Frequency error. EVM. In band emissions. Carrier leakage. Output RF spectrum emissions; Occupied bandwidth. SEM. ACLR. Spurious emissions. Transmit intermodulation. ### Issue 3-4: RX Agreement: - Use the following table as starting point for RF requirements impact study. The table is for information. - Encourage companies to provide views on RF requirements impact for different device types. RF Requirement for A-IoT device- RX part. RX requirement; Reference sensitivity. Maximum input power. ACS. ASCS. In-band blocking. Out-of-band blocking. Receiver intermodulation. Rx spurious emission. ### Issue 3-5: testability Agreement: - FFS on whether conducted conformance test is feasible for AIOT devices. - FFS on OTA test method, performance metric, etc.

Discussion

TX RF requirements

According to the WF [1], we provide some initial views about RF requirements for A-IoT device TX part among different device types as follows in Table 1.

Proposal 1: Some initial views about TX RF requirements for A-IoT devices in Table1.

Table1: some initial views about TX RF requirements for A-IoT devices

| RF Requirement for A-IoT device- TX part |  |  | Device1 | Device2a | Device2b |
| --- | --- | --- | --- | --- | --- |
| Tx requirement | Transmit output power | Maximum output power | transmit output power up to CW, need to meet the coverage target of 10meters and regulatory requirement | Device1: transmit output power up to CW, need to meet the coverage target and regulatory requirement | Device 2b: need to define max output power,PC3 as a starting point |
|  |  | Transmit time mask | Repeated with transmit ON/OFF time mask |  |  |
|  |  | Minimum output power | Need to define, to ensure that signal is not submerged by noise | need to define minimum output power | need to define minimum output power |
|  |  | Power control requirement | no need to define, power is decided by CW power and the distance between CW and device. No need to control additional gain | Need to define, device 2a has amplifier, amplifier gain impact power, need to define related power tolerance | Need to define, legacy UE requirements as a starting point |
|  | Transmit ON/OFF power | Transmit OFF power | no need to define, there is no active devices | need to define transmit off power, amplifier will have power consumption | need to define transmit off power |
|  |  | ON/OFF time mask | No need to define | Need to define, switching between impedance matching and mismatch | Need to define, the legacy UE requirement as a starting point |
|  | Transmit signal quality | Frequency error | No need to consider carrier frequency error. Need to consider sample frequency error. | No need to consider carrier frequency error. Need to consider sample frequency error. | Need to define, consider CFO |
|  |  | EVM | Need to define, depends on waveform | Need to define, depends on waveform | Need to define, depends on waveform |
|  |  | In band emissions | No need to consider image suppression, just need to consider general requirement | If device2a support large shift, image suppression need to consider; general requirement need to consider | The legacy UE requirement as a starting point |
|  |  | Carrier leakage | no need to define, carrier from CW | no need to define, carrier from CW | need to define , device 2b is generated UL, the legacy carrier leakage requirement as a starting point |
|  | Output RF spectrum emissions | Occupied bandwidth | depends on chip rate | depends on chip rate | depends on chip rate |
|  |  | SEM | depends on co-existence study | depends on co-existence study | depends on co-existence study |
|  |  | ACLR | depends on co-existence study | depends on co-existence study | depends on co-existence study |
|  |  | Spurious emissions | depends on co-existence study and related regulatory requirements | depends on co-existence study and related regulatory requirements | depends on co-existence study and related regulatory requirements |
|  | Transmit intermodulation |  | Need to consider CW with two tone intermodulation | Need to consider CW with two tone intermodulation | Need to consider intermodulation |

With regard to device with the ultra-low complexity and power consumption, therefore we should not define many kinds of RF requirements. Otherwise it will cause more testing cost.

Proposal 2: With the ultra-low complexity and cost, the test cost of Device1 should be reduced as much as possible.

RX requirements

According to the WF [1], we provide some initial views about RF requirements for A-IoT device RX part among different device types as follows in Table 2.

Proposal 3: Some initial views about RX RF requirements for A-IoT devices in Table2.

Table2: some initial views about RX RF requirements for A-IoT devices

| RF Requirement for A-IoT device- RX part |  | Device1 | Device2a | Device2b |
| --- | --- | --- | --- | --- |
| RX requirement | Reference sensitivity | Refer to co-existence simulation parameters (e.g.,-36dBm) | Refer to RAN1’s LLS result, the traditional sensitivity formula as a starting point | Refer to RAN1’s LLS result, the traditional sensitivity formula as a starting point |
|  | Maximum input power | Need to consider to meet the dynamic range of envelop detection | Need to consider to meet the dynamic range of envelop detection | Legacy UE requirement as a starting point |
|  | ACS | depends on co-existence study | depends on co-existence study | depends on co-existence study |
|  | ASCS | depends on co-existence study | depends on co-existence study | depends on co-existence study |
|  | In-band blocking | depends on co-existence study | depends on co-existence study | depends on co-existence study |
|  | Out-of-band blocking | depends on co-existence study | depends on co-existence study | depends on co-existence study |
|  | Receiver intermodulation | Need to consider CW intermodulation | Need to consider CW intermodulation | Need to consider CW intermodulation |
|  | Rx spurious emission | depends on co-existence study and related regulatory requirements | depends on co-existence study and related regulatory requirements | depends on co-existence study and related regulatory requirements |

Testability

Agreement: - FFS on whether conducted conformance test is feasible for AIOT devices. - FFS on OTA test method, performance metric, etc.

Conducted conformance test

The conducted test is connected to the test instrument by radio frequency cable, and the test is carried out without an antenna. With regard to device1 and device2a, the switching of impedance matching and mismatching is controlled by a digital baseband signal. In our understanding, conducted conformance testing is feasible for all A-IoT devices, we just need to consider the size and fixing of the antenna connector because of the device size is small.

Proposal 4: Conducted conformance test is feasible for all A-IoT devices, but testing cost needs to be considered.

OTA test

At present, there are two main methods to investigate the radiation performance: one is to judge the radiation performance of the antenna, which is a traditional antenna testing method, called passive testing. The other is to test the radiation power and receiving sensitivity in a specific microwave darkroom, which is called active testing. Passive testing cannot obtain radiation emission power and receiving sensitivity. Active testing is more accurate and comprehensive compared to passive testing. Passive testing is suitable for antenna products, active testing is suitable for the existing wireless terminal. In our understanding, active testing of OTA is too high cost for A-IoT device. Therefore, we need to simplify the testing condition and environment of OTA, shielding box for OTA test method could be considered.

Proposal 5: How to simplify existing OTA test method needs to be further studied.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: Some initial views about TX RF requirements for A-IoT devices in Table1.

Proposal 2: With the ultra-low complexity and cost, the test cost of Device1 should be reduced as much as possible.

Proposal 3: Some initial views about RX RF requirements for A-IoT devices in Table2.

Proposal 4: Conducted conformance test is feasible for all A-IoT devices, but testing cost needs to be considered.

Proposal 5: How to simplify existing OTA test method needs to be further studied.

Reference

- R4-2410597, WF  on impacts of A-IoT on RF requirements, Huawei, TSG RAN WG4 meeting #111