3GPP TSG-RAN WG4 Meeting#112                             R4-2412697

Maastricht , NL, 19th – 23th Aug, 2024

Agenda item:	8.20.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Co-existence evaluations

Document for:	Approval

Introduction

In the RAN#102 meeting, Study on solutions for Ambient IoT (Internet of Things) in NR has been approved and it’s expected to start the normative work from this April meeting. In the previous RAN4 meetings, we have reached lots of consensus on simulation assumptions for coexistence study, however there are still some issues needed to be further updated according to some initial intermediate calibration results.

Discussion

- 2.1. Discussion on coexistence cases and simulation assumptions

First of all, during last RAN4 meeting, it was agreed to have all A-IoT BS to be activated within single indoor factory. According to this simulation assumption, we provide some initial SINR curve for R2D reception as shown in Figure 2.1-1. It could be observed that around 86% tag’s received SINR lower than 10.3dB. However according to the BLER performance for OOK signal detection under different transmission bandwidth as shown in Figure 2.1-2, SINR@1% BLER is expected to be larger than 10dB if transmission bandwidth is restricted to 180kHz. Even for transmission bandwidth as 720kHz with OOK M=4~24, the required SINR @1% BLER would be also up to 10dB. In other words, roughly around 80% Tag/device cannot decode the R2D OOK signal correctly if all A-IoT BSs in the indoor factory scenario are activated simultaneously.

Figure 2.1-1. R2D SINR in D1T1 scenario

Figure 2.1-2. BLER performance for OOK detection at device/tag side

Proposal 1: further study the assumption of A-IoT BS activation within the indoor factory in D1T1 scenario.

In addition, it is agreed to assume R2D transmission bandwidth as 180kHz for the initial calibration, however according to RAN1’s discussions, chip rate for 180KHz transmission bandwidth might be two limited, instead 720kHz transmission bandwidth for R2D signal transmission might be appropriate option.

Proposal 2: for the transmission bandwidth of R2D signal, propose to use 720KHz transmission bandwidth for formal evaluation.

Conclusions

In this contribution, we shared some further views on coexistence evaluations and proposals are made as following:

Proposal 1: further study the assumption of A-IoT BS activation within the indoor factory in D1T1 scenario.

Proposal 2: for the transmission bandwidth of R2D signal, propose to use 720KHz transmission bandwidth for formal evaluation.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410567,WF on co-existence study for ambient IoT and NR/LTE, Approved.

- R4-2410567,WF on co-existence study for ambient IoT and NR/LTE, Approved.