3GPP TSG RAN WG4 #112	R4-2413236

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda item:		8.19.2.2

Source:	Nokia

Title:	On existing BS RF Tx requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#111 [1] regarding evolution of NR duplex operation. The following agreements were made that have impact on the existing BS RF requirements for SBFD operation:

  - Modification of existing requirements - TX

#### Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols

- Agreement:

  - PSD scaling for normal and SBFD slots/symbols:

    - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols

    - Vendors can declare different TX power values for normal and SBFD slots/symbols

#### Issue 2-2-1: TX intermodulation requirement

- Agreement:

  - The transmitter intermodulation requirement is applicable in SBFD slots/symbols:

    - Follow the existing requirement

    - No receiver requirement is specified.

#### Issue 2-3-1: The necessity of Co-location ACLR requirement

- Agreement:

  - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots

    - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement.

#### Issue 2-3-2: OBUE

- Agreement:

  - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

The Way Forward also describes the necessity for additional co-existence study for in-band blocking:

- Way forward:

  - The group will define the case/scenarios for additional co-existence, with the assumptions to be discussed in details.

  - Companies are encouraged to provide the evaluation results for in-band blocking in the next meeting:

    - The scenarios to be studied are Scenario 1, 3, 5, 6, 9, defined in TR 38.858

    - The case to be studied is Case 3 (TDD DL to SBFD UL subband).

    - FFS the power level which should be collected from the simulation results to derive in-band blocking levels:

      - Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement.

      - Other options are not precluded.

    - FFS the reference point for the power level which should be before or after RX beamforming.

      - Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB.

    - FFS different grid-shift values should be considered.

      - Option-1: 10%.

      - Option-2: Other percentage values are not precluded.

    - FFS how to use the power level probability distributions to derive the in-band blocking levels

      - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios.

        - FFS the value of X

        - FFS the impact of CLI handling scheme introduced in RAN1.

This contribution further elaborates Nokia’s views on the existing BS RF Tx requirements for SBFD operation at gNB.

# Impact on BS TX requirements

## Transmitter signal quality

In the technical report of the study item [2] it is agreed that all the existing requirement for frequency error, modulation quality (EVM) and time alignment error (TAE) shall also be applied to BS in SBFD symbols/slots. However, there was a note that RAN4 should further discuss the joint measurement for normal DL symbols/slots and SBFD DL symbols/slots during WI phase. We do not believe having a joint measurement for normal DL symbols/slots and SBFD DL symbols/slots is a beneficial way forward. In order to have a joint measurement, a new complex test model to support both normal DL and SBFD DL symbols/slot is needed.

New complex test model would be needed to support joint measurement of transmitter signal quality for normal DL symbols/slots and SBFD DL symbols/slots.

The signal quality might be worse after the switching to SBFD DL slot/symbol and the new requirement of transmitter transient period that is required between DL symbols/slots and SBFD DL symbols/slots plays a pivotal role here also.

- Separate transmitter signal quality measurements are preferred for normal DL symbols/slots and SBFD symbols/slots. As this issue is related to BS conformance testing, we proposed to take it into account during performance part of SBFD work item.

# Conclusion

In this contribution, our views on the SBFD BS RF Tx requirements are presented. There were observations and proposals as listed below.

- New complex test model would be needed to support joint measurement of transmitter signal quality for normal DL symbols/slots and SBFD DL symbols/slots.

# References

- R4-2409958 “Way Forward for [111][313] NR_duplex_evo”, RAN4#111, Fukuoka, Japan, 20th-24th May, 2024

- TR 38.858 V18.1.0 “Study on Evolution of NR Duplex Operation”

- TR 25.942 V18.0.0 “Radio Frequency (RF) system scenarios”