3GPP TSG-RAN WG4 #114	R4-2500054

Athens, Greece, February 17-21, 2025

Agenda item:	7.22.3.1		Enhancement for UE-initiated/Event-driven Beam Management

Source: 	Qualcomm Incorporated

Title: 	Further considerations for enhanced UE-initiated/event-driven beam management

Document for:	Discussion

# Introduction

In RAN4 #112 the work on Rel-19 MIMO Phase 5 was started. The following agreements were	made in RAN4 #113 for UE-initiated/Event-driven beam management and additional open issues have been identified [1].

Issue 1-1: How to define Event triggered measurement reporting delay?

< Agreement>

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a first PUCCH triggered by the event.

Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay?

< Agreement>

Agreement: For specifying the RAN4 delay requirement for event 2

- Measure both the current beam and new beam after the starting point, FFS the details.

- Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion.

- Start the RAN4 RRM discussion for FR2.

- Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS.

Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2?

Case 1: no time window based

Case 2: with time window based, M is configurable

< Agreement>

Agreement:

It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

In this paper we provide our views for the open issues identified in the way forward in [1] in RAN4 #113 and the LS received from RAN1 in [3].

# Discussion

In RAN4 #113 good progress was made on the definition of	even triggered measurement reporting delay. First it was agreed that the end point for the reporting delay is the point when the UE transmits a first PUCCH. This implies that for both mode A and mode B the same end point has been defined which is the transmission of a first PUCCH. The starting point of the measurement reporting delay is the time when the event is met over the air interface. The second main agreement was that both the current beam and the new beam are measured after the starting point.

Figure 2-1 illustrates what is meant by this agreement for the case when the RS resources for the current and the new beam have same periodicity. The circles illustrate the RS resources over time for both the current and the new beam. In the left figure event-2 is detected after a new measurement of the new beam is available and in the right figure event-2 is detected after a new measurement of the current beam is available.

Figure 2-1: Event-2 is triggered by the New Beam (left hand side) and the Current Beam (right hand side)

The agreement for Issue 1-2 says that both scenarios are considered for defining the measurement reporting delay. One of the discussion points in RAN4 #113 was how to define the measurement reporting delay when the periodicities of the RS resources for the current and the new beam are different. In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam that the evaluation periodicity equals the periodicity of the RS of the current and the new beam, i.e., both RS of the current and the new beam have the same periodicity as well. A LS by RAN1 to RAN4 will be received in RAN4 #114 confirming this agreement [3]. The case of different periodicities of the current beam RS and the new beam RS(s) is still under discussion in RAN1 but has not been agreed yet. Hence, RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

Observation 1: In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam that the evaluation periodicity equals the periodicity of the RS of the current and the new beam.

Observation 2: The case of different periodicities of the current beam RS and the new beam RS(s) is still under discussion in RAN1 but has not been agreed yet.

Proposal 1: RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

In the following we consider how the measurement reporting delay can be defined. Figure 2-2 illustrates the contributions to the measurement reporting delay. It can be seen from the figure that there are several contributions to the overall measurement reporting delay, i.e., Tgap, Tmeas and TPUCCH. In the following the individual contributions are addressed separately.

Figure 2-2: Illustration of Contributions to Measurement Reporting Delay

Figure 2-2 shows a contribution to the overall measurement reporting delay Tgap. Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS or SSB resource for measuring. Tgap represents a delay uncertainty and is bounded by the periodicity of the CSI-RS or SSB resources. It is a similar uncertainty as TEvent_DU in the conditional handover. In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted, see for example the test case definition for conditional handover in section A.6.3.3 in TS 38.133.

Observation 3: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring.

Observation 4: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted.

The next contribution to address is TPUCCH in Figure 2-2. Once the triggering event has been identified by the UE, the first PUCCH can be sent by the UE when the UL resources are available. After the event has been detected by the UE, there is waiting time, indicated as TPUCCH in the figure, till the UE can sent the first PUCCH. The situation is like the case of event-triggered periodic reporting of L3 measurements as described in sections 9.2.4.2 and 9.2.4.3 of TS 38.133. Here the measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. […] This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

Observation 5: For event-triggered periodic reporting of L3 measurements delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay.

The same principle should be reused for UE-initiated/event-triggered beam reporting. Requirements should be defined in such a way that the case of no uplink resources is excluded and the waiting time till UL resources are available is also excluded.

Proposal 2: Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

The third contribution to address in the measurement reporting delay is Tmeas as shown in Figure 2-2. Tmeas is the required time by the UE to identify the event that triggers a beam management report and is analyzed further in the next step. So far, the agreement in RAN1 is that L1-RSRP is supported as a measurement quantity. Defining L1-SINR as a second measurement is under discussion in RAN1 but has not yet been agreed. Requirements for L1-RSRP reporting based on SSB and CSI-RS measurement are already defined in TS 38.133, section 9.5 fulfilling the absolute and relative measurement accuracy requirements defined in TS 38.133, section 10.1.

It has been agreed in RAN4 #113 in Issue 1-2 that first focus to define RRM requirements is FR2. The measurement periods TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS are defined for FR2 as well. If these measurement periods are reused, no additional accuracy requirements are required for L1-RSRP. This implies in particular that the beam sweeping factor N = 8 should be reused.

Observation 6: If the measurement periods TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS and the existing beam sweeping factor N = 8 are reused, no additional accuracy requirements need to be defined.

Hence, we propose to reuse existing requirements as much as possible since it is not desirable to define new accuracy requirements. Also, the beam sweeping factor should not be further optimized in the MIMO Phase 5 work item.

Observation 7: It is desirable to reuse the existing L1-RSRP measurement accuracy requirements.

Proposal 3: RAN4 should reuse the measurement periods as already defined for L1-RSRP measurements for FR2 as much as possible, i.e., TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS. Also, the beam sweeping factor in the existing requirements for L1-RSRP measurements should be reused.

Table 2-1 shows the measurement period TL1-RSRP_Measurement_Period_SSB for FR2 as defined by TS 38.133 depending on the reporting periodicity TReport. This factor can be ignored since for UE-initiated / event-driven beam management the reporting is even driven. Hence, the factor that determines the measurement period is ceil(M*P*N)*TSSB for SSB and ceil(M*P*N)*TCSI-RS for CSI-RS, respectively.

Table 2-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

N = 8 is the beam sweeping factor for Rx beam refinement, M  {1, 3} depending on whether the higher layer parameter timeRestrictionForChannelMeasurement is configured (M = 1) or not (M = 3). P is a scaling factor dependent on the overlapping of RS resources with measurement gaps. Assuming for simplicity of further consideration that M = P = 1, it is seen that the measurement period allows to N = 8 RSs to determine the L1-RSRP measurement.

So far, RAN1 has agreed to support

- Event-2: Quality of at least one new beam becomes a threshold value better than the current beam

- Event-1: Quality of the current beam is worse than a certain threshold

- Event-7: Quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality

The existing measurement period allows to measure for a given beam a L1-RSRP measurement with sufficient measurement accuracy. Since in event-1 one the current beam needs to be measured, the existing measurement period can be directly reused. For event-2 and event-7 the situation is different, since now L1-RSRP for two beams need to be measured. According to the current agreement in RAN1 [3] both beams have the same periodicity but potentially different offsets. For defining the measurement periods, we consider two different cases when event-2 is triggered by the new beam (case 1) and when event-2 is triggered by the current beam (case 2). The scenarios are illustrated in Figure 2-3 and Figure 2-4. In both cases, event-2 is fulfilled OTA at a certain point in time. Both the new beam and the current beam are continuously measured and the L1-RSRP is updated. The measurement process can be viewed as a sliding window process. In Figure 2-3, N = 8 samples are taken to identify event-2. For case 1 the first sample of the current beam would still be before the point in time when event-2 is met. Figure 2-4 illustrates case 2 when the current beam triggers the detection of event-2. In this case, the first sample of the new beam would still be before the point in time when event-2 is met.

Figure 2-3: Measurement Periods for Event-2 triggered by New Beam (Case 1)

Figure 2-4: Measurement Periods for Event-2 triggered by Current Beam (Case 2)

However, since the measurement process is a sliding window both the measurement for the new and the old beam is reliable and based on N = 8 samples. Hence, the existing measurement periods for L1-RSRP can be reused also for event-2 and event-7. Hence, we propose that the existing measurement periods are reused for all events.

Observation 8: For event-2 and event-7 the existing measurement periods can be reused. It should be noted that one sample for the non-event-detecting beam would be used in the measurement process before the event happened OTA.

To get a stable identification of the event, full Rx beam refinement (N = 8) should be allowed for the event-detecting beam. All N samples of the event-detecting beam should be in time after the point when the event is met OTA.

Proposal 4: For the case M = P =1 and non-DRX, the measurement period for event-1 can be defined based on N*TSSB. For event-2 the same measurement period as for event-1 should be defined.

RAN4 should discuss further whether the existing scaling factors M and P can be reused. As mentioned above, since existing L1-RSRP measurement periods can be reused for UE-initiated/event-driven beam management, no additional measurement accuracy requirements need to be defined.

Proposal 5: No additional accuracy requirements are needed for L1-RSRP measurements in UE-initiated / event-triggered beam management.

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward >

- Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek)

  - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

- Option 2: (Samsung)

  - Focus on core requirements firstly, FFS on accuracy requirements.

- Option 3: (CMCC)

  - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

  - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Option 4: (ZTE)

  - Discuss the accuracy for the two trigger-event solutions respectively.

    - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

    - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.

Issue 1-7 is related to the question whether CSI-RS based L1-RSRP should be defined for inter-cell beam management.

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung, MediaTek)

  - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

- Option 2: (CMCC)

  - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

- Option 3: (Apple, Huawei)

  - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

- Option 4: (ZTE)

  - RAN4 needs to identify whether additional clarifications on the following points are necessary:

    - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

    - The prerequisite on L3 measurement on NSC.

    - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Option 5: (Ericsson)

  - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

  - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP. We propose not to introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Observation 9: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-based L1-RSRP.

Proposal 6: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Depending on whether the target TCI state is known or unknow, different TCI state switch delays are defined in TS 38.133. The condition of a known TCI state includes that the TCI switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and that the UE has sent a valid L1-RSRP report for the target TCI state. The existing delay requirement already covers the case that the target TCI state is known but not yet in the active TCI state list. This adds additional switch delay. If there is no L1-RSRP report before the TCI state switch command, the TCI state is unknown which adds additional delay in the TCI state switch. The existing conditions for known and unknown TCI states can be reused.

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei)

  - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

- Option 2: (vivo)

  - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

- Option 3: (ZTE)

  - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point.

Proposal 7: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management.

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Samsung)

  - No unknown case is applied if the TCI states switching is based on UEIBM.

- Option 3: (ZTE)

  - Need to decide whether to define L1-RSRP requirements for unknown cell

After the gNB receives the beam report and executes a TCI state switch to a target TCI state, this target TCI state can be known or unknown since there is no guarantee how quickly the gNB would trigger the switch after the report has been received. Also, the target TCI state does not need to be in the active TCI state list for PDSCH. In section 8.10.3, 8.10.4 and 8.10.5 of TS 38.133, the TCI state switch delays are defined for MAC CE based, DCI based and RRC based switching, taking all the conditions mentioned above into account. Based on current RAN1 agreements, we believe that these TCI state delay switches can also be reused for UE-initiated/even-triggered beam management.

Proposal 8: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

Finally, we address the question of the time window in Issue 1-3. It has been agreed in RAN4 #113 that the UE does not do any filtering of L1-RSRP measurements if a configurable time window is applied. If no filtering is applied, it is not obvious whether additional reporting delay requirements for the configurable time window needs to be defined. The measurement would simply extend by a factor based on the configured value M. Hence, we do not see that defining additional requirements tests much more functionality of the UE.

Observation 10: in case that no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 9: RAN4 should not define additional requirements for the window-based observation whether the event has occurred M times.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #113. Our observations and proposals are summarized as follows:

Observation 1: In RAN1 #119 it was agreed that at least the case is supported in Rel-19 that the evaluation periodicity for event-2 is identical for the current and the new beam that the evaluation periodicity equals the periodicity of the RS of the current and the new beam.

Observation 2: The case of different periodicities of the current beam RS and the new beam RS(s) is still under discussion in RAN1 but has not been agreed yet.

Proposal 1: RAN4 should focus on the case of same periodicities of the current beam RS and the new beam RS(s) to define measurement reporting delay requirements.

Observation 3: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring.

Observation 4: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted.

Observation 5: For event-triggered periodic reporting of L3 measurements delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay.

Proposal 2: Delay uncertainty till the first PUCCH can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting.

Observation 6: If the measurement periods TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS and the existing beam sweeping factor N = 8 are reused, no additional accuracy requirements need to be defined.

Observation 7: It is desirable to reuse the existing L1-RSRP measurement accuracy requirements.

Proposal 3: RAN4 should reuse the measurement periods as already defined for L1-RSRP measurements for FR2 as much as possible, i.e., TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS. Also, the beam sweeping factor in the existing requirements for L1-RSRP measurements should be reused.

Observation 8: For event-2 and event-7 the existing measurement periods can be reused. It should be noted that one sample for the non-event-detecting beam would be used in the measurement process before the event happened OTA.

Proposal 4: For the case M = P =1 and non-DRX, the measurement period for event-1 can be defined based on N*TSSB. For event-2 the same measurement period as for event-1 should be defined.

Proposal 5: No additional accuracy requirements are needed for L1-RSRP measurements in UE-initiated / event-triggered beam management.

Observation 9: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-based L1-RSRP.

Proposal 6: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

Proposal 7: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management.

Proposal 8: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

Observation 10: in case that no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 9: RAN4 should not define additional requirements for the window-based observation whether the event has occurred M times.

# References

- R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #113, November 2024

- R4-2418279, Topic summary for [113][221] NR_MIMO_Ph5_Part1, RAN4 #113, November 2024

- R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, RAN1 #119, November 2024