3GPP TSG-RAN WG4 Meeting #114	R4-2500112

Athens, GR, Feb. 2025

Source	:	Qualcomm, Nokia, Verizon, T-Mobile

Title	:	Redundancies in 2Tx/3Tx notes

Agenda item	:	7.1.1.1.3

Document for	:	Approval

# 1.	Introduction

The proliferation of higher power classes across various band combinations is an inevitable and necessary development for the industry. The UL CA combination specification framework in 38.101 was put in place before a lot of these developments came into view, and as such, it is not surprising that UL CA combinations are specified in a confusing manner with detail spread over both section 5 and section 6. In this contribution we propose a streamlining of the notes sprinkled across sections 5 and 6 of 38.101-1 pertaining to the Tx chain makeup of the UL band(s).

# 2. 	Discussion

### 2.1	Motivation in WI

In [1], the following was identified as direction for the WI:

Re-organize 2Tx/3Tx Notes in clause 5 and clause 6: Companies are encouraged to consider the necessity and possibility to re-organize the Notes in future meetings

### 2.2	Section 5 Tx connector count notes

In section 5 (5.5A.3.x), there are multiple notes pertaining to antenna connector count and CA power class. The example below is excerpted from the notes that apply to two-band CA combinations:

While they were introduced with an intent to provide more information, it is useful to revisit them in Rel-19 to evaluate utility and motivation. Consider the following observations:

- For example: NOTE 8 confirms 2 connectors are used when UL is in two bands and perhaps 1 or 2 connectors if the UL is limited to one band. This information is neither complete nor precise enough for compliance testing, due to lack of UE-implementation specific information.

- Network operators too are not particularly interested in the number of antennas used for transmission, their basic interest is TRP.

- UE vendors do not need this information either, because they are the source of this information in the first place.

- Connector count in section 5 is not needed to mark out MSD requirement availability in section 7 either, because MSD requirements are already differentiated based on Tx connector count and there is no ambiguity in which requirement would apply.

There is also overlap with section 6, which contains complete per-band Tx chain count detail where necessary.  Section 6 is a good home for that information because it also includes other tables notes, Tx-chain count specific MPR, etc. Example of per-band connector count information in section 6 is excerpted below:

Similarly, notes 9, 13 and 14 convey information on number of Tx antenna chains that are also beset by the problems identified above.

In conclusion, it is not clear whether connector count in section 5 is essential information, and who it is intended for.

Additionally, these kinds of ‘connector count’ notes invite many inconsistencies in the standard. For example, there is no mention of 4Tx in any band even for band combinations with UL in a single band. Another example of inconsistency is the omission of connector count for single band UL combinations (clause 5.5A.1 and 2). These omissions show it is difficult to keep section 5 notes abreast of changes elsewhere in the standard and relevant to the varied demands of diverse end-users.

Observation 1: Section 5 notes containing Tx antenna count information have limited (if any) utility due to overlap with section 6 and are a source of inconsistency.

It can be argued that section 5 notes are a way to strictly regulate UE implementation diversity. This kind of artificial limit may have been useful at one time, but late in 5G it merely impedes 3GPP from meeting evolving market needs in a timely fashion. Case in point: In a companion basket WI ‘HPUE_NR_CADC_SUL_R19’, a 4Tx 2-band UL CA combination has been approved at the plenary level [2]. Now, this type of combination (TDD+TDD) has minimal core requirement impact, primarily due to scheduling restrictions that render MSD specification unnecessary. The remaining aspects are unremarkable because the constituent per-band power classes are not new. Enabling this band combination in RAN4 should be routine. Unfortunately, with a business-as-usual model in RAN4, it is not a formulaic procedure to enable this combination: a whole new set of section 5 notes would first be required on top of the already long list. More notes only mean room for more inconsistency in the future.

Proposal 1: The per-band Tx connector count information is removed from section 5 table notes.

### 2.3	Power class information in Section 5

Note also that the section 5 notes referenced in the section above (8, 9, 13, 14) include power class information, specifically that PC2 or PC1.5 apply. Separately there is parallel description of the ULCA configuration including power class information in section 6 (6.2A.1.3). The associated table clearly lists the CA power classes of the approved combinations, making it ambiguous which set of allowed power classes should apply for an UL combination.

Observation 2: Notes pertaining to power class in sections 5 overlap with content in section 6 and cause ambiguity.

Historically, section 5 power class notes have been used to ‘meter’ introduction of new band combinations and power class aggregations, specifically to ensure MSD requirements are in place. During the work phase of Rel-19, a parallel ‘unified HPUE MSD’ effort is expected to overcome the HPUE MSD specification problem, so higher power classes will not need to be introduced strictly on a case-by-case basis anymore. See excerpt below for example, where n41 is only approved for PC2 in the band combination highlighted, but not PC1.5.

The argument can be summarized thusly: Contingent on success of the generalized HPUE MSD specification effort, the power class notes (restrictions) in section 5 can be removed.

Note that the information overlap problem does not apply to this note in section 5: ‘Only single uplink carriers with power class other than PC3 are listed’. Removal of this note will need further, more disruptive changes and is best pursued separately from this streamlining effort.

As is well acknowledged, a significant challenge for section 5 is the need to keep building on the notes as progressively higher power classes are implemented in section 6. Notes streamlining to capture all power class information in one place (section 6) will allow significant specification quality improvement.

Proposal 2: CA power class information is removed from section 5 table notes if a unified approach to HPUE MSD specification is agreed in Rel-19.

To accommodate corner cases, however, it would be better to retain a mechanism to accommodate band-combination specific exceptions in the future, even if the MSD specification issue is resolved. RAN4 can reserve section 5 notes for exceptions where the DLCA combination impacts the ability to implement a specific UL configuration in the HPUE, otherwise section 6 should remain the default location for technical notes.

Proposal 3: Section 5 notes are reserved for exceptions where the DLCA combination impacts the ability to implement a specific UL configuration in a certain power class, otherwise section 6 should remain the default location for technical notes

### 2.4	Implementation in standard

Note that the proposals 1and 2 are independent.

Proposal 1 is free-standing, and an accompanying draft CR is presented [3] to visualize how proposal 1 would be implemented.

A second draft CR [4] helps visualize the impact of both proposals together presuming the HPUE MSD specification issue is resolved.

# 3.	Conclusions

Observation 1	: Section 5 notes containing Tx antenna count information have limited (if any) utility due to overlap with section 6 and are a source of inconsistency.

Observation 2	: Notes pertaining to power class in sections 5 overlap with content in section 6 and cause ambiguity.

Proposal 1	: The per-band Tx connector count information is removed from section 5 table notes. (see dCR R4-2500113)

Proposal 2: CA power class information is removed from section 5 table notes if a unified approach to HPUE MSD specification is agreed in Rel-19. (see dCR R4-2500484)

Proposal 3: Section 5 notes are reserved for exceptions where the DLCA combination impacts the ability to implement a specific UL configuration in a certain power class, otherwise section 6 should remain the default location for technical notes

# 4.	References

[1] 	R4-2420329, ‘WF on HPUE RF requirements’, Samsung, RAN4#113, Nov. 2024

[2] 	RP-242865, ‘Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)’, China Telecom et al., RAN4106, Dec. 2024

[3] 	R4-2500113, ‘dCR to 38.101-1, to remove redundancies in 2Tx/3Tx notes’, Qualcomm, RAN4#114, Feb. 2025

[4] 	R4-2500484, ‘dCR to 38.101-1, to remove redundancies in 2Tx/3Tx notes’, Qualcomm, RAN4#114, Feb. 2025