3GPP TSG-RAN WG4 Meeting #114                                R4-2500224

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.1

Source:	Apple

Title:	Discussion on General Aspects on AI/ML for NR Air Interface

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on General Aspects on AI/ML for NR Air Interface.

## 2.1 RAN4 performance testing goals

As discussed in previous RAN4 meetings and captured in TR 38.843, the testing goals are described with the following options:

Requirements/tests for training will not be studied unless training procedures are defined. The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field.

For testing goals, Option 1 and/or Option 2 below will be selected depending on the test

-	Option 1: The testing goal is to verify whether a specific AI/ML model (if model identification is possible)/functionality can be conducted in a proper way.

-	FFS how to define the specific AI/ML model (e.g., a model captured in RAN4 spec as baseline)

-	FFS how to define that the model is properly conducted (e.g., by defining AI/ML dedicated performance/core requirements associated with model outputs)

-	Option 2: The testing goal is to verify whether the minimum performance gain of AI/ML model (if model identification is possible) /functionality/feature can be achieved for a static scenario/configuration.

-	FFS how to define a static scenario/configuration (e.g., by defining a related testing dataset based on channel models in TR 38.901)

-	FFS whether and how to define non-static specific scenarios/configurations

For the definition of AI/ML requirements, the following cases related to legacy performance should be considered

-	For the cases with the existing legacy performance

-	Take the legacy performance as baseline for existing use cases/procedures/functionalities /measurements that are to be enhanced by AI/ML based methods

-	Further study may be needed on what is baseline performance in conditions different to the requirement condition but within the expected range of operation.

-	New or enhanced performance requirements/tests could be considered for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods

-	For the cases without the existing legacy performance

-	New performance requirements/tests could be considered for the use cases/procedures/functionalities/measurements that are carried out or are to be enhanced by AI/ML based methods

According to our understanding, Option 1 primarily focuses on testing the DUT's capability to load and execute AI/ML models. Trained AI/ML models can be defined in RAN4 spec and are provided to DUT to implement the tests. RAN4 specifies baseline AI/ML models and performance requirements by using the model in the specification for each test case. If the DUT can choose the test AI/ML model and meet the performance requirements for each case, it can be considered as "conducted in a proper way".

A "model as baseline" may not necessarily need to be explicitly stated in RAN4 specifications. Instead, it can be communicated as a RAN4 agreement for companies' information before providing performance results for alignment.

Observation 1: "Model as a baseline" can either be explicitly captured in RAN4 specifications or agreed upon for aligning performance results.

Observation 2: For Option 1 and for verifying DUT’s AI/ML capability to load and execute the models, trained AI/ML models can be defined in RAN4 spec for different use case tests

Given that the AI/ML models are specified and trained within the RAN4 specifications, RAN4 can establish performance requirements for various use cases using these AI/ML models as benchmarks (reference models). The DUT is deemed successful if it meets the specified performance requirements during testing.

Observation 3: For Option 1 and for ensuring the model is properly conducted, performance requirements can be established for RAN4-defined AI/ML models across various use cases. The DUT is considered to have successfully passed the tests if it meets the specified performance requirements.

However, the UE vendor will implement its own AI/ML model which will be different that the baseline reference model. We suggest focusing on option 2 testing goals to meet performance requirements for the actual devices used in deployment.

Proposal 1: Deprioritize Option 1 and focus on Option 2 for performance requirements as a testing goal

Option 2 mainly focuses on verifying the performance gain of AI/ML models/functionalities. Moreover, the methods in handling of generalization test can be related to these functionalities/configurations:

TR 38.843 Clause 7.3.2.6 The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is As for the handling of generalization tests, the following option is considered as baseline: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

How to define the identified scenarios and/or configurations will depend on how to specify an AI/ML functionality/model by other WGs. There are two kinds of LCM procedures that have been studied according to the newly updated TR 38.843, one is the model-ID based LCM and another is the functionality-based LCM.

For functionality-based LCM, according to TR 38.843, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Therefore, the identified scenarios and/or configurations can be initially interpreted as the scenarios and/or configurations that UE report by capability signaling. However, there is currently no definition provided for scenarios, except one sentence in the TR as follows: Scenario/configuration specific (incl. site-specific configuration/channel conditions) Models

Our views on the two FFS in Option 2 are as follows:

For Option RAN4 can define multiple independent test cases with different scenarios and configurations. The scenarios and conditions can be related to UE capability signaling and may include the following contents:

-	Propagation conditions/scenarios, e.g., channel modes defined for different scenarios (CDL, AWGN, etc.) in TR38.901, Doppler conditions, SNR levels, various deployment scenarios (Uma, Umi, ISD), different cell with different gNB height, various indoor/outdoor UE distributions, various UE mobility etc.

-	Configurations:

- Various UE parameters: number of UE beams, antenna panels, antenna array dimensions, different AI/ML configurations: different set B of beams, T1 for measurement/T2 for prediction for BM-case2

- Various gNB settings: Tx codebooks (set A/B beams), beam widths, antenna spacings, Tx antenna dimension

Proposal 2: For verifying performance gain of AI/ML models/functionalities, RAN4 can define multiple independent test cases with different scenarios and configurations/scenarios as reported through UE capability signaling, which could include:

Propagation conditions/scenarios, e.g., channel modes defined for different scenarios (CDL, AWGN, etc.) in TR38.901, Doppler conditions, SNR levels, various deployment scenarios (Uma, Umi, ISD), different cell with different gNB height, various indoor/outdoor UE distributions, various UE mobility etc.

-	Configurations:

- Various UE parameters: number of UE beams, antenna panels, antenna array dimensions, different AI/ML configurations: different set B of beams, T1 for measurement/T2 for prediction for BM-case2

- Various gNB settings: Tx codebooks (set A/B beams), beam widths, antenna spacings, Tx antenna dimension, antenna port layouts, TXRU antenna virtualization (e.g. (8,8,2,1,1,2,8), vs (8,4,2,1,1,2,4))

RAN4 should conduct a down-selection process to narrow down the scenarios and configurations (parameters) for the conformance test. This test will serve as a benchmark for evaluating the model's performance across a range of unseen scenarios and configurations.

2.2 Static/non-static scenarios/configurations

Discussions surrounding the utilization of static and non-static scenarios or configurations in AI/ML use cases have taken place within the framework of testing goals during the SI phase. Although there is consensus on employing testing for a static scenario or configuration as a baseline approach, there remains some ambiguity due to the absence of an explicit definition for static or non-static scenarios or configurations. For instance, a channel experiencing multipath fading and Doppler effects exhibits time-varying channel realizations, rendering it technically a non-static channel.

During RAN4 #110bis the following agreement was reached:

Issue 1-3: Testing environment/framework

Agreement:

- Both static and non-static scenarios/configurations could be needed for AI testing

  - RAN4 will further discuss how to use them case by case

    - FFS whether to use static scenarios/configurations as baseline.

  - Refine the definitions of static and non-static scenarios/configurations based on two bullets below

    - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic

    - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

Regarding the definitions of static and non-static scenarios and configurations, we provide our input below.

When considering whether and how to define non-static specific scenarios/configurations, it's crucial to recognize the frequent changes in wireless environments during real deployments. Therefore, it's important to identify the use cases where it would be beneficial to define test cases incorporating non-static scenarios/configurations to verify DUT performance as realistically as possible. This may involve considering gradually changing scenario-related parameters and configurations could be considered.

Various aspects of what could constitute a non-static scenarios/configurations are listed below:

- Varying Channel models parameters: Large scale parameters of the channel model type (Uma,Umi, Indoor) including delay spread, doppler spread, AoA and AoD spread, etc.

- Varying SNR conditions: In legacy system, variation of SNR settings may be needed to meet specific test scenarios for CRI reporting and some RRM test cases. With respect to AI/ML features testing the use of non-static SNR may be potentially considered depending on the use case.

- Varying Tx configurations: Other parameters, such as Tx configurations, are typically assumed to be pre-defined and remain unchanged over the tests. Varying Tx configurations may include: set A/set B Tx beams, SSB (wide beams) versus CSI-RS (fine beams), different codebook designs (representing different codebook designs from infra-vendors)

The usage of static and non-static scenarios or configurations in AI/ML applications can differ based on the specific use case. Static scenarios provide a stable and controlled testing environment for deploying AI/ML models, ensuring dependable and consistent outcomes. However, certain AI/ML functionalities may necessitate the utilization of non-static scenarios and propagation conditions.

For instance, CSI and beam management temporal prediction features entail forecasting future states using historical measurements. Therefore, they may necessitate a more dynamic (non-static) environment that evolves over time to accurately simulate real-world conditions. Moreover, for BM use case spatial prediction we need to introduce enough randomness in terms of AoAs and AoDs to capture the spatial mapping from set B to set A. If the testing setup doesn’t support multiple probes for AoAs, a UE dynamic rotation will be needed.

Therefore, in some cases, non-static propagation conditions, including varying channel models, SNR, and trajectory movement modeling that simulate the movement patterns of users, may be required.

In summary, while static scenarios are generally suitable for most AI/ML use cases, the CSI and beam management temporal prediction use cases may necessitate non-static scenarios and may warrant further investigation. We suggest considering non-static conditions only if static conditions cannot fulfill the test objectives.

Examples of static versus non static channels are shown in Table 1.

Table 1. Examples of Static versus Non-static Conditions

Proposal 3: If non-static scenarios/configurations are supported for certain use cases, they should be included as part of generalization tests.

Proposal 4: Non-static scenarios/configurations should be considered for test cases when static scenario/configuration testing fails to fulfill the testing objectives. CSI and beam management temporal prediction use cases are particularly suitable for introducing non-static environments during testing. Feasibility of supporting more complex non static scenarios should also be studied.

## 2.3 Generalization/scalability aspects

The performance of generalization is of importance for a successful AI/ML feature deployment and the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML has been studied. The content below from the TR discusses achieving minimum performance under certain conditions while ensuring that performance is not significantly degraded under other conditions Accordingly for the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

For achieving a better understanding it is beneficial to distinguish between three specific sets of scenarios/conditions:

- All possible Deployment Scenarios/Conditions: These include the entirety of scenarios in which the AI functionality is intended for deployment. These conditions encapsulate the comprehensive real-world scenarios in which the AI system will operate, with the aim of meeting the specified RAN4 requirements within this context.  The requirements could in general be met with either AI/ML or legacy procedures.

- RAN4 Specified Test Conditions: These conditions encompass the specific scenarios for which RAN4 tests will be defined, ensuring that the requirements for these tests are met. These conditions should include the identified scenario as well as the other scenarios for generalization tests.

- Non-Tested Conditions that are part of possible Deployment scenarios/conditions: This category includes additional conditions that are part of the possible deployment conditions but are not explicitly addressed during RAN4 testing. These additional conditions could be met with either AI/ML or legacy operation through LCM procedures

As an example of the diverse set of possible deployment scenarios/conditions that could influence the generalization of CSI compression as outlined in TR 38.843, we provide the list below:

- Deployment Scenarios: Various deployment scenarios, which likely include different network architectures, environments, and usage patterns, are considered to understand how CSI compression performs across diverse deployment settings.

- UE Distributions: The distribution patterns of UE across the network, including factors like density and mobility, are analyzed to gauge the impact on CSI compression under different load conditions/distributions.

- Carrier Frequency: The frequency bands used for communication, including considerations such as sub-6 GHz or mmWave, are taken into consideration to assess how CSI compression behaves across different frequency ranges.

- TxRU Mappings: The mappings of Transmission Resource Units (TxRU) within the network infrastructure are examined to understand how CSI compression adapts to different antenna port layouts.

Additionally, specific factors are considered for scalability evaluations related to CSI compression, including:

- CSI Payload Sizes: The sizes of Channel State Information (CSI) payloads, are analyzed to determine how CSI compression scales with varying payload sizes.

- Tx Port Numbers: The number of transmission ports utilized in the communication process is evaluated to understand the impact on scalability when different numbers of ports are employed.

These considerations provide valuable insights into the performance and scalability of CSI compression techniques in diverse network environments and usage scenarios.

For beam management, the performance is impacted by deployment scenarios, UE mobility, set A, set B patterns, set A/B association mapping, antenna spacings, carrier frequency etc. Similar situation for positioning.

The primary objective when defining the test scope is to ensure reasonable confidence in achieving performance in the other applicable non-tested conditions. One possible approach might be to incorporate the full range of deployment conditions as test scenarios. However, this approach is often impractical for the majority, if not all, use cases. The complexity of requirements, test definitions within the 3GPP framework, and the constraints of time and resources needed for testing make such a broad approach infeasible.

The challenge AI faces lies in the current lack of understanding regarding how model performance may vary across different conditions. Setting requirements and defining a test scope necessitates an examination of performance variations across diverse conditions and scenarios.

Proposal 5: To determine the granularity of additional scenarios/conditions for defining generalization tests for each use case, it's essential to study how the AI model's behavior changes with different scenarios and conditions.

Proposal 6: To improve the generalization behavior of the model, training with a diverse dataset should be investigated.

For example, considering a TXRU antenna implementation variation involving the antenna element spacing and layout specifics (like 4x8 or 8x4), we could outline several generalization tests as follows:

Test 1:   Port layout 4 x 8, antenna spacing = 0.6

- Port layout 4 x 8, antenna spacing =0.8

- Port layout 4 x 8, antenna spacing =0.9

- Port layout 8 x 4, antenna spacing =0.7

- Etc

- We could significantly reduce the number of generalization tests by training with a mixed dataset containing samples from different configurations, such as configuration 1 and configuration 2,etc Subsequently, we can conduct a single generalization test where the testing data consist of a random mixture of these configurations. If the performance degradation between the model trained on the mixed dataset and tested with random mixture configurations and the model trained and tested specifically for each configuration meets a predefined performance margin criterion, the test could pass.

Proposal 7: Investigate the reduction of generalization tests by training with a mixed dataset containing samples from different configurations. Investigate the definition of a single generalization test where the testing data consist of a random mixture of these configurations. If the performance degradation between the model trained on the mixed dataset and tested with random mixture configurations and the model trained and tested specifically for each configuration meets a predefined performance margin criterion, the test could pass.

We propose to split the generalization/scalability framework into 2 parts:

- Generalization testing for other scenarios/configurations (generalization for conformance testing)

- Post deployment generalization

Fig. 1: Generalization testing and post deployment generalization framework

The split of the generalization framework and the associated tools or procedures we could employ to guarantee performance across all the applicable scenarios is shown in Fig 1. Ensuring performance across all other possible but non-tested deployment scenarios/conditions could prove infeasible given reasonable test complexity. In such cases, we propose that the post-deployment procedures described in Section 2.7 can be utilized.

- Proposal 8: Study post-deployment procedures to augment conformance testing for effectively managing performance across all possible deployment conditions/scenarios (which are not tested)

How to define the identified scenarios and/or configurations will depend on how to specify an AI/ML functionality/model by other WGs. We notice that there are two kinds of LCM procedures that have been studied according to the newly updated TR 38.843, one is the model-ID based LCM and another is the functionality-based LCM.

For the discussion on “details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality”, it is closely related to the RAN1 agreement for AI/ML functionality and AI/ML model, which are proposed for AI/ML functionality/model identification respectively, as provided in the following content from agreed RAN1 part of TR 38.843:

<RAN1 Agreement, captured from TR38.843 v1.3.0> For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. ... For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

Based on the paragraph from TR, a UE will report its supported configurations as part of capability signaling for a certain model or functionality. The other scenarios and/or configurations for generalization can be decided based on the supported configuration reported by UE.

Observation 4: The identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 5: Different scenarios that will be part of generalization test could act as the additional conditions for the AI/ML model training but do not constitute a part of UE capability for the AI/ML-enabled feature/FG

Observation 6: Configurations utilized for generalization test should be associated with UE capability of an AI/ML-enabled Feature/FG (set the configuration and vary the conditions under the configuration). Different conditions should be applied for each configuration.

Proposal 9: For UE-side models and/or UE-part of two-sided models it is suggested that the scenarios and configurations utilized for generalization tests can be determined based on the supported configuration reported by the UE as part of capability signaling.

The following paragraph from the TR defines the term of additional conditions:

For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions.

Note: whether specification impact is needed is a separate discussion.

The concept of additional conditions was introduced during the latter stages of the study item when the necessity for these conditions became apparent due to the presence of AI/ML training variables that may lack standardization. For example, in AI/ML-based beam management use cases, elements of network topology such as cell layout or antenna configuration, or internal variables of user equipment (UE) such as UE speed, can influence the selection of the AI/ML model to be utilized during AI/ML operations.

We provide an example to illustrate our understanding on additional conditions: Assuming a scenario where a NW employs a configuration with a set B of beams and each beam has a beamwidth Tx of 30 degrees. Thus, ideally a model would be trained with training data from those set of beams with the exact beam shapes (Additional conditions). This additional condition is not part of UE capability. However, it is possible that UE had a model ID stored in its device that is identified with these exact additional conditions. (through model transfer, or based on a training ID associated with the additional conditions).

A model associated with configurations based on conditions associated with UE capability and additional conditions is shown in Fig 4

Fig. 2: Model associated with configurations based on conditions associated with UE capability and additional conditions

Our view on "details about the scenarios and/or configurations for testing and the corresponding AI/ML models/functionality" is shown in Fig 2 for the use case of beam prediction for example. For the AI/ML functionality the UE capability report involves grouping of AI/ML models into buckets corresponding to non-overlapping sets of supporting conditions/configurations (conditions associated with Model A, Model B, etc). These conditions are associated with the UE capability. For each of these models there are sub-models that can be associated with additional conditions.

There are two types of additional conditions. The first type is the additional conditions that can be specified (and they can be signaled as assistance information to the UE to select the proper model). These NW side additional conditions should be aligned between training and inference for the UE sided model as well as during test procedures. According to this assistance information, UE can select the most suitable model to improve performance and pass the test.

For defining generalization tests, their configurations shall be associated with the UE capability of an AI/ML-enabled Feature FG. For example, we could define a generalization test associated with model B with a particular functionality (conditions/configurations) as part of UEs capability as it is shown in Fig 3. Model B is associated with some specific conditions/configurations, and under those conditions there are sub-models associated with some additional conditions as part of model identification. The various additional conditions can serve the purpose of defining generalization tests. Some of the additional conditions can be shared as assistance information, while other additional conditions that can’t be shared due to proprietary concerns or they can’t be standardized.

Fig. 3: Scenarios and/or configurations for testing and the corresponding AI/ML models/functionality

From RAN 4 perspective and concerning the testing implications, we must ensure the consistency between training and inference in the testing procedures. The following paragraph in TR describes the issue:

TS 38.843 UE capabilities for the functionality definition and additional conditions.

For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion. For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model-identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: The possibility that different approaches can achieve the same function is not denied

For example, in the BM use case, the association of Set B beams and Set A beams throughout training and inference should be guaranteed in the RAN4 core requirements. This consistency serves as additional assistance information for inference at the UE from the network side. For example, in BM use case, simulation results have confirmed that the consistency / association of Set B beams and Set A beams across training and inference is “beneficial” from performance perspective from as it is captured in the TR:

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective. Note: Whether specification impact is needed is a separate discussion.

Proposal 10: In the RAN4 core requirement, it is mandated that the consistency or association between of additional conditions during both training and inference is guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

TR does not address whether or how to define UE supported site-specific configuration or channel conditions in UE capability. Although it's possible that future releases may specify reporting of UE supported site-specific configuration/channel conditions via UE capability signaling, handling a scenario where a wide range of diverse UE capabilities are involved in defining requirements could be a challenge for RAN4.

The granularity of the scenario and configuration is also not clear. For example, as it shown in Fig 5, UE-1 may report that it supports models A,B,C with different condition/configurations. Each of the models A,B,C can be associated with additional conditions for generalization tests. For examples, these models can be identified as A{1,3,4}, B{2,4,6}, C{3,7} with the additional conditions inside the brackets. While UE-2 may report that it supports models D,F with the corresponding additional conditions D{3,5,7}, F{6}. Therefore, we suggest discussing how to specify the identified scenarios and/or configurations per use case, if other WGs can specify the granularity and the capability signaling.

Observation 7: The existence of a wide range of diverse UE capabilities poses a challenge for RAN4 in identifying a typical configuration or scenario for specifying test cases.

Proposal 11: RAN4 will explore methods to specify the identified scenarios and/or configurations per use case in future release, contingent upon other WGs can specify the granularity and the capability signaling.

Proposal 12: For defining generalization tests, RAN4 should define identified scenarios associated with the UE capability report of an AI/ML-enabled Feature FG. RAN4 should also define minimum level of performance for the identified scenarios and/or conditions.

Proposal 13: Other scenarios and/or configurations can be interpreted as the scenarios and/or configurations that are not reported by UE capability signaling for an AI/ML-specific functionality or the set of additional conditions.

2.3.1 Scenario/configuration specific Models (Fine-tuning)

Scenario/configuration specific (including site-specific configuration/channel conditions) models may provide performance benefits in some studied use cases (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites).

-	At least, when UE has limitation to store all related models, model delivery/transfer, if feasible, to UE may be beneficial, at the cost of overhead/latency associated with model delivery/transfer.

-	Note: On-device Finetuning/retraining, if feasible, of a single model may be an alternative to model delivery/transfer.

-	Note: a single model may generalize well in some studied use cases.

-	Note: Model transfer/delivery to UE may also face challenges, e.g., proprietary issues /burdens in some scenarios

Various approaches for achieving good performance across different scenarios/configurations/sites are studied, including

-	Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

-	Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

-	Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.

-	Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Observation 8: On device fine-tuning based on retraining could be beneficial to model delivery/transfer to reduce overhead/latency

If a UE has a model that partially meets the conditions/additional conditions as signalled by the NW, it can initiate a data collection procedure for training and on device fine-tuning. During fine-tuning it can perform performance monitoring and the model can be deployed after fine-tuning and performance monitoring completion. Subsequently, the UE can update its lists of models with the updated and finetuned model. After fine-tuning UE can update its stored AI/ML models with the new model ID, where this ID can be associated with the training data (which implicitly have the additional conditions) used to fine tune the model.

Proposal 14: RAN4 to discuss the practicality of formulating a framework that facilitates on-device fine-tuning. The focus will be on exploring the feasibility of creating a dynamic and site-specific approach to online training and fine-tuning (e.g reinforcement learning)

Proposal 15: For the purpose of limiting the amount of data needed for fine-tuning and reducing the number of fine-tuning iterations investigate transfer learning and meta-learning for efficient re-learning across different scenarios to aid generalization

Proposal 16: UE to update its stored AI/ML models with the new model ID after fine-tuning, where this ID can be associated with the training data (which implicitly have the additional conditions) used to fine tune the model.

Observation 9: Having a separate AI/ML model for each unique scenario, configuration, and additional condition could significantly increase the complexity and storage requirements of the User Equipment (UE). Additionally, it may introduce overhead in terms of model delivery, transfers, and associated latency

Regarding this sentence from TR “Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites ” we would like to introduce the concept of AI/ML super model:

Employing a smaller set of "super models" with enhanced generalization capabilities would be more advantageous. This approach would allow these models to handle a wider range of additional conditions. To enhance the generalization aspects of an AI/ML model, the following options could be considered:

- Train the AI/ML model with a diverse dataset that covers multiple Scenario/configurations

- Increase the number of AI/ML input signals with assistance/side information which will become integral part of its inference engine. The assistance/side information could indicate some information on the radio conditions (high/low SNR, LOS/NLOS conditions, delay spread, etc) or on the specific configuration (the physical angles of set B in BM, the configuration of the grid of beams in terms of wide/narrow beams, information of antenna spacing, antenna layout etc)

Currently, the support of additional conditions through assistance information only serves the purpose of selecting the appropriate AI/ML model. We propose introducing assistance information in the form of an auxiliary signal that participates in the inference propagation through the Neural Network. The concept of the AI/ML super model is shown in Fig 4. The AI/ML super model with assistance/side information could ideally alleviate the need of having multiple AI/ML models because it can self-generalize with the provision of side information as supplementary input signal. Of course, the generalization ability of the super model could not be assumed universal, so the objective would be to minimize the number of models required to address most of scenarios/configurations compared to the number of "simple" AI/ML models needed for that purpose.

Fig. 4: AI/ML Super Model with assistance/side signal information

- The notion of a "super-model" can be likened to a collection of models, where each model within the group possesses a shared structure but varies in terms of input/output configurations or utilizes different pre/post-processing techniques.

- Proposal 17: RAN4 should investigate the options for enhancing the generalizability of AI/ML models by providing the appropriate assistance/side information as input signal to the inference engine of the AI/ML model and discuss the feasibility of training with diverse datasets across different additional conditions

## 2.4 Principles on the definition of requirements

For the definition of AI/ML requirements, the following cases related to legacy performance should be considered

-	For the cases with the existing legacy performance

-	Take the legacy performance as baseline for existing use cases/procedures/functionalities /measurements that are to be enhanced by AI/ML based methods

-	Further study may be needed on what is baseline performance in conditions different to the requirement condition but within the expected range of operation.

-	New or enhanced performance requirements/tests could be considered for existing use cases/procedures/functionalities/measurements that are to be enhanced by AI/ML based methods

-	For the cases without the existing legacy performance

-	New performance requirements/tests could be considered for the use cases/procedures/functionalities/measurements that are carried out or are to be enhanced by AI/ML based methods

- For the legacy performance baseline we would need to clarify/agree that the side conditions of the testing procedures (e.g. ES Iot, Io range) should remain the same for legacy and AI/ML methods.

- Proposal 18: RAN4 should clarify/agree that the side conditions of the testing procedures should remain the same for legacy and AI/ML methods.

## 2.5 Requirements for LCM (Performance Monitoring)

As agreed in RAN4#107, RAN4 should study how/whether RAN4 core requirements could be defined for model monitoring in LCM. For the model monitoring (in which the performance of AI/ML model inference and/or the other environment conditions are under monitoring), it is similar to radio link monitoring (RLM, in which the downlink radio link quality on the RLM-RS resources).

In the CSI compression sub-use case, model monitoring can occur at the UE side. This approach allows for more accessible perception of CSI and other conditions, aiding the purpose of model monitoring. Metrics derived from this monitoring may include model inference accuracy, system performance, data distribution, and more. Additionally, it's feasible to derive monitoring metrics using the pairing of Encoder/Decoder, albeit with potential added complexity for the UE. These various model monitoring schemes are listed as possible performance monitoring methods outlined in TR38.843

Performance monitoring The following metrics/methods for AI/ML model monitoring in lifecycle management per use case are considered: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system performance KPIs - Other monitoring solutions, at least the following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE

Fig. 5: Illustration of model monitoring for CSI compression and BM sub-use cases:

The inference output becomes part of the input for model monitoring. Using this data, the monitoring operation evaluates KPIs (e.g., SGCS) between the monitoring data and the inference output to assess the model's performance. This is shown in Fig 3 for CSI compression and BM sub-use cases. This evaluation will influence subsequent processes like model retraining or transfer/delivery requests.

The computations of the model monitoring metrics in UE will result in additional complexity. Therefore, the frequency of performing monitoring metric computations should be kept low. Therefore, RAN4 should investigate the requirements about the frequency of invoking the monitoring procedure per use case. Moreover, the inputs of the model monitoring consist of model monitoring input and inference output may not be available at the same time. For example, as shown in Fig 5 for BM sub use case, the inference output (RSRPs of the best beam (pairs) is available at time t1. The top K beams (pairs) will need to be signalled from UE to the NW. Following this, the network transmits these specified beams to the UE, where measurements are conducted, without the involvement of AI/ML techniques. The RSRP of those measurements will serve as the model monitoring inputs and become available at time t2. For model monitoring metric computations there should be an investigation on the latency requirements regarding the availability of model monitoring input data (setting a bound on the timing difference t2-t1)

Model monitoring plays a pivotal role in ensuring the performance of AI/ML model inference. This is particularly crucial because the generalization aspect is commonly acknowledged as a significant challenge in AI/ML operations. Defining thresholds for abstracting system performance will be crucial for model monitoring to ensure consistent performance. If, for example, a threshold such as SGCS above 0.8 or a specific dB RSRP prediction accuracy (in BM) is indicative of better performance, introducing a tolerance margin to this criterion proves beneficial. This approach assists in the assessment of KPIs and aids in the accurate determination of model performance tailored to specific use cases. Moreover, the size of the measurement (number of samples) may need to be specified for obtaining a reliable monitoring result.

Proposal 19: RAN4 shall define RAN4 core requirement for performance monitoring tests based on RAN1/2 defined monitoring metrics/methods for particular (sub-)use case

Proposal 20: RAN4 shall consider the latency requirements for model monitoring input data as well as the establishment of tolerance margin requirements for the specified KPIs for performance monitoring per use case

In the scenario where the model monitoring functionality is located within the NW, the NW calculates monitoring metrics using RSRP values reported by numerous UEs. This setup can be considered as cell-level Beam Management (BM) model performance. The AI/ML model located at the NW may detect failures for a portion of UEs.

## 2.5.1. How to define requirements for Model monitoring

We are facing with the challenge of how to define requirements for Model Monitoring?  How to evaluate if the monitoring procedure is “Accurate”?

Current approach: Accuracy of the KPI Model monitoring metric: In R18 study, SGCS accuracy for model monitoring is computed by averaging the per-sample difference between the genie SGCS and calculated SGCS based on ground-truth reporting and SGCS estimation, i.e., ESGCSgenie-SGCSmeasured

However, this accuracy metric fails to reflect the performance of decision-making processes critical for monitoring. We would prefer to make use of a different approach: performance of decision-making processes that indicates the accuracy of predicting an event that indicates AI/ML model drift and performance degradation

Monitoring systems are not just about measuring accuracy; they are about identifying and reacting to anomalies, model degradation, or environmental changes.

Therefore, there is a need for Decision-Making Accuracy:

The primary goal of performance monitoring is to detect issues (like data drift, model degradation, hardware faults) and trigger appropriate actions (e.g., model re-training, re-configuration, AI/ML model switch or deactivation).

Thus, what's more important is the accuracy of monitoring decisions rather than just the numerical accuracy of SGCS measurements.

Performance monitoring in AI/ML-driven wireless networks is essential to detect issues such as data drift, model degradation, and hardware faults, and to trigger appropriate actions like model re-training, re-configuration, or deactivation. To achieve this, the system must ensure high event detection accuracy with low false positives and negatives, real-time monitoring, and multi-dimensional metrics. Proactive prediction capabilities, adaptive thresholds, and automated decision-making are critical to address issues before they impact performance. The implementation requires scalable and flexible solutions, including robust data collection, advanced detection algorithms, and predictive models. By meeting these requirements, the system will enhance reliability, maintain optimal performance, and ensure seamless adaptation to changing channel conditions.

Proposal 21: RAN4 to investigate a performance monitoring framework to detect issues such as data drift, model degradation, and AI/ML model aging, triggering actions like re-training, re-configuration, or model switching. The system will ensure high detection accuracy, real-time monitoring, and proactive predictions, enabling reliable adaptation to changing channel conditions while maintaining optimal performance.

## 2.6 Post Deployment validation and On Device Fine-tuning

The WF from RAN4# 110 captured the following issues regarding post deployment handling and on device training and fine tuning.

Issue 1-2: Post deployment handling

Agreement:

- To ensure the AI performance after device deployment, discuss the following options further

  - Option 1: Conduct the conformance testing for AI model/functionality before deployment

    - FFS on the feasibility

  - Option 2: Design the test to verify the performance monitoring

    - Depend on the other WG progress

    - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable

  - Other options are not precluded

Issue 1-5: On device training/fine-tuning

Agreement:

- Come back to this issue after the other WG finalizes the corresponding procedure.

In the late phase of Rel-18 study item, the post-deployment validation for model change/drift are proposed to be considered, and in corresponding contribution [R4-2319824], the following non-mutually exclusive options are proposed:

Issue: Post deployment testing

- Proposals

  - Option 1: RAN4 should study a framework to enable post deployment tests for model updates and/or drift validation(and possible other use cases)

    - Following options can be taken for reference in further discussion:

      - Option 1- a: The changes/updates to the ML-enabled Functionality/Feature are tested and declared by the device vendor against RAN4 requirements before any deployment to the UE is performed.

      - Option 1- b: After deployment to the UE and before changed/updated ML-enabled Functionality/Feature is activated in the UE, a post-deployment validation is performed, e.g., a sanity check test loop is run, e.g., using the functionality performance monitoring and LCM activation/deactivation/switching procedures,

      - Option 1- c: At least one fallback/default Functionality/Feature that passed conformance testing must always be present in the device.

      - Other options can also be discussed

  - Option 2: RAN4 does not need to study such framework

  - Option 3: others, please provide some proposals

- Recommended WF

In RAN4#112 bis the following agreements were reached on post deployment handling:

Issue 1-2: Post deployment testing

When operating in the field, two aspects of UE operation may impact performance:

- Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance

- Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive:

- Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs

  - Validation takes into account the UE hardware in which the model is to be deployed/activated.

  - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model)

  - FFS whether validation can be done at the device along with inference for another active functionality(ies)

  - FFS on the feasibility

  - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models.

  - Other aspects not precluded.

- Option 2: Using performance monitoring and LCM procedures

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable

FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device.

For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed.

  - Performance monitoring will be designed in other groups

  - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed.

  - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

UE vendor shall be responsible to guarantee the changes/updates to the ML-based functionality/feature (just like any legacy feature), therefore it is normal practice that enough in-house validation has already been conducted before any deployment to the UE is performed. However, the number of scenarios/configurations, conditions and additional conditions that could be encountered in practice could impose a large burden for the number of tests to de defined and it could complicate the testing procedures. Additionally, the additional conditions required by a particular scenario couldn’t be supported by the UE, therefore solution (Option 1-a) wouldn’t work.

The necessity for post-deployment management of AI/ML models stems mainly from the likelihood of frequent modifications or updates to these models. Essentially, these models function as software components capable of being replaced, enhanced, and executed on the same hardware within the device. With AI/ML models/functionality, those updates present a new challenge, i.e., how it can be ensured that a device that has passed conformance testing with one version of AI/ML model/functionality can also pass the same test after the upgrade?

A series of modifications to AI/ML functionality over the device's lifetime, post-deployment, can potentially lead to the following issues:

(1) Incorporating a newly updated AI/ML model onto the device without thorough validation can lead to inaccurate outcomes.

(2) Adjusting, updating, or fine-tuning an AI/ML model may result in diminished performance under specific conditions, despite potential enhancements in other scenarios.

Therefore, it becomes essential to invent a mechanism to ensure the adaptability and flexibility of AI/ML based functionalities for post-deployment validation/testing of AI/ML features

Observation 10: The post-deployment phase can be considered within the broader context of the generalization framework

Regarding Option 1-b: If LCM procedures indicate that the model performance has drifted, a question arises as to which model should be deployed next. If we rely on model monitoring, this will imply that a number of inactive models should be activated for monitoring procedures in order to select the next most appropriate model. This solution would impose a burden on signaling, management complexity, and overhead.

Observation 11: Options 1-a and 1-b for post deployment testing would impose a large burden for testing, signal overhead and complexity.

The UE capability report can be dynamic, and the UE will have to maintain a dynamic database of AI/ML models. New models can be updated and added/transferred to the UE.  NW can associate the training data used to train this model along with the NW-additional conditions assumed for the training and assign a label ID to this model trained with this dataset.

Proposal 22: If the AI/ML model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

If the UE doesn’t have the ID but the NW has the model ID, then the NW can transfer the model to UE and UE update its list of models:

Proposal 23: If the AI/ML model drifts due to misalignment of network-side additional conditions, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

If the UE has a model that partially supports the NW-sided additional conditions then the NW can trigger data collection for fine tuning of the UE-sided model:

Proposal 24: If the AI/ML model drifts due to misalignment of network-side additional conditions, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

If there is no model at the network or UE, then the network configures the UE to fallback to legacy mode.

In case there is no ID available, then NW can configure a model monitoring procedure with UE-side measurements to check if the consistency of the additional conditions can be achieved. If performance criteria are met the consistency is achieved and the model has been verified. Otherwise, fallback to legacy mode is configured.

Proposal 25: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 26: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

Given the above procedures of model updates, model transfers, model finetuning, a framework should be established to monitor/access the performance of the active and inactive AI/ML models.

We identify the following issues:

Issue (1): If during the model monitoring procedure the reported KPI’s fail the requirement, this would imply that the current deployed model has drifted due to changing conditions/propagation environment. How UE/NW selects the new model to be deployed?

Issue (2): How can we verify that the new model to be deployed will not cause further degradations? Or avoid improving the performance in some scenarios/conditions but degrade the performance in other conditions?

Issue (3): Current framework for monitoring procedure is a reactive framework. That is, we first identify the degradation and UE/NW acts upon this degradation

We would need to employ a model monitoring procedure which is proactive. Actions to improve performance (switch models) are taken before the model’s performance degrades. This approach would maintain the performance while not wasting resources for extra monitoring and latency associated with falling back in legacy

It would be hard to guarantee performance even if the updated model goes through some conformance testing at an OTA server since the model will not be tested with the actual field data and exact conditions.

Any verifications on updated models should be done with data collected on the field with the exact UE hardware and implementation. Some models that have been trained at NW or OTA server (under identical conditions) don’t reflect the exact UE HW implementations and therefore their verification is needed with the real UE hardware (RF architecture, etc)

Proposal 27: (Propose Option 3 for post deployment) For the post deployment framework, investigate an on-device or server-side model monitoring and performance assessment framework to track both active and inactive AI/ML models that experience actual UE hardware and field data (could be uploaded to the server). The aim is to dynamically manage the model database, enabling proactive monitoring for computing and updating key performance indicators (KPIs). This will facilitate seamless transitions to newly updated and fine-tuned models as needed.

Additionally, option 3 captures concepts from both Option 1 and Option 2:

- Validation takes into account the UE hardware in which the model is to be deployed/activated

- Validation takes place in the field (i.e., on each individual device) or in the server with data sent from the device (thus capturing UE HW and actual field data)

- Validation can be done at the device (inactive models) along with inference for another active functionality(ies) (active and inactive models)

- Using performance monitoring and LCM procedures

- Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

To minimize the number of samples required for fine-tuning/model update and reduce training costs, the framework will leverage Transfer Learning and Meta Learning techniques, ensuring efficient model management. The ultimate goal is to optimize performance, reduce overhead, and ensure models adapt seamlessly to changing conditions for post deployment and fine-tuning in a proactive fashion.

  - KPIs based on monitoring procedures are used to update the monitoring scores for all models at the UE side (active or inactive models stored). Model with currently best KPI (that meets requirements) is deployed

  - KPIs computed at UE can be signaled to the NW, KPIs are used to update the monitoring scores for each model (updates the database of active and inactive models). NW can decide if any of the models need update or fine tuning. It requests a small amount of data from the UE for fine tuning (data captures UE HW and field data). NW fine tunes the model and sends only the updated parameters along with the model ID. Based on KPIs the NW can indicate the model to be deployed or switched to at UE side or instruct the UE to fallback into legacy.

  - NW can predict performance degradation (in future) and signal UE to a) switch model or (b) to identify which model to fine-tune

For post-deployment we propose to signal the conditions for UE monitoring through RRC signaling.

- (1) Through RRC signaling, NW configures N candidate models to UE: (e.g. model A, model B, etc). During deployment, some of the models can be active or in active. Each of the model has a specific model ID

- (2) Through RRC signaling, NW configures monitoring conditions for each model to UE for monitoring all models (active and inactive monitoring)

  - Model specific conditions: (when these conditions are met, then monitoring procedures are triggered)

  - Measurements to determine if these conditions are met can be performed by UE or triggered by NW

    - Model A : {Condition A1, A2, etc} conditions to be configured

    - Model B: {Condition B1, B2, etc} conditions to be configured

Examples of model specific conditions signaled through RRC

- Absolute levels of RSRP/RSRQ level of serving cell (e.g if RSRP levels drop below a threshold trigger monitoring)

- Threshold of RSRP/RSRQ delta: if (measured RSRP - predicted RSRP ) > threshold then initiate monitoring procedures otherwise don’t trigger monitoring

- Timer based: e.g. timer expires after 5sec, only when timer expires, then trigger monitoring procedures for specific models (active or inactive)

- Periodicity based (e.g. T_period = 1280ms)  for active and inactive models

- Serving cell change: AI/ML model is site specific (new cell have different spatial characteristics, triggered by UE or NW)

- Channel characteristics change: (doppler change, scenario change: Uma vs Umi vs indoor channels, triggered by UE or NW, set B of beams are below threshold (beams are blocked) )

- Network triggered: (e.g. measurement configuration update)

The main concepts of the proposed framework for post-deployment are as follows:

- Introduce the concept of a database of AI/ML models and the dynamic management of the database for post deployment through model monitoring based on NW configured conditions. The database consists of AI/ML models which are configured for model monitoring based on the specific NW configurations (through RRC signaling). Based on the model monitoring procedures the KPI scores of the AI/ML model are dynamically adjusted to facilitate post deployment selection of best models. The states/scores of the various AI/ML models are updated to reflect their continuous evolution and adaptation.

- NW configures a list of AI/ML models through capability exchange (RRC signaling).

- NW configures a list of conditions for each model for model monitoring. UE through measurements, can determine if these conditions are met or NW could directly trigger the model monitoring for a particular model  (active or inactive)

- These procedures will facilitate seamless transitions to newly updated or introduced models according to the radio conditions ensuring efficient model management for ensuring system performance. Based on monitoring scores NW or UE can decide on which model to deploy next or switch to fall back legacy operation.

The following procedures will facilitate the management of the database of models:

- Through RRC signaling the NW configures N candidate models to UE: {Model ID/updated Fine-Tuned Model}

- Through RRC signaling, NW configures monitoring conditions to the UE for monitoring all models (active and inactive monitoring)

- UE performs periodic measurements to determine if the NW configured conditions are met to initiate monitoring procedures per model ID

- If conditions for monitoring are met, UE computes KPI for each model and updates the monitoring scores for each model (updates the database)

- UE signals the monitoring scores to NW and based on those KPIs, the NW can signal the new model ID to be deployed or updated/fine-tuned or signal the UE or switch back to legacy operation.  (through RRC signaling)

The post-deployment procedure through the dynamic management of an AI/ML model database and RRC signaling is shown in Fig 7.

Fig. 6: Post-deployment procedure through the dynamic management of an AI/ML model database and RRC signaling for enabling monitoring

Proposal 28: RAN4 to employ the post-deployment procedures described through the dynamic management of an AI/ML model database and RRC signaling for enabling monitoring procedures

## A flowchart of the generic post deployment procedure is shown below:

Fig. 7: Post-deployment procedure

## The procedure described so far is performing dynamic model switching based on updated performance KPIs (from model monitoring). It doesn’t capture the fine-tuning process of the model and their continuous adaptation and evolution. Moreover, the model monitoring dataset is only used for inference and model validation. For post deployment framework we would allow the models to be fine-tuned and use the model monitoring dataset for fine-tuning and re-training purposes.

## Proposal 29: In the post-deployment framework, models would be allowed to undergo fine-tuning, utilizing the model monitoring dataset for ongoing fine-tuning and re-training purposes.

## Which model to be fine-tuned can be signalled by the NW while the fine-tuning can take place at UE or NW.

Proposal 30: In the post-deployment framework, the network (NW) can indicate which model should be fine-tuned, with the fine-tuning process occurring either at the UE or within the network itself or an OTA server

Proposal 31: If fine-tuning takes place at the network (NW) or an over-the-air (OTA) server, the UE transmits a small sample dataset with high precision (e.g., PMI) over the air. The NW or OTA server (where inference takes place) then performs fine-tuning using transfer learning or meta learning techniques on the identified model to be fine-tuned, minimizing the number of parameters that need updating. It subsequently requests another dataset for validation to validate the model for post deployment. The updated set of parameters is then downloaded to the UE, with the option to quantize the coefficients to reduce overhead.

Proposal 32: If fine-tuning occurs at the UE, the UE is responsible for performing the fine-tuning using a small sample training dataset, with the network (NW) signaling which model to undergo updating. The UE then performs model monitoring to verify and validate the model post-deployment. The goal is to achieve optimal model performance as quickly as possible, minimizing the number of samples required during Transfer Learning to reduce training costs and achieve fast adaptation.

## 2.7 Data collection for testing

It is necessary to assemble test data in order to perform testing procedures. The following methods were discussed during the SI and collected in the TR:

Different generating methods of test dataset can be used for different tests. The following candidate methods are to be considered:

- Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc.

  - “Legacy approach” refers legacy test in which a channel model is used

- Field dataset (data collected directly from field measurements)

- TE generates dataset for test based on assumptions/parameters defined by RAN4 (e.g. by defining some rules/function to generate data)

- Other methods are not precluded

Utilizing a "legacy approach" rooted in an existing channel model is indeed highly preferable from a practical standpoint. Constructing a dataset of field measurements is likely to entail a significant campaign and necessitate widespread cooperation to accomplish. Therefore, considering the use of a legacy approach as a baseline is reasonable.

It would be possible that for some use-cases, models trained using real field data but evaluated using synthetic channel models may exhibit poor performance and could unfairly fail tests. Similarly, if the field dataset is not sufficiently representative, "real" field data could also lead to similar issues. Therefore, it will be important to assess model performance for each specific use case and ascertain whether the use of synthetic channel models poses any risk of generating "false fails."

Proposal 33: RAN4 must conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

From RAN4 #110 WF the following agreement was captured:

Issue 1-7: Test data handling

Agreement:

- For inference test, use synthetic channels as baseline, and check whether it can be used for the individual use case

# 3. Conclusion

In this contribution, we discussed the following observations and proposals for General Aspects on AI/ML for NR Air Interface:

Observation 1: "Model as a baseline" can either be explicitly captured in RAN4 specifications or agreed upon for aligning performance results.

Observation 2: For Option 1 and for verifying DUT’s AI/ML capability to load and execute the models, trained AI/ML models can be defined in RAN4 spec for different use case tests

Observation 3: For Option 1 and for ensuring the model is properly conducted, performance requirements can be established for RAN4-defined AI/ML models across various use cases. The DUT is considered to have successfully passed the tests if it meets the specified performance requirements.

Observation 4: The identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 5: Different scenarios that will be part of generalization test could act as the additional conditions for the AI/ML model training but do not constitute a part of UE capability for the AI/ML-enabled feature/FG

Observation 6: Configurations utilized for generalization test should be associated with UE capability of an AI/ML-enabled Feature/FG (set the configuration and vary the conditions under the configuration). Different conditions should be applied for each configuration.

Observation 7: The existence of a wide range of diverse UE capabilities poses a challenge for RAN4 in identifying a typical configuration or scenario for specifying test cases.

Observation 8: On device fine-tuning based on retraining could be beneficial to model delivery/transfer to reduce overhead/latency

Observation 9: Having a separate AI/ML model for each unique scenario, configuration, and additional condition could significantly increase the complexity and storage requirements of the User Equipment (UE). Additionally, it may introduce overhead in terms of model delivery, transfers, and associated latency

Observation 10: The post-deployment phase can be considered within the broader context of the generalization framework

Observation 11: Options 1-a and 1-b for post deployment testing would impose a large burden for testing, signal overhead and complexity.

RAN4 performance testing goals

Proposal 1: Deprioritize Option 1 and focus on Option 2 for performance requirements as a testing goal

Proposal 2: For verifying performance gain of AI/ML models/functionalities, RAN4 can define multiple independent test cases with different scenarios and configurations/conditions as reported through UE capability signaling, which could include:

Propagation conditions/scenarios, e.g., channel modes defined for different scenarios (CDL, AWGN, etc.) in TR38.901, Doppler conditions, SNR levels, various deployment scenarios (Uma, Umi, ISD), different cell with different gNB height, various indoor/outdoor UE distributions, various UE mobility etc.

-	Configurations:

- Various UE parameters: number of UE beams, antenna panels, antenna array dimensions, different AI/ML configurations: different set B of beams, T1 for measurement/T2 for prediction for BM-case2

- Various gNB settings: Tx codebooks (set A/B beams), beam widths, antenna spacings, Tx antenna dimension, antenna port layouts, TXRU antenna virtualization (e.g. (8,8,2,1,1,2,8), vs (8,4,2,1,1,2,4))

RAN4 should conduct a down-selection process to narrow down the scenarios and configurations (parameters) for the conformance test. This test will serve as a benchmark for evaluating the model's performance across a range of unseen scenarios and configurations.

Static/non-static scenarios/configurations

Proposal 3: If non-static scenarios/configurations are supported for certain use cases, they can be included as part of generalization tests.

Proposal 4: Non-static scenarios/configurations should be considered for test cases only if static scenario/configuration testing fails to fulfill the testing objectives. CSI and beam management temporal prediction use cases are particularly suitable for introducing non-static environments during testing

Generalization/scalability aspects

Proposal 5: To determine the granularity of additional scenarios/conditions for defining generalization tests for each use case, it's essential to study how the AI model's behavior changes with different scenarios and conditions.

Proposal 6: To improve the generalization behavior of the model, training with a diverse dataset should be investigated.

Proposal 7: Investigate the reduction of generalization tests by training with a mixed dataset containing samples from different configurations. Investigate the definition of a single generalization test where the testing data consist of a random mixture of these configurations. If the performance degradation between the model trained on the mixed dataset and tested with random mixture configurations and the model trained and tested specifically for each configuration meets a predefined performance margin criterion, the test could pass.

- Proposal 8: Study post-deployment procedures to augment conformance testing for effectively managing performance across all possible deployment conditions/scenarios (which are not tested)

Proposal 9: For UE-side models and/or UE-part of two-sided models it is suggested that the scenarios and configurations utilized for generalization tests can be determined based on the supported configuration reported by the UE as part of capability signaling.

Proposal 10: In the RAN4 core requirement, it is mandated that the consistency or association between of additional conditions during both training and inference is guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

Proposal 11: RAN4 will explore methods to specify the identified scenarios and/or configurations per use case in future release, contingent upon other WGs can specify the granularity and the capability signaling.

Proposal 12:  For defining generalization tests, RAN4 should define identified scenarios associated with the UE capability report of an AI/ML-enabled Feature FG, and other scenarios (additional conditions). RAN4 should also define minimum level of performance for the identified scenarios and/or conditions.

Proposal 13: Other scenarios and/or configurations can be interpreted as the scenarios and/or configurations that are not reported by UE capability signaling for an AI/ML-specific functionality or model ID.

Scenario/configuration specific Models (Fine-tuning)

Proposal 14: RAN4 to discuss the practicality of formulating a framework that facilitates on-device fine-tuning. The focus will be on exploring the feasibility of creating a dynamic and site-specific approach to online training and fine-tuning (e.g reinforcement learning)

Proposal 15: For the purpose of limiting the amount of data needed for fine-tuning and reducing the number of fine-tuning iterations investigate transfer learning and meta-learning for efficient re-learning across different scenarios to aid generalization

Proposal 16: UE to update its stored AI/ML models with the new model ID after fine-tuning, where this ID can be associated with the training data (which implicitly have the additional conditions) used to fine tune the model.

- Proposal 17: RAN4 should investigate the options for enhancing the generalizability of AI/ML models by providing the appropriate assistance/side information as input signal to the inference engine of the AI/ML model and discuss the feasibility of training with diverse datasets across different additional conditions

## Principles on the definition of requirements

- Proposal 18: RAN4 should clarify/agree that the side conditions of the testing procedures should remain the same for legacy and AI/ML methods.

## Requirements for LCM (Performance Monitoring)

Proposal 19: RAN4 shall define RAN4 core requirement for performance monitoring tests based on RAN1/2 defined monitoring metrics/methods for particular (sub-)use case

Proposal 20: RAN4 shall consider the latency requirements for model monitoring input data as well as the establishment of tolerance margin requirements for the specified KPIs for model monitoring per use case

Proposal 21: RAN4 to investigate a performance monitoring framework to detect issues such as data drift, model degradation, and AI/ML model aging, triggering actions like re-training, re-configuration, or model switching. The system will ensure high detection accuracy, real-time monitoring, and proactive predictions, enabling reliable adaptation to changing channel conditions while maintaining optimal performance.

## Post Deployment validation and On-Device Fine tuning

We are making a new proposal for post deployment (Option 3) combined with on-device fine tuning. Based on the principles below:

Proposal 22: If the AI/ML model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

Proposal 23: If the AI/ML model drifts due to misalignment of network-side additional conditions, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

Proposal 24: If the AI/ML model drifts due to misalignment of network-side additional conditions, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

Proposal 25: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 26: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

Proposal 27: (Proposed Option 3 for post deployment) For the post deployment framework, investigate an on-device or server-side model monitoring and performance assessment framework to track both active and inactive AI/ML models that experience actual UE hardware and field data (could be uploaded to the server). The aim is to dynamically manage the model database, enabling proactive monitoring for computing and updating key performance indicators (KPIs). This will facilitate seamless transitions to newly updated and fine-tuned models as needed.

Additionally, option 3 captures concepts from both Option 1 and Option 2:

- Validation takes into account the UE hardware in which the model is to be deployed/activated

- Validation takes place in the field (i.e., on each individual device) or in the server with data sent from the device (thus capturing UE HW and actual field data)

- Validation can be done at the device (inactive models) along with inference for another active functionality(ies) (active and inactive models)

- Using performance monitoring and LCM procedures

- Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

To minimize the number of samples required for fine-tuning/model update and reduce training costs, the framework will leverage Transfer Learning and Meta Learning techniques, ensuring efficient model management. The ultimate goal is to optimize performance, reduce overhead, and ensure models adapt seamlessly to changing conditions for post deployment and fine-tuning in a proactive fashion.

  - KPIs based on monitoring procedures are used to update the monitoring scores for all models at the UE side (active or inactive models stored). Model with currently best KPI (that meets requirements) is deployed

  - KPIs computed at UE can be signaled to the NW, KPIs are used to update the monitoring scores for each model (updates the database of active and inactive models). NW can decide if any of the models need update or fine tuning. It requests a small amount of data from the UE for fine tuning (data captures UE HW and field data). NW fine tunes the model and sends only the updated parameters along with the model ID. Based on KPIs the NW can indicate the model to be deployed or switched to at UE side or instruct the UE to fallback into legacy.

  - NW can predict performance degradation (in future) and signal UE to a) switch model or (b) to identify which model to fine-tune

Proposal 28 (Option 3) : RAN4 to employ the post-deployment procedures described through the dynamic management of an AI/ML model database and RRC signaling for enabling monitoring procedures

## Proposal 29 (Option 3): In the post-deployment framework, models would be allowed to undergo fine-tuning, utilizing the model monitoring dataset for ongoing fine-tuning and re-training purposes.

Proposal 30: In the post-deployment framework, the network (NW) can indicate which model should be fine-tuned, with the fine-tuning process occurring either at the UE or within the network itself or an OTA server

Proposal 31 (Option 3): If fine-tuning takes place at the network (NW) or an over-the-air (OTA) server, the UE transmits a small sample dataset with high precision (e.g., PMI) over the air. The NW or OTA server (where inference takes place) then performs fine-tuning using transfer learning or meta learning techniques on the identified model to be fine-tuned, minimizing the number of parameters that need updating. It subsequently requests another dataset for validation to validate the model for post deployment. The updated set of parameters is then downloaded to the UE, with the option to quantize the coefficients to reduce overhead.

Proposal 32 (Option 3): If fine-tuning occurs at the UE, the UE is responsible for performing the fine-tuning using a small sample training dataset, with the network (NW) signaling which model to undergo updating. The UE then performs model monitoring to verify and validate the model post-deployment. The goal is to achieve optimal model performance as quickly as possible, minimizing the number of samples required during Transfer Learning or Meta Learning to reduce training costs and achieve fast adaptation.

## 2.7 Data collection for testing

Proposal 33: RAN4 must conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

# 4. Reference

[1] RP-242399 – “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface “

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”