# 3GPP TSG-RAN WG4 Meeting #114	R4-2500236

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.17.2.2

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#113, and the open issues have been summarized in the topic summary for [113][218] NR_RRM_Ph5_Part1 [2].

In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

- Option 1 (CATT, Apple, HW, Ericsson): The enhanced CSSF apply to all the 3 aspects and do not define the priority. - Option 2 (Xiaomi, MTK, ZTE): the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Option 2a(ZTE): To be more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered. - Option 3 (Nokia, Samsung): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion. - Recommended WF - [Moderator]: As a compromise, is option 3 agreeable?

We also think RAN4 can prioritize the baseline scenarios for CSSF optimization study as starting point, i.e., SSB based Intra-frequency measurement without MG; and the other aspects can be further discussed after concluding on the baseline case. In last meeting, companies also commented that we can focus on the solution rather than discussing the prioritization for those aspects. We can compromise to option 1 to move forward.

Proposal 1: The enhanced CSSF apply to all the 3 aspects and do not define the priority.

Issue 2-4-1: Tentative solution 1: measure one serving CC per band

[Wayforwad]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

For the case that UE is not in multiple-Rx simultaneous reception mode but it’s in CA/DC mode, the CSSF enhancement can be considered if multiple serving carriers are in the same band. In the current spec, the UE measurement capability requirement is defined as following:

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

For intra-frequency layer measurement, RAN4 assumed UE is allowed to only measure one CC in a same band, as highlighted in red above, but for serving CC measurement we require UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band. In R15 discussion, the reason behind such requirement is because RSRQ might be different among different serving carriers (different interference level), however, FR2 is actually noise-limited system rather than interference-limited system, and therefore we don’t think RSRQ would be too much different among serving CCs in the same band. Thus, we can enhance to require UE only measure one serving CC in the same FR2 band like what RAN4 specified for intra-frequency neighbor cell measurement.

In previous meeting, companies mentioned that the candidate solution of optimizing CSSF (UE only needs to measure one serving CC per band if multiple serving CCs are in the same band) can be implemented by network configuration, e.g., Only one servingCellMO is configured on one SCC and no MOs are configured on the other SCCs, if multiple serving CCs are in the same band. We think it’s same situation as the discussion in R15 for neighbor cell measurements, i.e., we cannot limit the network implementation and configuration behavior, and therefore such enhancement to measure one serving cell in one band is based on the scenario when network configures multiple serving MOs in the same FR2 band to the UE. In last meeting, Infra companies had concern for UE to pick the SCC for measurement by random implementation, and we are also fine to further compromise to follow network indication on which SCC for measurement if following conditions are met:

- neither PCC nor PSCC is in the same FR2 band, and

- if there is no SCCs on which UE is configured to report SSB based measurements on same FR2 band, or

- if there is multiple SCCs on which UE is configured to report SSB based measurements on same FR2 band.

Proposal 2: We propose to consider the following solution as compromise for issue 2-4-1:

  - UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured,

    - Reuse the legacy principle for neighbour cell measurement

      - Measure PCC if PCC in band

      - Measure PSCC if PSCC in band

      - Measure SCC with neighbor cell MO, if

        - there is only one SCCs on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

      - follow network indication to measure a SCC, if

        - neither PCC nor PSCC is in band, and

        - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

        - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

  - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

[Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported

2nd possible enhancement is to reduce the searcher occupancy ratio of PCC and PSCC for some special conditions, and then the SCC measurement can be speeded up by increasing the searcher occupancy ratio. For example, in current spec, for NR SA (e.g., FR2 only CA), the CSSF table is specified as following,

It means that: PCC measurement utilizes a dedicated searcher, and all the SCCs share the 2nd searcher. If SCC has FR2 neighbor cell configured CCs, the SCCs with FR2 neighbor cell utilize the 50% of the 2nd searcher. And all the other SCCs and inter-freq CCs equally share the other 50% of the 2nd searcher. If SCC has no FR2 neighbor cell configured CCs, all the SCCs and inter-freq CCs without MG equally share the 2nd searcher.

It's obvious that, if the PCC searcher occupancy ratio can be reduced, e.g., to 50% of the 1st searcher, the other 50% of the 1st searcher can be shared by SCCs to speed up the measurement. However, PCC measurement is most important thing to the RRM, and we can discuss the conditions in which PCC measurement can reduce the searcher occupancy ratio. Such enhancements and condition can also be applied to PSCC in DC case.

The possible condition to reduce the occupancy of the searcher resource on PCC or PSCC could be similar as the RRM relaxation conditions in IDLE/Inactive, i.e., not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both. Alternatively, we can consider to use QoS to decide which CC shall be prioritized, i.e., allocate more measurement/searcher resource. For example, QoS could be traffic type specific, e.g., CCs with voice traffic, XR traffic may have higher QoS level than the other traffic; and the SCCs with such higher QoS level will be allocated with more searcher resource. Another consideration is: in a sTAG RAN4 defined the reference cell for DL timing tracking, we think the reference cell in TAG shall be prioritized from measurement perspective (allocate more searcher resource).

In last meeting, some companies had concern on the searcher resource sharing from PCC/PSCC to other SCC, however, we think if the solution is based on the network indication, then it shall be general enough for all kind of CCs. If PCC/PSCC is always most important measurement objective from network perspective, network may not indicate to share the resource from PCC/PSCC to SCC.

Proposal 3: We propose to consider the following solution for issue 2-4-2:

- UE can change the searcher occupancy ratio of PCC, PSCC and SCC with neighbor cell measurement to speed up other FR2 SCC measurement based on one of the following:

  - the network indication to do the searcher resource sharing, or

  - the mobility conditions, e.g., not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both.

  - the QoS of target CCs.

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF

Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios.

RAN plenary agreed in the WID that the baseline assumption on number of searchers is 2, and that’s also aligned with the RAN4 fundamental assumption from Rel-15. Some companies raised one issue that: in current spec, if UE supports per-FR MG, and if one FR measurement is using MG and the other FR measurement is not using MG, we don’t have a joint section of CSSF requirement to define a per-UE CSSF factor including both FRs, and that implies UE supports 3 searchers in this case. Unfortunately, we disagree with this assumption and interpretation, our view is: this is just a missing piece of current spec, and we don’t have specific test case to verify UE behavior in this case. In the field, the deployment of FR2 is very rear and the UE measurement speed cannot be easily verified in the current field log (at least no error observed in current industry). Additionally, searcher number change is a quite big challenge to UE implementation and design, and it will even impact the most essential part of the UE architecture. In Rel-15 discussion, it was agreed in R4-1807957 that,

Agreements - FR1 only operation - Option 1: 1 additional searcher equivalent (2 searchers)

Then in R4-1811399, it was agreed that,

Agreements:

|  | Kca for FR1 PCell/PSCell | Kca for FR1 SCells | FR1 overlap definition | Kca for FR2 PCell/PScell | Kca for FR2 SCC where UE measures up to 6 cells | Kca for other FR2 SCells | FR2 overlap definition |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 CA without NR-NR DC | 1 | Number of configured SCells |  | N/A |  | N/A | N/A |
| FR2 CA without NR-NR DC | N/A | N/A | N/A | 1 | N/A | 1 or number of configure SCell |  |

It's obvious that, 2 searcher is assumed in the current spec for both FR1 and FR2 case, and the interpretation of 2 searchers from the current spec is only because we don’t have a joint section to discuss MG based and MG-less based CSSF. Thus, we don’t think the current spec implies certain UE capability beyond 2 searcher assumption, and 2 searcher assumption shall still be kept for this R19 WI.

The searcher number increasing would be a big challenge to UE implementation and complexity, and it shall be carefully checked only if certain special condition is met. For instance, when UE is capable of per-FR MG and UE with FR1+FR2 CA/DC is configured to perform measurement on FR1 carrier and FR2 carrier, the more than 2 searchers might be considered only if FR1 measurement doesn’t need MG but FR2 measurement needs per-FR2 MG. Moreover, the limitation will be probably needed on that only single searcher can be assumed on FR2 carrier measurement while up to 2 searchers are assumed on FR1 carrier measurement if UE is in FR1+FR2 CA/DC. Another possible condition could be that UE can only consider these 3 searchers in high mobility status or at cell edge status(serving cell quality is below certain threshold) to speed up the measurement. But such discussion shall not be the prioritized one in this WI.

To move forward, we can compromise to consider 3 searchers for CSSF outside MG, RAN4 only considers UE is using 3 searchers for FR1 scenario only (FR1 NR CA or FR1 NR-DC). If companies cannot achieve consensus, RAN4 work can stick to the baseline assumption in WID, i.e., 2 searchers.

But if majority companies would like to go with 3 searchers for FR1+FR2 CA/DC, we would like to have optional UE capability to indicate if such UE can support 3 searchers for FR1 only CA/DC or for FR1+FR2 CA/DC, i.e., per band combination, or per FR combination. Alternatively, we may reuse similar approach from ‘need for gap’ feature, that is, UE can indicate the 3 searcher capability based on current serving cell and the configured MOs on SCC, and accordingly UE may need to update this capability when serving cell is changed or MO(measurement objects) configuration is changed.

Proposal 4: for 3 searchers in CSSF outside MG, RAN4 only considers UE is using 3 searchers for FR1 scenario only (FR1 NR CA or FR1 NR-DC).

- If companies cannot achieve consensus, RAN4 work can stick to the baseline assumption in WID, i.e., 2 searchers.

Proposal 5: for 3 searchers in CSSF outside MG, RAN4 to introduce an optional UE capability to indicate whether UE can support 3 searchers for FR1 or not.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: The enhanced CSSF apply to all the 3 aspects and do not define the priority.

Proposal 2: We propose to consider the following solution as compromise for issue 2-4-1:

  - UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured,

    - Reuse the legacy principle for neighbour cell measurement

      - Measure PCC if PCC in band

      - Measure PSCC if PSCC in band

      - Measure SCC with neighbor cell MO, if

        - there is only one SCCs on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

      - follow network indication to measure a SCC, if

        - neither PCC nor PSCC is in band, and

        - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

        - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

  - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Proposal 3: We propose to consider the following solution for issue 2-4-2:

- UE can change the searcher occupancy ratio of PCC, PSCC and SCC with neighbor cell measurement to speed up other FR2 SCC measurement based on one of the following:

  - the network indication to do the searcher resource sharing, or

  - the mobility conditions, e.g., not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both.

  - the QoS of target CCs.

Proposal 4: for 3 searchers in CSSF outside MG, RAN4 only considers UE is using 3 searchers for FR1 scenario only (FR1 NR CA or FR1 NR-DC).

- If companies cannot achieve consensus, RAN4 work can stick to the baseline assumption in WID, i.e., 2 searchers.

Proposal 5: for 3 searchers in CSSF outside MG, RAN4 to introduce an optional UE capability to indicate whether UE can support 3 searchers for FR1 or not.

# References

[1] R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN4#113

[2] R4-2418276, Topic summary for [113][218] NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#113

[3] R4-2420082, Ad-hoc minutes for NR_RRM_Ph5 WI, Apple, RAN4#113