3GPP TSG-RAN4 Meeting  #114	R4-2500244

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.6.3

Source:	Apple

Title:	On UE RF requirements for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues.

# 2	Discussion

Requirements for UE supporting fragmented carriers with shared RF chains

In the WF, the point on how to specify requirements for UE supporting fragmented carriers with shared RF chains is in brackets, as shown below

- [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements]

It is important to have a common understanding on it. As is well understood, the current RF requirements for intra-band non-contiguous CA were derived based on a partially shared RF chain architecture captured in TR 36.823. Given the blocker in the gap, it is evident that a shared RF chain architecture cannot meet such requirements. That is the reason why the SI scope explicitly contains the bullet “Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking.”

To be clear, whatever the outcome of the SI is, the existing requirements should still apply to UEs using partially shared RF chain architecture to support fragmented carriers (or intra-band non-contiguous CA). With that, we think it is reasonable to say the adjusted requirements for UE supporting fragmented carriers with shared RF chains should be separated from existing requirement. Of course, there should be a UE capability or other means to indicate such UEs.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Discussion of impacts on DL performance for UE enabling fragmented carrier mode

In current CA receiver sensitivity requirement, there is ΔRIBNC to allow relaxation due to TX noise at RX band, i.e., TX noise at SCC, which assumes UE architecture of separate RF chains, as shown in the specification excerpts below.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 | 5MHz + 5MHz | Wgap = 25.0 | 5 | 4.0 | FDD |
|  |  |  | Wgap = 5.0 | 20 | 0.0 |  |
|  |  | 10MHz + 5MHz | Wgap = 20.0 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 9) | 2.3 |  |
|  |  | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | Wgap = 5.0 | 20 (RBstart = 19) | 5.2 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |

With common RF chain architecture, the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, both the PCC and SCC performance may take some hit.

In the WF [2], companies are encouraged to bring analysis with their own assumptions following the table format. Our analysis is shown in Table 1 for CA configuration CA_n3(2A), 5MHz + 5MHz, Wgap = 65MHz.

Table 1. Link budget of Self-interference for CA_n3(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - |
| LPF output (dBm) | -36.4 | -102 | -97.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.5 | - |
| Noise floor (including ADC noise) | - | - | - | -90.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -11.5(PCC) |
|  |  |  |  |  | -7.5(SCC) |

We make the following notes compared to the analysis in [3]:

- The front-end assumptions are reused as they are common and reasonable.

- ADC noise is included as part of the overall noise in the receiver, and hence it is reflected in NF increase due to AGC adjustment. As a result, the NF increase is 10.5dB.

- We do not have two separate ADC setpoints. In real implementation, there is only one and is implementation dependent. The key is to reflect the ADC noise floor increase in the NF.

Table 2 shows our analysis for CA configuration CA_n25(2A), 5MHz + 5MHz, Wgap = 55MHz.

Table 2. Link budget of Self-interference for CA_n25(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -96.5 | -91.5 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -100.5 | -95.5 | - | - |
| LPF attenuation | 12.9 | 1 | 1 | - | - |
| LPF output (dBm) | -35.9 | -101.5 | -96.5 | - | - |
| Noise floor for basic REFSENS | - | - | - | -100.5 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 11 | - |
| Noise floor (including ADC noise) | - | - | - | -89.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -12(PCC) |
|  |  |  |  |  | -7.7(SCC) |

Observation 1: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), ΔRIBNC may need to be increased by about 11dB for PCC and 7dB for SCC.

On how to derive the final values of the required relaxation, there are several options:

- Each company uses its own assumption. RAN4 discusses a way to pick a value among the analyses submitted by all companies and capture it as part of the SI conclusion.

- Each company uses its own assumption. RAN4 captures the values proposed by each company as part of the SI conclusion. The final value is left to the WI phase.

Proposal 2: It is proposed to discuss how to proceed with the final values in RAN4 for SI conclusion.

Triggering condition to enable ”One RF Rx chain”

In the WF, the following is captured:

#### Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied - Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2

To study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2, we consider the following factors:

- The gap width: As shown in Table 7.3A.2.2.-1 from 38.101-1, the maximum gap width is 65MHz, which can certainly be supported by one RX RF chain.

- UE BB capability: In NR, UE is required to measure NR carrier RSSI and hence should be able to use the same hardware/software resources to measure the blocker power.

- Maximum blocker power: this is currently being discussed and we agree that the current maximum input level in 38.101-1 can be used as a starting point to avoid receiver saturation.

- The need of measurement gap for such measurement. This may depend on UE implementation and if needed, can be further discussed.

Observation 2: It is feasible to measure total aggregated power of all interferences in the gap between CC1 and CC2.

Image rejection requirement

Currently there is no requirement on image rejection for NR. On the need of a rejection level of the image caused by the in-gap interferer, our current view is if PSD of blocker is limited, the image rejection requirement can be indirectly ensured. Since any new requirement would incur testing overhead, it is not preferred to specify a new image rejection requirement.

Proposal 3: It is not preferred to specify a new image rejection requirement.

# 3	Conclusions

In this contribution, we make the following proposals and observations.

Proposal 1: It is proposed to agree that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements. A TP is needed to capture this agreement in the TR.

Observation 1: For the two CA configurations, CA_n3(2A) (5MHz + 5MHz, Wgap = 65MHz) and CA_n25(2A) (5MHz + 5MHz, Wgap = 55MHz), ΔRIBNC may need to be increased by about 11dB for PCC and 7dB for SCC.

Proposal 2: It is proposed to discuss how to proceed with the final values in RAN4 for SI conclusion.

Observation 2: It is feasible to measure total aggregated power of all interferences in the gap between CC1 and CC2.

Proposal 3: It is not preferred to specify a new image rejection requirement.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2420337, "WF on fragmented DL carriers study," MediaTek

R4-2418718, "On RF requirements of fragmented carriers," Huawei, HiSilicon