3GPP TSG-RAN WG4 Meeting #114	R4-2500305

Athens, Greece,  17th February – 21st February 2025

Agenda item:	6.12.2

Source:	Apple

Title:	On simultaneous RxTx for CA_n40A-n41A

WI/SI:	LTE_NR_R19_Simult_RxTx

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

A new work item [1] on simultaneous Rx/Tx has been agreed in RAN#104. One of its objectives is to clarify and improve existing mandatory simultaneous Rx/Tx requirements. This discussion paper discusses three main topics. The first topic is about the Rel-17 agreement on FR1+FR1 FDD-TDD combinations and the missing requirement in TS 38.101-1/3. The second topic is about the correct handling and request of fallbacks. The third topic is about CA_n40A-n41A and the requirements when supporting simultaneous Rx/Tx. The l

# 2  Discussion

#### 2.1 On second MSD test point for CA_n40A-n41A

The WF [2] from RAN4#113 captures the agreement to introduce a second test point for CA_n40A-n41A. The new test point features a wider channel on band n41 and reduced channel on n40. The frequency separation of the channels in bands n40 and n41 are larger which reduces the noise impact during simultaneous transmit and receive.

We conducted measurements for the new test point and assumed 10dB antenna isolatiion between the shared and diversity antenna.

Table 1 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 12.7 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 28.9 | ACLR2 |

Table 2 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 15.5 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 32.0 | ACLR2 |

Proposal 1: Use the MSD provided in Table 1 and Table 2 when specifying the second test point.

#### 2.1 On MIMO performance of CA_n40A-n41A

According to TS38.101-1 clause 7.2 a UE is required to feature a minimum of four Rx antenna ports for band n41. This requirement applies if n41 is configured as standalone band or if n41 is part of a band combination. Due to near proximity of n40 and n41, those bands are typically grouped together using the same antennas to reduce implementation complexity and number of required antennas.

Due to small form factor of handhelds the devices are likely to share antennas between Tx and Rx. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe de-sense as well as the diversity antennas. However, there is an imbalance of noise between shared and diversity antennas. The shared antenna features higher noise impact compared to the diversity antennas. In case of MIMO reception, the larger noise of the shared antennas is affecting all the layers. This has been analysed in [3]. The assumption is that the proposed 2Rx relaxation for n41 will avoid the additional interference from the shared antennas and should improve the overall throughput.

To gain more insight, link-level simulations have been conducted, and the results are provided in Fig 1. Three cases have been considered.

- The first one is Rank-4 reception on n41. The four receive antennas are separate from the transmit antennas. Therefore, there exists no noise imbalance between the four antennas. This is the ideal case / benchmark.

- The second case is Rank-4 reception on n41 with two shared and two diversity antennas. Due to the shared antennas there exists a noise imbalance between the shared and diversity antennas. The imbalance has been assumed to be 10dB relating to the antenna isolation. We can observe a performance degradation compared to case 1.

- The third case is Rank-2 reception on n41. Two antennas are exclusively used for reception on n41 while the transmit antennas are solely used for transmission. Through the antenna isolation the noise at the receiver is lower which results into higher throughput at low SNR.

Fig 1: Link Level Simulation for CA_n40A-n41A with transmission on n40 and receiver on n41

As indicated in Fig. 1 a UE with noise imbalance suffers throughput loss at low SNR on Rank-4 while Rank-2 provides superior throughput. Only at high SNR Rank-4 surpasses Rank-2 performance. The performance difference occurs from the noise imbalance which is present at Rank-4. For a device experiencing strong noise imbalance between shared and diversity antennas it might be beneficial to allow relaxation to 2Rx on band n41. This could be implemented by introducing a new note to Table 7.3.2-1b. The note specifies the relaxation for handheld devices only and only if it does not support 6Rx. FWA/CPE devices shall always be subject to 4Rx. UE capable of 6Rx can be discussed as next step. The proposal for the implementation can be found below. As alternative a new signalling could be introduced to indicate that a UE supporting MIMO and simultaneous Rx/Tx cannot support both features at the same time for a specific combination. To reduce signalling overhead the indication would be optional and only needed if a UE does not support MIMO and simultaneous Rx/Tx at the same time.

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands Operating band / SCS / Channel bandwidth / REFSENS. Operating band; SCS kHz; Channel bandwidth (MHz); REFSENS (dBm)8; Duplex Mode. …. n40; 15; 5, 10, 15, 20, 25, 30, 40, 50; -100 + 10log10(NRB/25); TDD. 30; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100; -97.5 + 10log10(NRB/11). n411,11, n901,11; 15; 5, 10, 15, 20, 25, 30, 35, 40, 45, 50; -94.8 + 10log10(NRB/52); TDD. 30; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.1 + 10log10(NRB/24). 60; 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; -95.5 + 10log10(NRB/11). …. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. Note 11: For this band the UE is only required to support a minimum of two Rx antenna ports when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. This relaxation does not apply to FWA form factor devices..

Observation: Due to close frequency distance of n40 and n41 these bands are typically grouped together to reduce implementation complexity and number of required antennas. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe de-sense as well as the diversity antennas. However, there is an imbalance of noise between shared and diversity antennas. The shared antenna features higher noise impact compared to the diversity antennas. In case of MIMO reception, the larger noise of the shared antennas is affecting all the layers. Link Level simulations show that the UE with noise imbalance suffers throughput loss at low SNR on Rank-4 while Rank-2 provides superior throughput. Only at high SNR Rank-4 surpasses Rank-2 performance. The performance difference occurs from the noise imbalance which is present at Rank-4. For a device experiencing strong noise imbalance between shared and diversity antennas it might be beneficial to allow relaxation from 4Rx on band n41.

Proposal 2: Consider relaxing 4Rx requirement for n41 when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. As alternative a new signalling could be introduced to indicate that a UE supporting 4Rx in that band and simultaneous Rx/Tx, cannot support both features and the UE is allowed to fall back to 2Rx operation.

# 4	Conclusions

This contribution provides considerations on concurrent MIMO and simultaneous Rx/Tx operation. The following observations and proposals are made:

Proposal 1: Use the MSD provided in Table 1 and Table 2 when specifying the second test point.

Observation: Due to close frequency distance of n40 and n41 these bands are typically grouped together to reduce implementation complexity and number of required antennas. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe de-sense as well as the diversity antennas. However, there is an imbalance of noise between shared and diversity antennas. The shared antenna features higher noise impact compared to the diversity antennas. In case of MIMO reception, the larger noise of the shared antennas is affecting all the layers. Link Level simulations show that the UE with noise imbalance suffers throughput loss at low SNR on Rank-4 while Rank-2 provides superior throughput. Only at high SNR Rank-4 surpasses Rank-2 performance. The performance difference occurs from the noise imbalance which is present at Rank-4. For a device experiencing strong noise imbalance between shared and diversity antennas it might be beneficial to allow relaxation from 4Rx on band n41.

Proposal 2: Consider relaxing 4Rx requirement for n41 when carrier aggregation with band n40 and simultaneous Rx-Tx operation is configured. As alternative a new signalling could be introduced to indicate that a UE supporting 4Rx in that band and simultaneous Rx/Tx, cannot support both features and the UE is allowed to fall back to 2Rx operation.

# 4	References

- RP-242088, “Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19”, Huawei, 3GPP TSG RAN Meeting #105

- R4-2420351, “WF on simultaneous Rx-Tx for CA_n40A-n41A”, Huawei, 3GPP TSG-WG4 Meeting #113

- R4-2417744, “On simultaneous Rx/Tx for CA_n40A-n41A”, Apple, 3GPP RAN WG4 Meeting #113