3GPP RAN TSG-RAN4 Meeting # 114		R4-2500313

Athens, GR, February 17 – 21, 2025

Agenda item:	7.8.2

Source:	Apple

Title:	Initial Views on UE RF requirements for LB CA via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the most recent RAN plenary meeting (Meeting #106, Madrid, Spain, December 9 –12, 2024), discussions were held on the proposal [1] previously summitted by a group of operators on LB-LB carrier aggregation via band switching. As a result, a new Rel19 WI was introduced to handle this topic in the upcoming meetings [2]. The main objectives of the WI are highlighted below.

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this discussion paper, we will share our initial views on the UE RF requirements, using CA_n5A-n29A as an example of the band combination.

# Discussion

  - LB-LB carrier aggregation using band switching

To increase the low band spectrum, LB-LB carrier aggregation has been considered by the operators. However, with the current duplexer and antenna technologies, a solution to aggregate LB carriers based on traditional carrier aggregation architecture (using multiplexers) is very challenging.  The operators have therefore requested 3GPP to consider specifying a solution based on a switching scheme which can potentially address the CA limitations. Under this scheme, when the SCell operation is triggered, the UE needs to switch to the SCell, and during the operation period there is no simultaneous Tx/Rx between the PCell and the SCell. The UE then switches back to the PCell after the SCell operation is finished. Using CA_n5A-n29A as an example of band combination, a simplified model of the RF front-end for the two switching states of UE operation is shown in Figure 2.1-1 below:

-    State 1: n5 (FDD band) is enabled, and n29 (SDL band) is disabled

-    State 2: n5 (FDD band) is disabled, and n29 (SDL band) is enabled

Figure 2.1-1:	Simplified model for realizing CA aggregation between n5 and n29 through switching

We will share our views on the UE RF requirements for the two switching scenarios, switching from n5 to b29, and switching back from n29 to n5. The main requirements that we will consider are the switching delays and time masks. The switching delay, or switching time, or switching period, is the total amount of time it takes for the UE to transition from one state to another state after the switching operation is initiated.

  - Switching time requirements for CA_n5A-n29A via switching

During the band switching process, several events have to occur within the UE baseband, transceiver, and RF front end. Depending on the UE implementation, the main events that could potentially contribute to the switching time are the following:

- PLL settling and relocking (or retuning), depending on the PLL architecture

- Antenna tuning

- RFFE parameters loading, including ET parameters

- RFIC programming and calibration,

To perform the switching, the events above can be performed sequentially or concurrently, depend on the UE implementation and capability. The switching time will be optimized if some of the events listed above can be performed concurrently (or in parallel).

Observation 1:	Performing in parallel some of the events involved in the switching process could shorten the required switching time

- If the local oscillators (LOs) are generated from the same PL, LO retuning will be necessary when switching between the two bands. In this case, the PLL retuning time could be the dominant contributor on the switching time budget. In this scenario, we could use 140uSec as a candidate for the switching time, as is the case for UL Tx switching feature.

- However, if the two LOs are generated from two different PLLs (PLL1 and PLL2), PLL retuning may not be necessary, depending on the UE implementation. In this case, antenna tuning and the other contributors listed above could play a role in estimating the switching time. This time could be than the value mentioned above when LO retuning is required, depending on the UE implementation.

Proposal 1:	To accommodate different UE implementations and considering the existing requirements for UL Tx switching, the set of values {100us, 140us} can be used as a switching period candidate for low NR band carrier aggregation via switching.

The switching period could be different from UE to UE and therefore its value should be signaled to the network as a UE capability. Signaling design to be handled by RAN2.

From the UE perspective, we assume that the UE will report only one value of switching period for both cases (switching from n5 to n29 and from n29 to n5)

Proposal 2:	The switching period value is a UE capability that can be signaled to the network. The UE will report to the network only one value of the switching period for both cases (n5n29 and n29n5)

From the UE perspective, we assume that the UE will report only one value of switching period for both cases (switching from n5 to n29 and from n29 to n5)

  - Timing diagrams for CA_n5A-n29A via switching

Below are potential timing diagrams that can be used for switching between n5 and n29. We assume that both carriers are in the same timing advance group (TAG) and are co-located.

- Case 1: n5n29

Figure 2.3-1: Timing diagrams for n5n29 switching

- Case 2: n29n5

Figure 2.3-2: Timing diagrams for n29n5 switching

  - Proposed liaison text to RAN1

In accordance with the Work Plan [3], a liaison to RAN1 is needed to inform the RAN4 decision on the switching time.  This liaison is needed upon the conclusion of the RAN4 #114 meeting in order to allow RAN1 to make efficient progress in their discussions.

<begin text proposal>

RAN4 would like to update RAN1 on the progress of the discussions related to the switching time for low NR band carrier aggregation via switching.

To accommodate different UE implementations and considering the existing requirements for UL Tx switching, the set of values {100us, 140us} can be used as a switching period candidate for low NR band carrier aggregation via switching.  The switching period value is a UE capability that can be signaled to the network.

RAN4 respectfully requests RAN1 to take this information into account in their work.

<end text proposal>

# 3	Conclusions

Observation 1:	Performing in parallel some of the events involved in the switching process could shorten the required switching time

Proposal 1:	To accommodate different UE implementations and considering the existing requirements for UL Tx switching, the set of values {100us, 140us} can be used as a switching period candidate for low NR band carrier aggregation via switching.

Proposal 2:	The switching period value is a UE capability that can be signaled to the network. The UE will report to the network only one value of the switching period for both cases (n5n29 and n29n5)

# 4	References

RP-241537, “Rx switched nX-n29 low-low carrier aggregation”, TELUS, Bell Mobility,  3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024

RP-243317, “New WID on low band carrier aggregation via switching”, by Apple, TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

R4-2500089, “Work plan for low NR band carrier aggregation via switching,” AT&T, 3GPP RAN4 #114