# 3GPP TSG-RAN WG4 Meeting #114		R4-2500328

Athens, GR, 17th - 21nd February 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	7.18.2

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for UE with 8Rx:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

General

Definition of MMSE-IRC receiver capability for 8Rx UE

Status in the WF [1]: - Proposals - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver. - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI. - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

The existing Rel-18 8Rx UE receiver capability definition in TS38.306 cannot be applicable for MU-MIMO scenarios:

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

In Rel-17, instead of introducing new mandatory UE capability for 2/4Rx MMSE-IRC, we only defined applicability rules to clearly state that the MU-MIMO requirements are applicable for all UEs with 2/4Rx. With the common understanding that the 8Rx UE receiver implementation is the same as Rel-18, we propose option 1:

  - Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver.

As for option 2 or option 4 as above, TS 38.306 needs to be updated as well thus a RAN4 to RAN2 LS is required. And option 3 that define multiple UE capabilities for single UE receiver implementation is not preferred.

Proposal 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability.

PDCSH requirements with inter cell interference

Number of interference cells

- Background: - We have an existing agreement from RAN#112bis, to reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx, with TBD for applicability rules to reduce UE efforts [R4-2416555] - Proposals: - Option 1: Both 1 interference cell and 2 interference cells

In RAN4#112bis, we have agreed to reuse the existing interference profiles (i.e., cover HomNet scenario with 2 neighbor cells and Hetnet scenario with 1 neighbor cell) as Rel-17 2/4Rx. Therefore, option is the only choice.

We could also understand some companies’ concern on the test burden if multiple test scenarios are defined. It is now still FFS on the test applicability rule to reduce the test burden.

In the 2/4Rx requirement definition phase, companies have agreed the following applicability rule for the UE to only select one scenario for TDD and FDD respectively [2]:

‘If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode’

In the final test requirement definition, the test applicability rule is as below:

- 5.1.1.10 Applicability of requirements for PDSCH with inter cell interference Tests; Applicability notes. All tests in Clause 5.2.2.1.15, 5.2.3.1.15, 5.2.2.2.16 and 5.2.3.2.16; If UE supporting both duplex mode TDD and FDD with 2RX, only test 1-1 in clause 5.2.2.1.15 and test 1-2 in clause 5.2.2.2.16 will be applied. If UE supporting both duplex mode TDD and FDD with 4RX, only test 1-1 in clause 5.2.3.1.15 and test 1-2 in clause 5.2.3.2.16 will be applied..

We propose to reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements.

Proposal 2: To follow the previous meeting agreement, model 2 neighbor cells for HomNet and 1 neighbor cell for Hetnet scenario.

Proposal 3: Reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements, i.e., if UE supporting both TDD and FDD with 8Rx, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Applicability rule and release independent

Test applicability

Status in the WF [1]: Test applicability for 8Rx UEs Agreement: - New requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. - TBD: additional test applicability rule. Test applicability to skip 2/4Rx requirements - Proposals - Option 1: Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed. - Option 1a: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 1b: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below. We believe the same approach can be reused for the Rel-19 new requirement to be defined.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

Proposal 4: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Release independent

For release independent, the 8Rx UE feature is introduced in Rel-18 and the corresponding 8Rx UE performance requirements are release independent from Rel-17 as specified in TS38.307. In addition, the UE MMSE-IRC is a mandatory feature for all Rel-17 and forward UEs:

Table 5.4-1: Additional requirements of other release independent features Feature; Release independent from; Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced); Further information. UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference; Rel-15; Table B.3.3-1; Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration.. Demodulation requirements for 8Rx UE; Rel-17; Table B.3.5-1. Table B.3.5-1: UE PDSCH demodulation requirements with 8Rx Section / Clause; Description. 5.2.4.1; PDSCH demodulation requirements for 8Rx FDD. 5.2.4.2; PDSCH demodulation requirements for 8Rx TDD. 5.2A.4; PDSCH CA demodulation requirements for 8Rx. 6.2.4; CQI requirements with 8Rx.

We have already discussed the possibility to cover MMSE-IRC requirements during the performance part discussion under WI ‘NR_ENDC_ RF_FR1_enh2’. Such requirements are not introduced in Rel-18 mainly because of the limited time and huge workload. Therefore, we think the new requirements for 8Rx should be release independent from Rel-18.

Proposal 5: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

Conclusion

Proposal 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability.

Proposal 2: To follow the previous meeting agreement, model 2 neighbor cells for HomNet and 1 neighbor cell for Hetnet scenario.

Proposal 3: Reuse the same test applicability rule as 2/4Rx MMSE-IRC requirements, i.e., if UE supporting both TDD and FDD with 8Rx, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode.

Proposal 4: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 5: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

Reference

R4-2419784 Way forward for [113][322] NR_demod_Ph5_Part2_UE, Nokia, TSG-RAN WG4 Meeting #113

R4-2120707, WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021