3GPP TSG-RAN WG4 Meeting #114	R4-2500374

Athens, Greece, 17-21 Feb 2025

Title: 	On MPR Enhancement

Agenda Item:	7.1.1.2.1

Source: 	Nokia

Document for:	Approval

# 1				Introduction

In this contribution we express our views for single carier MPR enhancements.

# 2	Discussion

In main room discussion it was agreed that both scenarios 1 and 2 should be covered.

#### Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation

Agreement in main session:

- Extended CBW based approach

  - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE

  - Both scenario 1 and 2 should be covered

  - The network should ensure the co-existence be met when signaling the extended CBW.

In our view scenario 1 is not universal scenario, instead it is local scenario. Furthermore implications from interference point of view are not yet well understood. But as it was agreed to also address scenario 1 thus we propos that different UE capabilities are assigned to these scenarios, this is because it may be turned out later that UE requirements will be very different and or there are other reasons why it is convenient to refer these scenarios separately and not commonly.

Proposal 1: Define separate UE capabilities for scenario 1 and scenario 2.

#### Issue 1-1-2: Approaches of calculating the new inner RB region

We support the methodology in Figure 1 (Re-produced from [2]), where the new UE inner RB region is the light green colored area which is the intersection of Extended UE CBW inner and allocation triangle of SIB1 UE CBW. New outer is the light yellow colored area which is New UE allocation triangle – new UE Inner RB region.

Proposal 2: New UE inner RB region the light green colored area which is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW. New outer is the light yellow colored area which is New UE allocation triangle – new UE Inner RB region.

- New UE inner RB region is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW.

- New UE outer is New UE allocation triangle – new UE Inner RB region.

#### Issue 1-1-3: Width/RB numbers of extended CBW at each side of UE CBW

Best is to define this new method very generic therefore in scenario 2 no limitataions are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion.

Proposal 3: In scenario 2 no limitataions are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion.

#### Issue 1-2-3: Boundary to apply ACLR and SEM

- WF

  - ACLR and SEM should be applicable from the edge of extended UE CBW.

    - FFS the following alternatives

      - Alt 1: The OOBE requirements are based on UE CBW

      - Alt 2: The OOBE requirements are based on extended UE CBW

We support Alternative 2. If we choose alternative 1 then gains of this feature are unnecessarily limited.

Proposal 4: Alt 2: The OOBE requirements are based on extended UE CBW

#### Issue 1-2-4: Boundary to apply SE

- Proposals

  - Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW. (vivo, LGE)

  - Proposal 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz. (Nokia)

  - Proposal 3: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW (Qualcomm, OPPO)

- WF

  - FFS with consideration of Issue 1-2-3

Proposal 5: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal 1: Define separate UE capabilities for scenario 1 and scenario 2.

Proposal 2: New UE inner RB region the light green colored area which is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW. New outer is the light yellow colored area which is New UE allocation triangle – new UE Inner RB region.

- New UE inner RB region is the intersection of Extended UE CBW inner region and allocation triangle of SIB1 UE CBW.

- New UE outer is New UE allocation triangle – new UE Inner RB region.

Proposal 3: In scenario 2 no limitataions are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion.

Proposal 4: Alt 2: The OOBE requirements are based on extended UE CBW

Proposal 5: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz.

# 4	References

[1] WF

[2] R4-2418015,Redefining allocation types for improved MPR with BS BW larger than UE CBW	Skyworks Solutions