# 3GPP TSG-RAN WG4 Meeting #114                                                                R4-2500409

Athens, Greece, 17-21 February, 2025

Title: 	Remaining discussions on UE capability for type 4 UEs

Source: 	Huawei, HiSilicon

Agenda item:	7.5.2.2

Document for:	Discussion

# Introduction

RAN4 113 meeting the WF[1] was agreed, in which some open points are left without a conclusion. In this document we are going to address them.

# Discussion

### <Issue 2-2-1: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC (collocated)> Way forward: - WF on this Issue 2-2-1 is consolidated into one WF of Issue 2-2-2.

In Rel-18 switching type 2 to type1 was agreed, in order to enable more DL layers per CC (2 DL layers/CC for type 2, 4 DL layers/CC for type 1). As type 4 and type 1 have 4 DL layers/CC then such a switch does not show its advantage. On the other hand, by switching from type 4 (8 separated Rx chain) to type 1 (4 partially shared Rx chain) the UE can use the remaining Rx chains for diversity, which can be helpful for a better quality of service in scenarios with coverage difficulties.

Proposal 1: Switching from type 4a/4b to type 1 4layers/CC can be beneficial due to increase reception diversity.

Proposal 2: To allow a switching from type 4a/b (default type) to type 1 with 4Rx, RAN2 should, introduce a new NW IE.

### <Issue 2-2-2: New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)> Way forward: - For NR-CA, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC. - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration. - For EN-DC, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC. - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

To enable switching from type 4 to type 1 6/8Rx per CC, three different solutions can be considered. The first solution consists of using the legacy NW IE maxMIMO-layers, where setting it to 6 or 8 would mean switching UE to type 1 6Rx or 8 Rx per CC. This is a straightforward path but it can lead to other interpretations in the spec because this IE has been used repeatedly in the spec for different scenarios. The second solution consists of having two new NW IEs (1 for NR CA and 1 for EN-DC) which would indicate a switching from type 4 to type 1 4Rx, then the legacy NW IE maxMIMO-layers is used to indicate the number of layers per CC. The Third solution requires separate new IEs for switching from type 4 to type 1 4/6/8Rx per CC.

Proposal 3: To switch from type 4 (default type) to type 1 46/8Rx/CC, three solutions can be considered. We have a slight preference for solution 3, as it has no ambiguity.

| Solution | NR CA NW IE | EN-DC NW IE | Remark |
| --- | --- | --- | --- |
| 1 | maxMIMO-layers=8 | MIMO-CapabilityDL-r10=6,8 | Switching to type 1 4Rx would not be possible as these two IEs can be set to 4 for both type 4 UE and type 1 with4 Rx/CC. On the other hand, the legacy IEs could be misinterpreted as they were widely used in other applications |
| 2 | [nonCollocatedTypeFourToOne-NR-CA] + maxMIMO-layers=4,8 | [nonCollocatedTypeFourToOne-MRDC] + MIMO-CapabilityDL-r10=4,6,8 | It could be confusing if other non-collocated types with 6 or 8Rx/CC is introduced in future releases (however it has a low probability of happening). On the other hand, the legacy IEs could be misinterpreted as they were widely used in other applications |
| 3 | [nonCollocatedTypeFourToOne-NR-CA] + [maxMIMO-layers-collocated]=4,6 | [nonCollocatedTypeFourToOne-MRDC] + [MIMO-CapabilityDL-r10-collocated]=4,6,8 | Every scenario has a dedicated signalling without any confusion |
| NOTE: New NW IEs are within []. |  |  |  |

### <Issue 2-2-3: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b> Way forward: - When maxMIMO-layers = 2 - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. NOTE: This is specified Rel-18 behavior. No need to further discuss. - When maxMIMO-layers ＞4 - FFS in the next meeting. - When maxMIMO-layers = 4 - FFS in the next meeting. NOTE: maxMIMO-layers means the existing BS signaling.

The wording initial connection is ambiguous. It is not clear whether it is referring to initial access procedure or it refers to the first RRC reconfiguration. Probably ‘initial connection’ does not refer to initial access procedure, as NW IE maxMIMO-layers is not used at initial access procedures (MIB, SIB1 and etc) at all.

Observation 1: Initial connection does not refer to the initial access procedure and it is about the first RRC reconfiguration procedure.

Proposal 4: During the initial connection the network is aware of the UE capabilities. if solution 2 is adapted, the procedure can be given as follows:

If solution 2:

When NW IE maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE remains as Type 2.

When maxMIMO-layers >4

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is present, UE is Type 1 with maxMIMO-layers per CC

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is absent, UE remains as Type 4. (default behavior)

When maxMIMO-layers = 4

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is present, UE is Type 1 with maxMIMO-layers per CC

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is absent, UE remains as Type 4. (default behavior)

If solution 3 is adapted, the procedure can be given as follows:

If solution 3:

When NW IE [maxMIMO-layers-collocated] /[MIMO-CapabilityDL-r10-collocated] = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

When [maxMIMO-layers-collocated] /[MIMO-CapabilityDL-r10-collocated] ≥4

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is present, UE is Type 1 with [maxMIMO-layers-collocated] /[MIMO-CapabilityDL-r10-collocated]  per CC

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is absent, UE remains as Type 4.(default behavior)

# Conclusions

Proposal 1: Switching from type 4a/4b to type 1 4layers/CC can be beneficial due to increase reception diversity.

Proposal 2: To allow a switching from type 4a/b (default type) to type 1 with 4Rx, RAN2 should, introduce a new NW IE.

Proposal 3: To switch from type 4 (default type) to type 1 46/8Rx/CC, three solutions can be considered. We have a slight preference for solution 3, as it has no ambiguity.

| Solution | NR CA NW IE | EN-DC NW IE | Remark |
| --- | --- | --- | --- |
| 1 | maxMIMO-layers = 8 | MIMO-CapabilityDL-r10 = 6,8 | Switching to type 1 4Rx would not be possible as these two IEs can be set to 4 for both type 4 UE and type 1 with4 Rx/CC. On the other hand, the legacy IEs could be misinterpreted as they were widely used in other applications |
| 2 | [nonCollocatedTypeFourToOne-NR-CA] + maxMIMO-layers=4,8 | [nonCollocatedTypeFourToOne-MRDC] + MIMO-CapabilityDL-r10=4,6,8 | It could be confusing if other non-collocated types with 6 or 8Rx/CC is introduced in future releases (however it has a low probability of happening). On the other hand, the legacy IEs could be misinterpreted as they were widely used in other applications |
| 3 | [nonCollocatedTypeFourToOne-NR-CA] + [maxMIMO-layers-collocated] = 4,6 | [nonCollocatedTypeFourToOne-MRDC] + [MIMO-CapabilityDL-r10-collocated] = 4,6,8 | Every scenario has a dedicated signalling without any confusion |
| NOTE: New NW IEs are within []. |  |  |  |

Observation 1: Initial connection does not refer to the initial access procedure and it is about the first RRC reconfiguration procedure.

Proposal 4: During the initial connection the network is aware of the UE capabilities. if solution 2 is adapted, the procedure can be given as follows:

If solution 2:

When NW IE maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE remains as Type 2.

When maxMIMO-layers >4

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is present, UE is Type 1 with maxMIMO-layers per CC

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is absent, UE remains as Type 4. (default behavior)

When maxMIMO-layers = 4

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is present, UE is Type 1 with maxMIMO-layers per CC

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is absent, UE remains as Type 4. (default behavior)

If solution 3 is adapted, the procedure can be given as follows:

If solution 3:

When NW IE [maxMIMO-layers-collocated] /[MIMO-CapabilityDL-r10-collocated] = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

When [maxMIMO-layers-collocated] /[MIMO-CapabilityDL-r10-collocated] ≥4

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is present, UE is Type 1 with [maxMIMO-layers-collocated] /[MIMO-CapabilityDL-r10-collocated]  per CC

- If Rel-19 [nonCollocatedTypeFourToOne-NR-CA]/ [nonCollocatedTypeFourToOne-MRDC] is absent, UE remains as Type 4. (default behavior)

# Reference

[1]	“R4-2420399, WF on non-collocated intra-band EN-DC and NR-CA, KDDI.”