3GPP TSG-RAN WG4 Meeting #114	R4-2500415

# Athens, GR, 17th – 21st February, 2025

Source: 	KDDI

Title: 	Discussion on UE Capability and BS Signaling for non-collocated Type 4a4b

Agenda Item:	7.5.2.2

Document for:	Discussion

- 1. Introduction

In RAN#106 meeting, the revised WID for intra-band non-collocated EN-DC/NR-CA is approved [1]. The WI’s core part is focusing on specifying UE RF/RRM requirements for Type-4 UE capability for FWA and clarifying the UE behaviour for Type-4 supported UE. Some discussions were organized in the last RAN4#113 meeting and the agreements were captured in [2]. We try to solve remaining issues in this contribution.

- 2. Discussion

- 2.1 New BS signaling for Type 4a/4b

The revised WID [1] clearly says that Type 4a/4b capability is capable for FWA UE as follows.

- 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed.

FWA means Fixed Wireless Access. This means that FWA UE having Type 4 capability is not considered to have mobility functions. On the other hand, Type 1 and 2 are handheld UEs, and also they have mobility functions.

Observation 1: Type 4 capability for FWA UE is not considered to have mobility functions.

RAN4 had discussed both an initial access case (Idle-mode) and a switching case between Type 4 and Type 1/2 (Connected) at the same time in the previous meetings and those discussions were controversial same as Type 2 in Rel-18. Therefore, from the rapporteur point of view, we propose to give prioritization for two cases.

Considering remaining TU budgets and above Observation 1, we propose to firstly conclude an initial access case. Because it is mandatory to introduce Type 4. On the other hand, a switching case is optional for FWA.

Proposal 1: Firstly, conclude an initial access case. Next, discuss a switching access case.

- 2.2 UE behavior on initial access

In Rel-18, RAN2 designed BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC in a case of maxMIMO-Layers with value less than or equal to 2. The following description on Rel-18 BS signaling and UE behavior on Type 1 and Type 2 for NR-CA is excerpted from TS 38.331 v18.2.0 section 6.3.2.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

The above Rel-18 BS signaling nonCollocatedTypeNR-CA (nonCollocatedTypeMRDC for EN-DC) is not mentioned for a case with maxMIMO-layers equal to 4, but also in our understanding the case of maxMIMO-layers equal to 4 is collocated scenario Type 1.

Observation 2: Rel-18 BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for Type 2 is specified for the case with maxMIMO-layers less than or equal to 2.

Observation 3: In Rel-18, the case of maxMIMO-layers larger than 2 is collocated Type 1.

Based on the above Rel-18 BS signaling, we can follow a same direction on Rel-19 BS signaling to switch between Type 1 and Type 4a/4b. So specifically, Rel-19 BS signaling to switch between Type 1 and Type 4a/4b is needed to specify for the case with maxMIMO-layers larger than 2 and less than or equal to 4. Additionally, the case of maxMIMO-layers larger than 4 is collocated scenario Type1.

Proposal 2: Specify Rel-19 BS signaling for Type 4a/4b as the case with maxMIMO-layers larger than 2 and less than or equal to 4.

Proposal 3: The case of maxMIMO-layers larger than 4 is collocated Type 1.

Based on the previous observation 2 and 3, UE behavior on initial access for Rel-18 is as follows.

- The case of maxMIMO-layers larger than 2 is Type 1.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

We also think that same direction might be adopted for Rel-19 as follows.

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present is Type 1.

  - Rel-19 New BS signaling is absent is Type 4.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

On the other hand, we need to consider the following two cases raised during offline discussion after the last RAN4 meeting.

- Type 4 capable Rel-19 UE behavior operate under Rel-18 BS

- Only Type 2 capable Rel-19 UE behavior under Rel-19 BS

Especially, regarding above case A, assuming Rel-19 UE indicates both Type 4 and Type 2 capability, then Rel-18 BS only understand Type 2 and use maxMIMO-layers=4 to control UE work under collocated as Type 1 if following above Rel-18 based new BS signaling design. However, Rel-19 UE would see "new Rel-19 BS signaling=absent + maxMIMO-layers=4", then this Rel-19 UE misthink Type 4 under Rel-18 BS. Eventually, discrepancy between Rel-19 UE and Rel-18 BS happens.

So, we propose new BS signaling to control “present with having value 0/1” and “absent” adding to only “present/absent” of Rel-18 BS signaling as follows, so that new BS signaling can solve the discrepancy as above mentioned.

Proposal 4: Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling:

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

- 2.3 Switching between single CC, Type 1, Type 2 and Type 4

In the last meeting, we discussed UE behaviors/transitions between single CC operation, Type 1, Type 2 and Type 4 (4b) with using Rel-19 new BS signaling and maxMIMO-layers. And then, some companies pointed out to keep a consistency between an initial access case and a switching case.

Proposal 5: If RAN4 specify UE behavior on switching case, a consistency between an initial access case and a switching case should be kept.

- 3. Conclusion

Observation 1: Type 4 capability for FWA UE is not considered to have mobility functions.

Observation 2: Rel-18 BS signaling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for Type 2 is specified for the case with maxMIMO-layers less than or equal to 2.

Observation 3: In Rel-18, the case of maxMIMO-layers larger than 2 is collocated Type 1.

Proposal 1: Firstly, conclude an initial access case. Next, discuss a switching access case.

Proposal 2: Specify Rel-19 BS signaling for Type 4a/4b as the case with maxMIMO-layers larger than 2 and less than or equal to 4.

Proposal 3: The case of maxMIMO-layers larger than 4 is collocated Type 1.

Proposal 4: Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling:

- The case of maxMIMO-layers larger than 4 is Type 1.

- The case of maxMIMO-layers equal to 4 and

  - Rel-19 New BS signaling is present with value 0 is Type 4.

  - Rel-19 New BS signaling is present with value 1 is Type 1.

  - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification.

- The case of maxMIMO-layers equal to 2 and

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1.

  - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2.

Proposal 5: If RAN4 specify UE behavior on switching case, a consistency between an initial access case and a switching case should be kept.

- 4. Reference

[1] RP-242083 Revised WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), KDDI, RAN#105

[2] R4-2420399 WF on non-collocated intra-band EN-DC and NR-CA, KDDI, RAN4#112bis