3GPP TSG-RAN WG4 Meeting #114							         		R4-2500420

Athens, GR, 17th – 21st February 2025

Title: 	Inputs to 7MHz CBW

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.9.2	UE RF requirements and system parameters	[NR_FR1_7MHz_BW-Core]

Document for:	Approval

# 1	Background

In RP#106, a way forward [1] and a new work item [2] were approved to introduce 7MHz channel bandwidth for band n26 and band 5. In this contribution, we provide our initial input to the spectrum utilization and the UE RF UL and DL requirements.

# Discussion

## 2.1 Work Item Objectives

The core part of this WI in [2] includes the following objectives on the UE side:

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4]

  - Introduce the channel bandwidth 7MHz, Only include 15 kHz SCS

  - Specify the spectrum utilization.

  - Specify UE RF requirements for UL and DL

- Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4]

  - BS and UE Channel bandwidth per operating band

  - UE reference sensitivity

  - Additional Maximum Power Reduction (A-MPR)

## 2.2 Input for Spectrum Utilisation

To determine the 7MHz CBW spectrum utilisation for 15kHz SCS, we compare the 35RB and 36RB options in Table 1, with the existing 5MHz and 10MHz cases.

Table 1: 7MHz SU options versus 15kHz SCS 5MHz and 10MHz CBW cases

| CBW [MHz] | 5 | 7 | 7 | 10 |
| --- | --- | --- | --- | --- |
| NRB | 25 | 35 | 36 | 52 |
| RBBW [MHz] | 4.5 | 6.3 | 6.48 | 9.36 |
| GB [MHz] | 0.2425 | 0.3425 | 0.2525 | 0.3125 |
| SU | 90% | 90% | 93% | 94% |

Observations:

- The 35RB option has the same SU as the 5MHz CBW case, and results in a comfortable guard band that is higher than the 10MHz CBW case, and thus ensures margin for MPR.

- The 36RB option has a better SU close to the 10MHz CBW case, but results in a guard band that is barely higher than the 5MHz CBW case, and thus MPR may not be sufficient.

- Note that 35RB is already used for the LTE 7MHz broadcast channel.

Proposal: An NRB of 35RB is used for 15kHz SCS 7MHz channel bandwidth resulting in a 90% spectrum utilisation and 0.3425MHz lower guard band. With this type guard band, there is no need to recheck MPR.

## 2.3 UL Requirements

When checking the additional emission requirements for n5 and n26 in 38.101-1, We found that the NS summarized in Table 2, applies to those bands.

Table 2: NS additional emission requirements applying to band n5 and n26

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_12 | 6.5.3.3.17 | n26 | 3,5,10 | Table 6.2.3.21-1 | Table 6.2.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 3, 5 | Table 6.2.3.22-1 | Table 6.2.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10,15,20 | Table 6.2.3.23-1 | Table 6.2.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 3,5,10,15,20 | Table 6.2.3.24-1 | Table 6.2.3.24-2 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |

Observations:

- Band n5 is only subject to the UTRA ACLR NS_100 requirement. The A-MPR for NS_100 is the same no matter the channel bandwidth, and thus should also be covered for 7MHz.

- Band n26 is subject to NS_12/13/14/15, with specific CBW.

  - NS_12 and NS_15 already apply for 5MHz and 10MHz CBW, so it should also apply to the 7MHz CBW.

  - NS_13 applies up to 5MHz, however, it is unclear if a 7MHz CBW should apply.

  - NS_14 starts at 10MHz CBW, however, it is unclear if a 7MHz CBW should apply.

Proposal on additional emission requirements:

- For band n5, A-MPR for NS_100 should apply as is for 7MHz, if necessary, the current A-MPR values can be checked for 7MHz.

- For n26, A-MPR studies for 7MHz are needed for NS_12 and NS_15. For NS_13 and NS_14 it should be first clarified if the requirement applies for a 7MHz CBW, as currently, 5MHz is the maximum CBW for NS_13, and 10MHz is the minimum CBW for NS_14.

  - Given that the requirement applies at 1MHz offset for NS_13, it is likely that a 7MHz CBW applies as well.

  - Given that the requirement applies at 8MHz offset for NS_13, it is possible that a 7MHz CBW does not apply.

## 2.3 DL Requirements

Assuming a 35RB NRB is adopted for a 7MHz CBW for 15kHz SCS, and in order to determine its REFSENS for band sn5 and n26, we can first check if the REFSENS scales between 5MHz and 10MHz. If so, this means that no additional Tx noise was considered.

Table 3 provides the specified and RB scaled REFSENS for n5 and n26, for the range of 5MHz to 15MHz CBW.

Table 3: Specified and scaled REFSENS for band n5 and n26.

| Operating Band | Channel bandwidth [MHz] | 5 | 7 | 10 | 15 |
| --- | --- | --- | --- | --- | --- |
|  | NRB | 25 | 35 | 52 | 79 |
| n5 | Specified REFSENS [dBm] | -98 |  | -94.8 | -93 |
|  | Scalled REFSENS scalled [dBm] |  | -96.5 | -94.8 | -93.0 |
| n26 | Specified REFSENS [dBm] | -97.5 |  | -94.5 | -92.7 |
|  | Scalled REFSENS scalled [dBm] |  | -96.0 | -94.3 | -92.5 |

Observations: Aside from a minor anomaly in band n26, which is likely due to a straight copy of LTE REFSENS which ignored the wider RB BW, both band n5 and n26 have their REFSENS scaling up to 15MHz, thus RB scaling can be used to derive 7MHz CBW REFSENS.

Separately, the band n5 and n26 REFSENS UL configuration is 25RB for both bands for both 5MHz and 10MHz CBW, thus the same 25RB can be used for 7MHz RESENS.

For ACS, for bands <2700MHz, ACS for 3,5,10MHz is 33dB, with interferer level of REFSENS+45.5dB in case 1, and -56.5dBm in case 2, for 5 and 10MHz.

For in-band blocking, the requirement is the same for 5MHz and 10MHz, and can thus be copied for a 7MHz CBW. For out-of-band blocking, the requirement is the same for 5MHz and 10MHz and can thus be copied for a 7MHz CBW. Similarly, for narrow band blocking, spurious response and wideband intermodulation, the requirements are the same for 5MHz and 10MHz, and can thus be copied for a 7MHz CBW.

Proposal on 7MHz DL requirements:

- Assuming a 35RB spectrum utilisation, the 7MHz REFSENS is -96.5dBm for n5, and -96dBm for n26, with 25RB REFSENS UL configuration.

- The ACS requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW case 1 and case 2.

- The in-band blocking requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW case 1 and 2.

- The out of band blocking requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW, range 1, 2 and 3.

- The narrow band blocking requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW.

- The wideband intermodulation requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW.

- The spurious response requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW.

## 2.4 Option for Asymmetric UL/DL CBW for n26

As shown above, the DL requirements for 7MHz CBW seem straight forward. For n26 UL however, the A-MPR study is needed for the applicable NS. With this in mind, UEs with sufficient margin for blocking and ACS may already be capable of receiving the 7MHz channel bandwidth as is. However, for the UL, the A-MPR needs to be characterized and tested for the UEs. If an asymmetric 5MHz UL with a 7MHz DL, was supported as an option, this approach may enable an earlier implementation of the 7MHz CBW support in the DL.

# Conclusion

In this contribution, we discussed the 7MHz channel spectrum utilisation and the applicable additional emission requirements for band n5 and n26 with the following proposals.

Proposal: An NRB of 35RB is used for 15kHz SCS 7MHz channel bandwidth resulting in a 90% spectrum utilisation and 0.3425MHz lower guard band. With this type guard band, there is no need to recheck MPR.

Proposal on additional emission requirements:

- For band n5, A-MPR for NS_100 should apply as is for 7MHz, if necessary, the current A-MPR values can be checked for 7MHz.

- For n26, A-MPR studies for 7MHz are needed for NS_12 and NS_15. For NS_13 and NS_14 it should be first clarified if the requirement applies for a 7MHz CBW, as currently, 5MHz is the maximum CBW for NS_13, and 10MHz is the minimum CBW for NS_14.

  - Given that the requirement applies at 1MHz offset for NS_13, it is likely that a 7MHz CBW applies as well.

  - Given that the requirement applies at 8MHz offset for NS_13, it is possible that a 7MHz CBW does not apply.

Proposal on 7MHz DL requirements:

- Assuming a 35RB spectrum utilisation, the 7MHz REFSENS is -96.5dBm for n5, and -96dBm for n26, with 25RB REFSENS UL configuration.

- The ACS requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW case 1 and case 2.

- The in-band blocking requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW case 1 and 2.

- The out of band blocking requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW, range 1, 2 and 3.

- The narrow band blocking requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW.

- The wideband intermodulation requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW.

- The spurious response requirement for a 7MHz CBW is copied from the requirement that applies to both 5MHz and 10MHz CBW.

# References

[1] RP-243322 Way forward in irregular channel bandwidths, T-Mobile, RP#106

[2] RP-243323 New WID on 7 MHz Channel Bandwidth for n26 and n5, T-Mobile USA, China Telecom, RP#106