3GPP TSG-RAN WG4 Meeting #114	R4-2500421

Athens, Greece 17th – 21st February 2025

Source: 	Nokia

Title: 					n1 NS_05 A-MPR proposals

Agenda Item: 			4.2.1

Document for:	Approval

# 1	Introduction

We provide A-MPR proposals for band n1 and updated emission mask for NS_05. The proposals are based on our simulations presented in [1] and WF of previous meeting [2].

# 2	Simulation assumptions and scenario

Here we provide the simulation scenario and parameters that were used in the A-MPR simulations [1] leading to the proposals.

- Operating band n1 (UL 1920 MHz – 1980 MHz)

- Power classes PC2 and PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- Only additional spurious emission mask for Japan is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

- New additional spurious emission mask for Japan [3] signaled with NS_05:

  - CBW = 5MHz for PC2 and PC3

    - 1884.5-1910.0 MHz: -30 dBm@1MHz

    - 1910.0-1915.7 MHz: -25 dBm@1MHz

  - CBW = 10 MHz, 15 MHz, 20MHz for PC2 and PC3

    - 1884.5-1906.6 MHz: -30 dBm@1MHz

    - 1906.6-1915.7 MHz: -25 dBm@1MHz

# 3	A-MPR proposals

## 3.1	PC3

We provided both simulation results and proposals for the previous meeting [1]. For the present contribution we have adapted our A-MPR proposal for PC3 according to the agreement for A-MPR regions made during the previous meeting [2]. The only difference in the RB regions between [2] and our current proposal is the IMD3 region at 20 MHz CBW highlighted with green color. According to our analysis, the allocation-image IMD3 does not reach the additional mask when RBStart is 27 or larger with 15 kHz SCS. We have also provided our results from the simulations for the no-A-MPR center frequencies in the frequencies highlighted with yellow color.

Proposal 1: RAN4 shall consider defining A-MPR for n1 PC3 NS_05 as shown in tables 1 and 2.

Table 1: The proposed A-MPR regions for n1 PC3 NS_05.

| NS_05 | Fc MHz | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 10M | < 1929.43 | ≥ 1.5*RBstart + 4.7MHz/12/scs | >0 | <= 2.16MHz/12/scs | <= 1.8MHz/12/scs |
| 15M | 1927.5≦fc＜1932.5 | >= RBstart*1.45 + 4.68MHz/12/scs | >0 | <= 3.6MHz/12/scs | <= 3.6MHz/12/scs |
| 15M | 1932.5≦fc＜1936.70 | ≥ 12.96/12/scs | ≤ 0.54/12/scs | <= 2.16MHz/12/scs | <= 2.16MHz/12/scs |
| 20M | < 1944.07 | >= RBstart*1.5 + 4.14MHz/12/scs; | >0 | <= 5.76MHz/12/scs; <= RBstart*1.5 + 4.14MHz/12/scs; | <= 4.68MHz/12/scs |

Table 2: The proposed A-MPR values for n1 PC3 NS_05.

| Modulation/Waveform |  | CBW = 10 MHz, Fc < 1929.43 MHz |  | CBW = 15 MHz, 1927.5 MHz <= Fc < 1932.5 MHz |  | CBW = 15 MHz, 1932.5 MHz <= Fc < 1936.70 MHz |  | CBW = 20 MHz, Fc < 1944.07 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A1 | A2 | A1 | A2 | A1 | A2 | A1 | A2 |
| DFT-s-OFDM | PI/2 BPSK | 2 | 1.5 | 2 | 2.5 | 1 | 1.5 | 2 | 2.5 |
|  | QPSK | 3 | 2 | 3 | 3 | 1.5 | 2 | 3 | 3 |
|  | 16 QAM | 3 | 2 | 3 | 3.5 |  | 2 | 3 | 3.5 |
|  | 64 QAM | 3 |  | 3 | 3.5 |  |  | 3 | 3.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 4 | 3 | 4 | 5 |  | 3 | 4 | 5 |
|  | 16 QAM | 4 | 3 | 4 | 5 |  | 3 | 4 | 5 |
|  | 64 QAM | 4 |  | 4 | 5 |  |  | 4 | 5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |

## 3.2	PC2

Since no agreement on the harmonized A-MPR region definition for PC2 was made during the previous meeting [2], we resubmit our proposal from [1]. In Appendix we show complete sets of the A-MPR triangles from the proposal. In Fig. 1 we show some examples of them.

Proposal 2: RAN4 shall consider defining A-MPR for n1 PC2 NS_05 as shown in tables 3 and 4.

Table 3: The proposed A-MPR regions for n1 PC2 NS_05.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.37 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
|  |  | ≤ 0.18 | > 2.7, ≤ 3.6 | A3 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 3.42 | A2 |
|  |  | 0 | ≤ 1.62 | A4 |
|  | 1932.5 ≤ Fc < 1936.69 | ≤ 1.26 | ≥ 1.0*12*SCS*RBstart + 10.2 | A5 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
| 20 MHz | Fc < 1943.87 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 3.6 | A2 |
|  |  | ≤ 0.9 | > 4.14, < 1.0*12*SCS*RBstart + 4.7 | A6 |
|  |  | > 1.8, ≤ 3.42 | > 4.14, ≤ 5.76 | A7 |

Table 4: The proposed A-MPR values for n1 PC2 NS_05.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 1.5 |  | 3 | 1.5 | 1 |  |
|  | QPSK | 4 | 2 | 1.5 | 3.5 | 2.5 |  | 1 |
|  | 16 QAM | 4 | 2.5 |  | 4 | 2.5 |  |  |
|  | 64 QAM | 4 | 3 |  | 4 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 5 | 4 |  | 5.5 | 3.5 |  | 2.5 |
|  | 16 QAM | 5 | 4 |  | 5.5 | 3.5 |  |  |
|  | 64 QAM | 5 | 4 |  | 5.5 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |

Figure 1: Examples of the proposed A-MPR to satisfy the new NS_05 additional spurious emission limit for 10 MHz and 20 MHz CBWs at the lower edge of the operating band (Fc=1925 MHz and Fc=1930 MHz) for PC2.

# 4	Conclusions

Based on our simulation results we have made the following proposals.

Proposal 1: RAN4 shall consider defining A-MPR for n1 PC3 NS_05 as shown in tables 1 and 2.

Table 1: The proposed A-MPR regions for n1 PC3 NS_05.

| NS_05 | Fc MHz | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 10M | < 1929.43 | ≥ 1.5*RBstart + 4.7MHz/12/scs | >0 | <= 2.16MHz/12/scs | <= 1.8MHz/12/scs |
| 15M | 1927.5≦fc＜1932.5 | >= RBstart*1.45 + 4.68MHz/12/scs | >0 | <= 3.6MHz/12/scs | <= 3.6MHz/12/scs |
| 15M | 1932.5≦fc＜1936.70 | ≥ 12.96/12/scs | ≤ 0.54/12/scs | <= 2.16MHz/12/scs | <= 2.16MHz/12/scs |
| 20M | < 1944.07 | >= RBstart*1.5 + 4.14MHz/12/scs; | >0 | <= 5.76MHz/12/scs; <= RBstart*1.5 + 4.14MHz/12/scs; | <= 4.68MHz/12/scs |

Table 2: The proposed A-MPR values for n1 PC3 NS_05.

| Modulation/Waveform |  | CBW = 10 MHz, Fc < 1929.43 MHz |  | CBW = 15 MHz, 1927.5 MHz <= Fc < 1932.5 MHz |  | CBW = 15 MHz, 1932.5 MHz <= Fc < 1936.70 MHz |  | CBW = 20 MHz, Fc < 1944.07 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A1 | A2 | A1 | A2 | A1 | A2 | A1 | A2 |
| DFT-s-OFDM | PI/2 BPSK | 2 | 1.5 | 2 | 2.5 | 1 | 1.5 | 2 | 2.5 |
|  | QPSK | 3 | 2 | 3 | 3 | 1.5 | 2 | 3 | 3 |
|  | 16 QAM | 3 | 2 | 3 | 3.5 |  | 2 | 3 | 3.5 |
|  | 64 QAM | 3 |  | 3 | 3.5 |  |  | 3 | 3.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 4 | 3 | 4 | 5 |  | 3 | 4 | 5 |
|  | 16 QAM | 4 | 3 | 4 | 5 |  | 3 | 4 | 5 |
|  | 64 QAM | 4 |  | 4 | 5 |  |  | 4 | 5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |

Proposal 2: RAN4 shall consider defining A-MPR for n1 PC2 NS_05 as shown in tables 3 and 4.

Table 3: The proposed A-MPR regions for n1 PC2 NS_05.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 1929.37 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
|  |  | ≤ 0.18 | > 2.7, ≤ 3.6 | A3 |
| 15 MHz | Fc < 1932.5 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | > 0, ≤ 3.26 | ≤ 3.42 | A2 |
|  |  | 0 | ≤ 1.62 | A4 |
|  | 1932.5 ≤ Fc < 1936.69 | ≤ 1.26 | ≥ 1.0*12*SCS*RBstart + 10.2 | A5 |
|  |  | ≤ 1.44 | ≤ 1.8 | A2 |
| 20 MHz | Fc < 1943.87 | ≥ 0 | ≥ 1.0*12*SCS*RBstart + 4.7 | A1 |
|  |  | ≤ 1.62 | ≤ 3.6 | A4 |
|  |  | > 1.62, ≤ 4.68 | ≤ 3.6 | A2 |
|  |  | ≤ 0.9 | > 4.14, < 1.0*12*SCS*RBstart + 4.7 | A6 |
|  |  | > 1.8, ≤ 3.42 | > 4.14, ≤ 5.76 | A7 |

Table 4: The proposed A-MPR values for n1 PC2 NS_05.

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | PI/2 BPSK | 2.5 | 1.5 |  | 3 | 1.5 | 1 |  |
|  | QPSK | 4 | 2 | 1.5 | 3.5 | 2.5 |  | 1 |
|  | 16 QAM | 4 | 2.5 |  | 4 | 2.5 |  |  |
|  | 64 QAM | 4 | 3 |  | 4 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 5 | 4 |  | 5.5 | 3.5 |  | 2.5 |
|  | 16 QAM | 5 | 4 |  | 5.5 | 3.5 |  |  |
|  | 64 QAM | 5 | 4 |  | 5.5 |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |

# 5	References

[1] R4-2419197, n1 Japan A-MPR simulations and proposal, Nokia

[2] R4-2420481, WF on PHS protection update, KDDI, OPPO

[3] R4-2415439, NR A-MPR reevaluation scenarios on n1 for Japan, KDDI, NTT Docomo

# 6	Appendix: A-MPR triangles

### 6.1	PC2 A-MPR proposal