3GPP TSG-RAN WG4 Meeting #114	 R4-2500460

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.28.2

Source: 	Apple

Title:	Discussion of RRM impact of XR for NR Phase 3

Document for:	Discussion

# Introduction

RRM impact of XR for NR Phase 3 has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

## Issue 1-1: Deployment scenarios

< Agreement>:

- EN-DC and NE-DC for DCI-based solution

  - Is NOT supported for measurement gap skipping by the WID

<Way forward>:

- Further discussion needed for

  - FR2-2

  - NR-DC

We don’t have strong view on FR2-2 but it would be good to have it in the scope. Regarding DC, if only dynamic solution is supported in this work item, it would be quite challenging to support DC. Note that MG sometimes apply to both MCG and SCG. When MN/SN uses DCI to cancel some MG occasions,  SN/MN cannot realize that timely.

Observation 1: if only dynamic solution is supported, it could be challenging to enable it in DC scenario.

If RRC based solution is supported, then UE under DC scenario can also enjoy this feature since RRC based solution is stable and can be easily exchanged between MN and SN.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions

<Way forward>:

- Option 1: Skipping of measurements without gaps with scheduling restrictions are supported

  - Option 1a: Support skipping of measurements without gaps

  - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band

- Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported

  - Option 2a: Measurement skipping does not apply for measurements without gaps

  - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

## Issue 1-4: L1 measurements with scheduling restrictions and procedures

<Way forward>:

- Option 1: Support skipping of L1 measurements and procedures.

  - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

  - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

  - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

- Option 2: Not to support or deprioritize skipping of L1 measurements and procedures.

  - Option 2a: Measurement skipping does not apply for L1 measurements and procedures

  - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

We would like to highlight that intra-frequency handover is the most typical handover in real deployment, wherein UE is not configured with measurement gap. Therefore, if we only focus on the case with measurement gap configured, we cannot benefit from this feature in most typical scenario.

Observation 2: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

From feasibility point of view, we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions like power saving. It is straightforward to us that at least we can let NW keep scheduling UE, as necessary for XR traffic, when UE is not actually performing RRM measurement, RLM and BFD due to relaxation. Besides, we can also consider exchanging measurement order information between UE and NW so that NW can know whether there is scheduling restriction on each measurement occasion, when there are multiple carriers to measure. Thereby, NW can perform measurement scheduling restriction cancellation precisely.

Especially when after gap cancellation, there is still scheduling restriction due to SMTC or other L1-RS. Without any optimization, UE is still expected to perform RRM measurement on those SSB(s) within SMTC, which is overlapping with gap that is cancelled due to XR traffic.

However, if we can support measurement cancellation also on SMTC, then legacy scheduling restrictions do NOT apply on the SSB to measure within SMTC and we can enjoy data after gap cancellation.

Observation 3: after gap cancellation, if the SMTC needs to be measured with scheduling restriction, UE still cannot be scheduled with data. We cannot benefit from gap cancellation in this case.

Observation 4: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

Observation 5: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement.

Proposal 2: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap. Introduce an optional UE capability to support skipping of measurements without gaps.

Proposal 3: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. Introduce an optional UE capability to support skipping of RLM and BFD.

## Issue 2-2: Types of measurement gaps

<Way forward>:

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table

| Gap types | First priority | Second priority | Don’t include |
| --- | --- | --- | --- |
| Type 1 |  |  |  |
| Type 2 |  |  |  |
| NCSG |  |  |  |
| MUSIM |  |  |  |
| Positioning |  |  |  |
| Rel 18 concurrent gap |  |  |  |
| PreConfigured |  |  |  |

We continue supporting the following types:

- Type-1 gap

- Type-2 gap

- NCSG

- concurrent gaps

- pre-MG

Proposal 4: priorities of each gap type:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | √ |  |  |
| Type 2 | √ |  |  |
| NCSG | √ |  |  |
| MUSIM |  | √ |  |
| Positioning |  |  | √ |
| Rel 18 concurrent gap | √ |  |  |
| PreConfigured | √ |  |  |

## Issue 3-1: How to capture impact of measurement skipping

<Agreement>:

- Reuse existing measurement accuracy requirements.

- RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping

  - For the delay requirements, FFS whether and how to extend the delay

<Way forward>:

Proposals related to measurement delay requirements:

  - Option 1: Proposals without distinction of XR and legacy requirements

    - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended.

    - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

    - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

    - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

  - Option 2: Proposals with distinction of conditions where measurement impact is not needed

    - Option 2a:

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

          - FFS how to extend.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions and necessary UE assistance information.

    - Option 2b:

      - Apply legacy measurement delay and accuracy requirements for the following condition:

        - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI

      - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

It is straightforward that measurement cancellation has negative impact on measurement. In order to maintain the same measurement performance, UE still needs same number of measurement samples in physical layer. Assuming during the measurement period, network cancels X measurement occasions, then the total measurement delay shall be extended by X measurement occasions. Take inter-frequency measurement with gap for example:

Note that if too many measurement occasions are cancelled on target carrier (UE cannot measure it for a long time), UE may need to adjust AGC and timing again before it can perform measurement on the carrier successfully. From UE measurement point of view, this is similar to measurement cancellation due to LBT failure. Similarly, max number of occasions shall be defined within a measurement period. If actual number of cancelled occasions exceeds the maximum value, UE shall be allowed to restart a measurement period.

Besides, another point we are considering is whether and how to guarantee mobility performance on some particular carriers, e.g. carrier(s) with high priority like PCC. One potential solution is to assign more measurement opportunities to carriers with high priority, such that legacy measurement requirements can still apply on those carriers. The impact is on other carriers, on which measurement delay may be further extended.

Proposal 5: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 6: max number of occasions cancelled during a measurement period shall be defined.

## Issue 5-1: General on UAI

<Way forward>:

- Companies are tasked to discuss the need and feasibility of UAI, based on the following options

- Proposals related to measurement occasions

  - Option 1: Define UAI related to measurement occasions

    - Option 1a: number of occasions which can be skipped

    - Option 1b: number of measurement occasions that are needed for measurement

    - Option 1c: UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 1d: patterns of gaps positions of MG that can be skipped

    - Option 1e: study how UE assistance information/UE capability is reported when DRX is configured.

    - Option 1f: Maximum number of consecutive MGs that can be skipped

  - Option 2:

    - Option 2a: Do not define UAI related to measurement occasions.

    - Option 2b: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution

    - Option 2c: RAN4 will not specify any XR-related requirement, based on XR UAI.

  - Option 3: The information about gaps not used for measurement are reported as UE capability.

  - Option 4: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions.

- Proposals related to channel conditions

Option 1: For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

The last thing we would like to discuss is about UE assistance information. UE knows better about measurement performance. For instance, when UE is in low mobility or not-at-cell-edge, UE can tolerate more measurement cancellation. When UE is at cell edge, UE doesn’t expect too many measurements cancellation. A simple solution is to let UE indicates NW a status whether it can tolerate measurement cancellation.

Proposal 7: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

# Conclusion

In this contribution, we provide discussion on RRM impact of XR for NR phase 3. After discussion, the following conclusions are provided:

Observation 1: if only dynamic solution is supported, it could be challenging to enable it in DC scenario.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

Observation 2: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

Observation 3: after gap cancellation, if the SMTC needs to be measured with scheduling restriction, UE still cannot be scheduled with data. We cannot benefit from gap cancellation in this case.

Observation 4: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

Observation 5: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement.

Proposal 2: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap. Introduce an optional UE capability to support skipping of measurements without gaps.

Proposal 3: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. Introduce an optional UE capability to support skipping of RLM and BFD.

Proposal 4: priorities of each gap type:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | √ |  |  |
| Type 2 | √ |  |  |
| NCSG | √ |  |  |
| MUSIM |  | √ |  |
| Positioning |  |  | √ |
| Rel 18 concurrent gap | √ |  |  |
| PreConfigured | √ |  |  |

Proposal 5: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 6: max number of occasions cancelled during a measurement period shall be defined.

Proposal 7: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

# Reference

- R4-2420103, WF on RRM requirements for XR_Ph3, Nokia