3GPP TSG-RAN WG4 Meeting #114	R4-2500466

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	RRM requirements for UE to UE CLI

Agenda item:	7.23.3.1

Document for:	Discussion

1. Introduction

In the last RAN4 meeting, the following agreements were made in the WF [1]:

Sub-topic 1-1: RRM impacts of UE-to-UE CLI handling Issue 1-1-2: Measurement methods - Agreements (from Thu AH session) - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions. Issue 1-1-3: Rx beam - Agreements (from Thu AH session) - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec. - Rx beam sweeping is not needed for L1 CLI measurement. Issue 1-1-6: Measurement period - Agreements (from Wed AH session) - For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement - For L1-CLI-RSSI, - Confirm measurement period requirements are defined based on 1 sample, and - Clarify in the requirements that the measurement is based on DL timing. Issue 1-1-9: Scheduling and measurement restriction - Agreements (from Thu AH session) - For scheduling restriction, - For L1-SRS-RSRP, - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing. - For measurement restriction, - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

2. Discussion

  - Measurement Methods

In RAN1 #119, it has been agreed that Method #3 for measuring UE-to-UE CLI is supported. This new method should be considered together with the already agreed Method #1 and Method #2 by RAN4.

Agreement (RAN1#119) For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

RAN4 shall specify L1 CLI measurement requirements for Method #3, in addition to the measurement requirements for Method #1 and Method #2.

Both Method #1 and Method #3 refer to CLI RSSI measurements. Currently, there are no measurement restrictions explicitly specified for L3 CLI-RSSI measurements, only scheduling restrictions, which are defined in TS 38.133 Clause 9.7.4. But with the following text in clause 9.7.3.5 below, the L3 CLI-RSSI is assumed deprioritized comparing to other L3 and RLM/BFD/CBD or L1-RSRP measurements.

TS38.133 clause 9.7.3.5

If the CLI-RSSI measurement resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TCLI_RSSI_measurement_period.

In R19, the CLI-RSSI is a L1 measurement. Similarly, as for L1-RSRP/L1-SINR measurement, the measurement restriction shall be defined for CLI-RSSI. The RAN1 agreement states that the UE cannot be configured to perform measurement using Methods #1 and #3 in the same OFDM symbol. This needs to be captured in the measurement restriction for L1-CLI-RSSI measurement. Therefore, our view is that a measurement restriction shall be defined at least in that case.

RAN4 shall specify measurement restrictions for L1-CLI-RSSI measurements in the DL subband i.e. Method #2 or in the UL subband i.e. Method #3.

Additionally, there is a need to consider the measurement restrictions when the CLI-RSSI measurement is configured in the same symbol as the SRS-RSRP, in method #2, or when it is configured in the same symbol as the RS for RLM, BFD, CBD, L1-RSRP or L1-SINR, or when it overlaps with measurement gaps.

Our view is that the L3 CLI-RSSI requirements can be used as baseline for the L1-CLI-RSSI measurement requirements, i.e., the L1-CLI-RSSI measurement requirements are not specified in case of overlap with the signals mentioned above.

L1-CLI-RSSI measurement requirements are not specified when the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with configured SRS-RSRP for method #2, SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

Moreover, there are differences between Method #1 and Method #3 regarding the side conditions. Comparing to Method #1, the UE is likely to measured higher interference power when measuring L1-CLI-RSSI in Method #3, hence a different side condition is expected to achieve the same measurement performance.

RAN4 shall specify different side conditions for L1-CLI-RSSI measurement in Method #1 and Method #3.

Regarding the SCS used for the CLI-RSSI measurements in methods #1 and #3, we would like to discuss RAN1 agreements and confirm our understanding in RAN4.  In the Rel-19 Duplex Evo work item description, we have the the following assumption for SBFD operation:

- SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

Our interpretation of the assumption above is that even though a BWP pair is configured, the SCS, centre frequency and cyclic prefix are the same, as in TDD. Otherwise, the symbol length and slot definition would be different between the DL and UL BWPs, resulting in different slot durations in UL and DL subbands.

In TS 38.133, clause 9.7.3.1, for L3 CLI-RSSI measurement we have that:

The subcarrier spacing for CLI-RSSI measurement resource configuration can be same or different from the subcarrier spacing of active BWP. UE shall perform CLI-RSSI measurement with the SCS of the active BWP.

Since the CLI-RSSI measurements in method #1 and method #3 are performed in the DL subband (DL active BWP) and in the UL subband (UL active BWP) respectively, we believe that the same principle above can be applied to both Method #1 and Method #3.

The UE shall perform CLI-RSSI measurement with the SCS of the DL active BWP when measuring in method #1 and in method #3.

  - Rx beam

Regarding the assumptions on the UE Rx beam to perform the L1 CLI measurements, RAN1 has agreed on the following:

Agreement (RAN1#118-bis) For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources Agreement (RAN1#119) For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

In RAN4 #113, this issue has also been captured in the WF:

Issue 1-1-3: Rx beam - Agreements (from Thu AH session) - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec. - Rx beam sweeping is not needed for L1 CLI measurement.

In the Rel-16 CLI measurements requirements, the FR2 UE assumes that the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET. As shown above, in Rel-19 Duplex evolution, RAN1 has agreed that the gNB can optionally configure the UE with a given TCI state for the CLI measurement, either by configuring a list of TCI states or configuring a unifiedTCI-StateType. Otherwise, the assumption from Rel-16 CLI measurements will be used by the UE. In all the cases above, RAN4 has agreed in RAN1 #113 that no Rx beam sweeping is needed for the L1 CLI measurement. Therefore, we believe that the same measurement period can be defined in all cases, disregarding how the UE will determine the Rx beam.

The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

  - Measurement resources

The following agreements/conclusions related to measurement resources were reached by RAN1:

Agreement (RAN1#118-bis) For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. Conclusion (RAN1#119) For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported. Conclusion (RAN1#119) There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

Given the RAN1 agreements above, a UE configured with CLI-RSSI measurement resources is to report a single wideband CLI-RSSI value. It is also noted that, in case the CLI-RSSI measurement resources span across two DL subbands, the UE shall exclude the frequency resources outside the DL usable PRBs, i.e. the PRBs dedicated to UL subband and guardbands, if configured. Moreover, no discussions are needed in RAN4 about subband CLI-RSSI reporting since RAN1 reached a conclusion that this feature will not be supported in Rel-19. For Method #3, RAN4 should also assume wideband reporting. RAN4 should discuss whether the legacy CLI-RSSI measurements accuracy requirements are applicable for L1 CLI-RSSI measurements.

Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI.

RAN4 should discuss whether the legacy CLI-RSSI measurements accuracy requirements are applicable for L1 CLI-RSSI measurements.

  - Side conditions

Timing offset for L1-SRS-RSRP

Given the reply from RAN1 in [2], intra-cell measurements can be prioritized for the requirements definition in RAN4. If the intra-cell scenario is agreeable as the starting point, the cell phase error can be zero as victim and aggressor UEs are assumed in the same cell. As the propagation delay difference is derived assuming the victim and aggressor UEs are located in the cell center and cell edge respectively, it remains the same irrespective of intra-cell or inter-cell. Hence the residual timing error can be derived assuming cell phase error is zero and same cell size as in R16.

For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

SRS Es/IoT

The SRS Es/Iot value may be higher considering the L1-SRS-RSRP measures the interference on UL subband where there is no other DL transmission hence less interference can be expected in the victim UE as compared to existing SRS-RSRP measurements. Current simulation assumptions for L1-SRSP RSRP measurement requirements cover SRS SNR values of 1 dB, 2 dB and 3 dB. We think RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

SRS bandwidth

Current simulation assumptions for L1-SRSP RSRP measurement requirements assume SRS bandwidths of 24 PRBs and 48 PRBs. Note that existing SRS-RSRP measurement requirements assume a SRS bandwidth of 48 PRBs. Based on our results, we conclude that the measurement reports show very similar performance with both bandwidth values. Given that certain SBFD configurations might have a limited UL subband size, we prefer that L1-SRS-RSRP measurement requirements should consider a SRS bandwidth of 24 PRBs.

The SRS bandwidth of 24 PRBs is assumed for defining L1-SRS-RSRP measurement requirements.

  - Measurement period

On the measurement period, the WF from RAN4#113 captured the following:

- Recommended WF - For L1-SRS-RSRP, further discuss the following options based on simulation results. - Option 1: single set of requirements based on [1] sample - Option 2: two sets of requirements based on [1] and [3] samples based on NW configuration, e.g. timeRestrictionForChannelMeasurement - Other options are not precluded - For L1-CLI-RSSI, - Confirm measurement period requirements are defined based on 1 sample, and - Clarify in the requirements that the measurement is based on DL timing.

In the last meeting, it has been discussed whether the L1-SRS-RSRP requirements are to be specified with 1 sample measurement (Option 1) or 1 and 3 samples according to NW configuration (Option 2).

The existing Rel-16 SRS-RSRP requirements consider 3 samples. This is mainly due to the nature of the Layer 3 measurements. For Layer 1 measurements, the L1-SRS-RSRP measurement period requirements can be defined separately for 1 and 3 samples. The network can indicate as part of the resource configuration if 1 sample (timeRestrictionForChannelMeasurement = configured) or 3 samples (timeRestrictionForChannelMeasurement = notConfigured) should be considered.

The L1-SRS-RSRP measurement period is separately defined according to 1 and 3 samples as configured by network.

  - Measurement reporting

RAN1 has agreed on supporting aperiodic reporting while periodic and semi-persistent reporting are still under discussion. We suggest that RAN4 focuses on aperiodic reporting and waits for RAN1 final decision about whether periodic and semi-persistent reporting shall be supported.

RAN4 to focus on aperiodic reporting while waits for RAN1 to decide on whether periodic and semi-persistent reporting shall be supported.

  - Report mapping

Regarding the L1-SRS-RSRP and L1-CLI-RSSI reporting, RAN1#119 agreed to introduce 2 new reporting quantities in the CSI-ReportConfig and to support absolute and differential reporting:

Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported - The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4} - The M CLI measurement resource ID(s) are reported - The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported - When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference - FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP - FFS: whether/how to report differential value in case the largest measurement is out of range

Current Rel-16 SRS-RSRP report mapping supports absolute power reporting with a reporting range from -140 dBm to -44 dBm with a resolution of 1 dB. The L3 CLI SRS-RSRP report mapping table from TS 38.133 Section 10.1.22.1. can be re-used for the L1-SRS-RSRP measurements. Since in R19 the CLI SRS-RSRP is a L1 measurement, differential L1-SRS-RSRP reporting shall also be considered. For the differential L1-SRS-RSRP reporting, which as shown in above was agreed by RAN1, we suggest reusing the same principle as the current L1-RSRP differential reporting (Table 10.1.6.1-2, TS 38.133). The differential L1-SRR-RSRP report consists then of 4 bits with 2 dB step size.

Re-use the existing Rel-16 SRS-RSRP report mapping for L1-SRS-RSRP absolute power reporting.

Introduce a new report mapping table for L1-SRS-RSRP differential power reporting. Consider the same report mapping table as for SS-RSRP and CSI-RSRP differential reporting.

Regarding the L1-CLI-RSSI reporting, we think that RAN4 shall first discuss whether different reporting values are needed for Method #1 and Method #3, respectively. In our opinion, the current CLI-RSSI report mapping table covers a large range of interference power, and therefore it can be applicable for both measurement methods. Note that current reporting range spans from -100 dBm to -25 dBm with 1 dB resolution. Similarly, as for L1-SRS-RSRP, a new report mapping table for differential L1-CLI-RSSI needs to be introduced.

Re-use the existing Rel-16 CLI-RSSI report mapping for L1-CLI-RSSI absolute power reporting.

The same report mapping table is applicable to both Method #1 and Method #3.

Introduce a new report mapping table for L1-CLI-RSSI differential power reporting. Consider the same report mapping table as for SS-RSRP and CSI-RSRP differential reporting.

  - Requirement applicability

In R16 CLI measurements, the measurement requirements apply only when the measurement resource is fully confined within active BWP. While SBFD operation is considered in R19, the SRS-RSRP and CLI-RSSI in Method#3 will be configured in UL subband. However, the UL subband is also considered within DL active BWP. Hence the existing applicability conditions in R16 are still applicable to R19 L1 CLI measurement.

TS 38.133 clause 9.7.1:

CLI measurements are only applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

Conclusion

The following Observations and Proposals were made:

- RAN4 shall specify L1 CLI measurement requirements for Method #3, in addition to the measurement requirements for Method #1 and Method #2.

- RAN4 shall specify measurement restrictions for L1-CLI-RSSI measurements in the DL subband i.e. Method #2 or in the UL subband i.e. Method #3.

- L1-CLI-RSSI measurement requirements are not specified when the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with configured SRS-RSRP for method #2, SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

- RAN4 shall specify different side conditions for L1-CLI-RSSI measurement in Method #1 and Method #3.

- The UE shall perform CLI-RSSI measurement with the SCS of the DL active BWP when measuring in method #1 and in method #3.

- The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

- Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI.

- RAN4 should discuss whether the legacy CLI-RSSI measurements accuracy requirements are applicable for L1 CLI-RSSI measurements.

- For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

- RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

- The SRS bandwidth of 24 PRBs is assumed for defining L1-SRS-RSRP measurement requirements.

- The L1-SRS-RSRP measurement period is separately defined according to 1 and 3 samples as configured by network.

- RAN4 to focus on aperiodic reporting while waits for RAN1 to decide on whether periodic and semi-persistent reporting shall be supported.

- Re-use the existing Rel-16 SRS-RSRP report mapping for L1-SRS-RSRP absolute power reporting.

- Introduce a new report mapping table for L1-SRS-RSRP differential power reporting. Consider the same report mapping table as for SS-RSRP and CSI-RSRP differential reporting.

- Re-use the existing Rel-16 CLI-RSSI report mapping for L1-CLI-RSSI absolute power reporting.

- The same report mapping table is applicable to both Method #1 and Method #3.

- Introduce a new report mapping table for L1-CLI-RSSI differential power reporting. Consider the same report mapping table as for SS-RSRP and CSI-RSRP differential reporting.

- The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

References

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei

- R1-2410890, “Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation”