3GPP TSG-RAN WG4 Meeting #114	R4-2500595

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.28.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirements for XR

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open items and it is summarized in WF [1]. This document proposes RRM core requirements for XR.

- Discussion

During previous meeting, the necessity of consideration for scheduling restrictions on the measurement without gap was discussed.

Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Skipping of measurements without gaps with scheduling restrictions are supported - Option 1a: Support skipping of measurements without gaps - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported - Option 2a: Measurement skipping does not apply for measurements without gaps - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction. Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures. - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures. - Option 2a: Measurement skipping does not apply for L1 measurements and procedures - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact. Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM - Option 3: Only L1 measurements for LTM within gaps are supported for gap skipping. - Option 4: For L1 measurement for mobility, apply same decision as normal L1 measurement.

In previous meeting, some companies mentioned that the performance gain is small for these scenarios because they only affect few symbols. However, even if one symbol is restricted, throughput degradation is large especially in 100MHz CBW band.

Proposal 1: At least for 100MHz CBW band, support skipping of measurements for L3 measurements without gaps

Proposal 2: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

Proposal 3: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

- Conclusion

In this contribution, we proposed our views on RRM core requirements for XR.

Proposal 1: At least for 100MHz CBW band, support skipping of measurements for L3 measurements without gaps

Proposal 2: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

Proposal 3: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

References

[1] 3GPP, R4-2420103, “WF on RRM requirements for XR_Ph3”, Nokia