3GPP TSG-RAN WG4 Meeting # 114	R4-2500617

# Athens, GR, 13th – 17th Feb., 2025

Source: 	Xiaomi

Title: 	Discussion on test framework and procedure for AI beam management

Agenda Item:	7.20.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability issues for beam management.

- Test motivation - Channel modelling - Test procedure

- Discussion

      - 2.1 Test motivation

Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs.

In last meeting, there is lots of discussions about channel model, e.g. single AOA or multiple AoAs.

Some company mentioned that without multiple AoA, UE RX beam sweeping functionality can’t be verified. On the other hand, some company mentioned that the main test motivation is to verify the AI algorithm and single AOA channel is enough to generate the emulated received signal.

If AI model can achieve final prediction accuracy, it involves two steps:

- Input data of AI model achieve accuracy requirement

- AI model algorithm achieve prediction requirement

From our understanding, it’s better to split the two parts in testing, otherwise, it’s hard to distinguish UE fails in which step. The total test includes two parts:

- Part 1: Verify input data accuracy of model AI based on RX beam sweeping

- Part 2: Verify AI model algorithm

For Part 1, it’s related to measured RSRP accuracy based on RX beam sweeping. This is not a new issue for AI. While for legacy beam management, it also has the similar issue. However, in legacy, the RX and TX beamforming gain has been transformed to equivalent SNR. RSRP accuracy is tested under certain SNR level. No RSRP accuracy requirement for RX beam sweeping is defined. For AI, if necessary, the RX beam sweeping performance can be tested separately, it can further discuss whether needs to test this functionality.

For Part 2, the emulated data can be used as input for AI model, then AI model prediction accuracy can be tested. At least we need to test AI model prediction algorithm first as it’s the core part.

Proposal 1: For OTA test, prefer to split the test motivation into two parts, RAN4 to at least test part 1 and further discuss whether to test part 2.

- Part 1: Verify AI model prediction algorithm

- Part 2: Verify input data accuracy of model AI based on RX beam sweeping

      - 2.2 Channel model

For AI algorithm performance test, it needs to discuss how to emulated data and which channel model is needed. there are several possible ways to emulate input data of AI model:

Case 1: if RX beam patten is known at TX side.

  - Since the emulated signal is after UE RX beamforming, single AOA is assumed, then the signal shall consider both TX and RX beamforming gain:

i=1NPT,iCiPR,i

Where N is number of multi-path signals. PT,i is Tx beamforming gain for ith-path signal. Ci is power of ith-path signal. PR,i is Rx beamforming gain for ith-path signal. It means that TX side needs to know the UE RX beam forming codebook in advance.

For this method, no simplified channel is needed as TX side can generate all kinds of channels. On the other hand, we would like to mention one thing that UE needs to apply omi-directional RX antenna to receive the emulated signal. it’s FFS whether omi-directional antenna can be applied in FR2.

Observation 1: If UE RX beam pattern can be known at TX side, it’s possible to use single AOA to emulate input data for AI model, the emulated data will be signal after UE RX beamforming. No simplified channel model is needed.

Case 2: If RX beam pattern is not known at TX side

  - The emulated signal is before UE RX beamforming, there are several options for channel model:

    - Option 1: Single AOA. Not reduce path number, but combine all TX signal by a fixed RX beamforming.

    - Option 2: Single AOA. Reduce multi-path into single-path.

    - Option 3: Multi-AOA. Reduce multi-path number.

For option 1, the emulated signal is before UE RX beamforming, then the signal only considers TX beamforming gain, it will combine all TX path together by multiply TX beamforming gain. Before RX beamforming, multiple TX signal has already be combined into one path. UE needs to receive the combined TX signal by RX beam. the final input data for AI model will be:

PRi=1NPT,iCi

It means that the same RX beamforming gain, PR, is applied for different TX path signals. however, in reality, there will be different RX beamforming gain for different TX path signals. There will be some mismatch between emulated signal and real signals.

Observation 2: if a fixed RX beamforming gain is used for multi-path TX signals, there will be some mismatch between emulated signal and real signals.

For Option 2,3, it’s related to channel simplification. Multi-path number can be reduced to single path or fewer paths. The following figure compare the L1-RSRP difference between different TX beam index with single path and 3 paths, the detail configuration and analysis can be found in[1]. By our simulation, the less the path number, the larger gap between different beams, which means that AI prediction performance will be relaxed. It’s more easiler for AI to distingwish the best Top-K TX beam.

(a) Simplified CDL-C with 8 cluster, 3 AOA         (b) Simplified CDL-C with 3 cluster, 1 AOA

Fig.1 CDF curve L1-RSRP difference between CDL-C and simplified CDL-C

Observation 3: For simplified channel model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 2: For channel modelling type in AI algorithm performance test, RAN4 to discuss whether UE RX beam pattern can be known in TX side first.

      - 2.3 Test procedure

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.2 test procedure

As shown in Fig.2, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.3.1 Ground truth for best beam index

If the test metric is beam index prediction accuracy, UE will report the best beam index in set A after measurement. However, as we analysed in performance metric part in [2], UE may not distinguish for the best 1st beam index and 2nd best beam index due to small L1-RSRP delta between the best 1st beam index and 2nd best beam index.

Fig.3 Ideal L1-RSRP difference for 32 TX beams

In Fig.4, we take the CDF curve “RSRP delta between 1st and 2nd best beam” as an example. 10 percentile of CDF curve shows the L1-RSRP delta between 1st and 2nd best beam is smaller than 0.16dB for 10% cases. If UE can successfully distinguish between the 1st and 2nd beams in 90% case, the measurement accuracy has to be within 0.16dB.

Therefore, UE may need to report for N best beam index. RAN4 can further discuss how to decide N.

Proposal 3: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A. RAN4 to discuss how to decide number N.

### 2.3.2 SNR impact

Same as SNR difference analysed for set B, there will be great SNR difference for 32 TX beams in set A either. If SNR for the worst TX beam is set as 10dB, then SNR for best TX beam will be too high. There may be some test issue. Suppose that if SNR of best several TX beams is set as 10dB, SNR for the worst 10 TX beam can be as low as -15dB. There will be great measurement error for the worst beam.

For set A, UE only needs to identify several best beam indexes and/or corresponding L1-RSRPs, UE doesn’t need to measure for all beams accurately. The reason is that since the L1-RSRP delta between best beam and worst beam is relatively large, the measured L1-RSRP for worst beam is still quite small, which will not exceed the value of L1-RSRP of best TX beams.

Therefore, for deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Observation 4: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Therefore, RAN4 needs to discuss how to set SNR condition for measurement of set A to derive ground truth.

Proposal 4: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

### 2.3.3 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 5: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 6: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.3.4 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 5: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.4(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index as shown in Fig.4(b). In this way, UE can only honestly report the best measured beam in T2.

Fig.4 (a) beam index and RS index mapping relation for set A in T1

Fig.4 (b) beam index and RS index mapping relation for set A for T2

Proposal 7: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Conclusion

In this contribution, we provide the following proposals:

Proposal 1: For OTA test, prefer to split the test motivation into two parts, RAN4 to at least test part 1 and further discuss whether to test part 2.

- Part 1: Verify AI model prediction algorithm

- Part 2: Verify input data accuracy of model AI based on RX beam sweeping

Observation 1: If UE RX beam pattern can be known at TX side, it’s possible to use single AOA to emulate input data for AI model, the emulated data will be signal after UE RX beamforming. No simplified channel model is needed.

Observation 2: If a fixed RX beamforming gain is used for multi-path TX signals, there will be some mismatch between emulated signal and real signals.

Observation 3: For simplified channel model, L1-RSRP delta between different TX beams will increase if cluser number and AOA number is further reduced. It’s more easiler for AI to distingwish the best Top-K TX beam. The AI prediction requirement will be relaxed.

Proposal 2: For channel modelling type in AI algorithm performance test, RAN4 to discuss whether UE RX beam pattern can be known in TX side first.

Proposal 3: If the test metric is beam index prediction accuracy, UE may need to report ground truth of N best beam index for set A. RAN4 to discuss how to decide number N.

Observation 4: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Proposal 4: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

Proposal 5: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 6: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Observation 5: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

Proposal 7: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Reference

- R4-2417948, Discussion on testability and interoperability issues for beam management, Xiaomi

- R4-2417949, Discussion on core part requirement for AI beam management, Xiaomi