# 3GPP TSG-RAN WG4 Meeting #114	R4-2500629

# Athens, GR, 17th – 21st February, 2025

Source: 	Xiaomi

Title: 	Discussion on RRM core requirements for LP-WUS/WUR

Agenda Item:	7.26.4.2

Document for:	Discussion

# 1Introduction

In RAN4#113 meeting, the RRM impacts on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was discussed and a WF was approved [1]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Based on the discussion in previous meetings, RRM measurement relaxation case#1 and case#3 has already be agreed. In case 1, i.e., fully offloading case, MR stops perform any RRM measurement and LP-WUR performs serving cell measurement. There is still FFS on the condition and details for the MR neighboring cell measurement. From our perspective, this is the case where UE could perform the greatest level of relaxation measurement. We prefer not to consider any neighbor cell measurement in this case.

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case 1, except for the higher priority frequency layer search.

In case 2, MR has RRM relaxation on serving cell measurement (no neighbour cell measurement is needed) and LP-WUR performs serving cell measurement. We think this is the case UE in the cell center, when UE does not need to measure neighbor cell, as well as no higher priority inter-f or inter-RAT configured. From our perspective, this is the typical case for starting LP-WUR monitoring and no need to start MR measurement. So, we prefer to de-prioritize this case.

Proposal 2: It is suggested to de-prioritize RRM relaxation case 2.

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

As we understand that the LP-WUR measurement is initially introduced to guarantee the performance of LP-WUS monitoring, we think the measurement is not expected to start later than the LP-WUS monitoring. Also, we see little advantage in early starting of the LR measurement. If UE still use MR for paging reception, it is reasonable to use MR for RRM measurements as MR also need measurement for T/F synchronization. In this case, the LR measuremnt results seems useless. So, we prefer to define the same entry/exit condition of LP-WUR measurement and LP-WUS monitoring. For the entry/exist conditions for LP-WUS monitoring, we understand that this issue is under discussion in RAN1 and RAN2. We think RAN4 should wait for RAN1/2 progress. RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by other working group(s) if needed.

Proposal 3: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1/2 if needed.

RAN4 also discuss the entry/exit conditions for MR RRM measurement relaxation. Based on our analysis in the previous section, we think it is more reasonable that the MR RRM relaxation is executed when LR is used for serving cell measurement. If the LR is not doing measurement, the evaluation of measurement relaxation can only depend on serving cell measurement in MR, which is aligned with Rel-16 power saving functionality. If the LR is doing RRM measurement while MR is also doing normal RRM measurement, there seems little benefit but more power consumption. So, from our perspective, the entry/exit conditions for MR RRM measurement relaxation should also be the same as LP-WUR serving cell measurement. However, we understand RAN2 is currently working on the criteria design, it is suggest RAN4 to focus on the relaxation cases.

Proposal 4: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring is under discussion. In our understanding, this issue is related to the enter/exist conditions for LP-WUS monitoring for LR as well as RRM measurement relaxation for MR. Here below, the entry/exit conditions based on existing RAN2’s agreements summarized in last meeting’s email discussion was attached:

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | FFS | FFS |

RAN2 agreed that NW can optionally configure thresholds for both MR and LR measurements. In this case, the entry condition is met when all the measured results are above the configured threshold(s). We understand the LP-WUR status should be ON as UE need to obtain LR measurement results to evaluate the entry condition. However, considering the UE implementation issue, we agree with the recommendation in WF that “LP-WUR status is ON for serving cell measurement, but when, how to turn on LR and the duration of “ON” for serving cell measurement is up to UE implementation, and no LP-WUR requirement will be applied”.

Proposal 5: LP-WUR status is ON for serving cell measurement, but when and how to turn on LR and the duration of “ON” for serving cell measurement is up to UE implementation, and no LP-WUR requirement will be applied.

Issue 1-1-11: Considerations on higher priority frequency layer

In accordance with the legacy measurement procedure for MR only UE, measurements for neighbor cell could be divided into two parts. One is for intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority, and the decision for MR to conduct these measurements hinges on the results from the serving cell measurements. When the signal quality of the serving cell reduce to certain threshold, the intra-frequency measurements and NR inter-frequency or inter-RAT frequencies measurements with equal or lower priority will be triggered. The other one is for NR inter-frequency or inter-RAT frequencies with higher priority, and the decision for MR to conduct the measurements depend on NW configuration. In this case, as long as NW configuration indicates the priority information, UE needs to perform measurements of higher priority NR inter-frequency or inter-RAT frequencies.

Observation 1: According to existing cell selection/re-selection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

We think the handling on NR inter-frequency or inter-RAT frequencies measurements with higher priority should also follow the legacy.

- Proposal 6: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case 1 and case 3.

- Detailed LP-WUR requirements

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity - Proposals - P1: RAN4 to define the lower limit for measurement delay requirements in case the periodicity of LP-SS are 80ms or 160ms. (xiaomi) Recommendations: Consider whether to define lower bound on periodicity for OOK based LP-WUR requirements when LP-SS periodicity is 80ms or 160ms. Issue 1-2-3: On requirements for LP-WUR for entry/exit criteria(threshold) evaluation Recommendations: Encourage companies firstly to consider whether to differentiate requirements based on cases Note: For entry condition evaluation requirements, requirements from legacy state to case 1 or case 3 or LP-WUS monitoring will be discussed separately. If No Same requirements will be defined on entry criteria(threshold) evaluation from one case to the other case (excluding scenarios in the note). Same requirements will be defined on exit criteria(threshold) evaluation from one case to the other case. If Yes, consider the following cases one by one Issue 1-2-3a: On requirements for entry/exit criteria(threshold) evaluation for Case 1 Issue 1-2-3b: On requirements for entry/exit criteria(threshold) evaluation for Case 3 Issue 1-2-3c: On requirements for entry/exit criteria(threshold) evaluation for LP-WUS monitoring On detailed evaluation requirements, the following options are available - Option 1: Evaluation requirements for entry/exit evaluation is the same as that of LP-WUR measurement period. - Option 2: Evaluation requirements for entry/exit conditions is longer than that of LP-WUR measurement period. - Option 3: Evaluation requirements for entry evaluation is longer than that of LP-WUR measurement period; evaluation requirements for exit evaluation is the same as that of the LP-WUR measurement period.

During previous discussion, the measurement delay requirement in IDLE/Inactive mode for LP-WUR was divided into two cases. One is for LP-WUR based on LP-SS, the other one is for LP-WUR based on SSB. For the former case, agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS shall be defined based on LP-SS periodicity. For the later case, i.e. LP-WUR based on SSB, it was agreed that the measurement delay requirements are defined based on LO periodicity.

For the case LP-SS is not configured, we understand only OFDM-based LP-WUR can work in this case. The OFDM-based LP-WUR obtains synchronization based on SSB. Basically, we think it is quite similar as MR. Also, the current agreement, i.e. based on LO periodicity, also aligns with measurement delay requirements for MR. As no upper bound has been defined for MR, we prefer not to define upper bound for LR either.

Proposal 7: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

For the case LP-SS is configured, the agreement was achieved that measurement delay requirements for LP-WUR based on LP-SS shall be defined based on LP-SS periodicity. However, based on the latest RAN1 progress, we can see that the range of LP-SS periodicity is still open. There is possibility that the LP-SS periodicity can be configured to 80ms or 160ms, which is comparable to the periodicity of SSB. We prefer to set a lower limit for the measurement delay requirements in case the periodicity of LP-SS can be configured to small value like 80ms or 160ms. Or, if majority companies can accept, we also support to define the measurement delay requirements based on max(LP-SS, LO periodicity).

Proposal 8: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms, RAN4 to

- Option 1: define the lower limit for measurement delay requirements;

- Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

On requirements for LP-WUR for entry/exit criteria(threshold) evaluation, companies’ opinions are diverse. We think it is very likely that the entry/exit criteria(threshold) evaluation for LP-WUS monitoring would be the same as entry/exit criteria(threshold) evaluation for Case 1. But the final decision depends on RAN2. So, we prefer to focus on MR RRM relaxation cases.

Proposal 9: RAN4 to focus on discussing requirements for LP-WUR for entry/exit criteria(threshold) evaluation for MR RRM relaxation, whereas the LR-WUS monitoring requirement discussion pending on RAN2’s progress.

Then, for MR RRM relaxation evaluation requirement except for the exit/entry cases from legacy state to case 1 or case 3, the entry condition for case 3 is the exit condition for case 1, and vice versa. From our view, we prefer not to define additional evaluation requirements, i.e. RAN4 to define the evaluation requirements for entry/exit evaluation the same as that of LR measurement period requirements.

Proposal 10: When not considering the legacy state (i.e. excluding scenarios in the Note), the evaluation requirement for entry/exit evaluation is the same as that of LP-WUR measurement period for both case 1 and case 3.

If we taking the legacy state into consideration, basically, we prefer the same logic that the evaluation requirements is the same as the measurement requirements. For case 1 to legacy state, and case 3 to legacy state, the evaluation requirements is the same as the LP-WUR measurement requirements. For legacy state to case1 or case3, the evalution requirements is the same as the MR serving cell measurement requirements.

Proposal 11: When considering the legacy state (i.e. considering the scenarios in the Note),

- For case 1/ case 3 to legacy state, the evaluation requirement is the same as the LP-WUR measurement requirements;

- For legacy state to case1 or case3, the evaluation requirement is the same as the MR serving cell measurement requirements.

- MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3 Agreement: Scaling factor(s) is/are >= 4. Further down-select from - Option 1: 1 fixed scaling factor - Option 2: multiple values configured by NW - Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo) DRX cycle length [s]; Scaling factor. 0.32; TBD. 0.64; TBD. 1.28; TBD. 2.56; TBD. Issue 1-3-2: On Neighbour cell and serving cell relaxation factor Agreement: Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state.

For MR RRM relaxation in Case3, RAN4 has agreed to apply the same relaxation factors for serving cell measurements and neighbor cell measurements. The main issue left is about the exact number of relaxation factor(s). As this is the scaling factor for UE in RRC_IDLE mode, our initial preference is to define one fixed scaling factor. But considering the big concern from NW side, we can comprise to fixed multiple values, i.e. option3.

Proposal 12: RAN4 to define multiple scaling factor values for MR RRM relaxation in case3, while the value depends on DRX cycle length.

- Accuracy

During discussion in previous meeting, it was agreed to wait RAN1 progress in target SNR/SINR and use the RAN1 agreed target SNR/SINR as baseline for RRM simulation assumption. Meanwhile, companies still have different understanding on whether/how to define the accuracy. From our perspective, we support the idea that the accuracy discussed here is for simulation assumption and no need to define dedicated accuracy requirement in performance section. As per the existing serving cell measurement requirements defined in RRC_IDLE/INACTIVE state, no corresponding measurement accuracy requirement introduced. Since we are discussing RRM requirements for LP-WUR in RRC_IDLE/INACTIVE state, we prefer to follow the same logic as for MR.

Proposal 13: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

- LP-WUR requirements at RRC_CONNECTED mode

In RRC_CONNECTED mode, as clearly described in the objectives, UE RRM/RLM/BFD/CSI measurements are performed by MR. So, we think there should be no RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode expected. However, we are fine not to jumps to conclusions in this early stage.

Proposal 14: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Conclusion

Proposal 1: RAN4 not to consider any neighbor cell measurement in RRM relaxation case 1, except for the higher priority frequency layer search.

Proposal 2: It is suggested to de-prioritize RRM relaxation case 2.

Proposal 3: RAN4’s discussion on entry/exit conditions for LP-WUS monitoring can be triggered by RAN1/2 if needed.

Proposal 4: It's suggested that RAN4 concentrate on discussing relaxation cases, while the entry/exit criteria should be based on RAN2 design.

Proposal 5: LP-WUR status is ON for serving cell measurement, but when and how to turn on LR and the duration of “ON” for serving cell measurement is up to UE implementation, and no LP-WUR requirement will be applied.

Observation 1: According to existing cell selection/re-selection procedure:

- The initiation of intra-frequency measurements, as well as NR inter-frequency or inter-RAT frequency measurements with equal or lower priority, depends on the serving cell measurement results from the MR;

- The initiation of NR inter-frequency or inter-RAT frequencies measurements with higher priority depends on NW configuration.

- Proposal 6: When NR inter-frequency or inter-RAT frequencies measurements with higher priority are configured by NW, MR perform relaxed higher priority frequency layer search in RRM relaxation case 1 and case 3.

Proposal 7: RAN4 not to define upper bound for the measurement delay requirements for LP-WUR when LP-SS is not configured.

Proposal 8: In case the periodicity of LP-SS can be configured to small value like 80ms or 160ms, RAN4 to

- Option 1: define the lower limit for measurement delay requirements;

- Option 2: define the measurement delay requirements based on max(LP-SS periodicity, LO periodicity).

Proposal 9: RAN4 to focus on discussing requirements for LP-WUR for entry/exit criteria(threshold) evaluation for MR RRM relaxation, whereas the LR-WUS monitoring requirement discussion pending on RAN2’s progress.

Proposal 10: When not considering the legacy state (i.e. excluding scenarios in the Note), the evaluation requirement for entry/exit evaluation is the same as that of LP-WUR measurement period for both case 1 and case 3.

Proposal 11: When considering the legacy state (i.e. considering the scenarios in the Note),

- For case 1/ case 3 to legacy state, the evaluation requirement is the same as the LP-WUR measurement requirements;

- For legacy state to case1 or case3, the evaluation requirement is the same as the MR serving cell measurement requirements.

Proposal 12: RAN4 to define multiple scaling factor values for MR RRM relaxation in case3, while the value depends on DRX cycle length.

Proposal 13: RAN4 to determine the measurement accuracy in RRC_IDLE/INACTIVE state for simulation purpose, and NO need to define dedicated accuracy requirement in the performance section.

Proposal 14: No RAN4 impact of LP-WUS/WUR features in RRC_CONNECTED mode is expected.

- Reference

- R4-2420100, WF on RRM requirements for NR_LPWUS, Vivo

- R4-2414909, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1